|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 16 R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 16 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Clause 1 – Text input for scope section |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

Clause 1 is still lacking an initial text proposal. First input text is proposed by this contribution, primarily based on the work item description. An editor’s note shall reflect the still premature state of this Draft.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

Summary

Recommendation ITU-T H.248.TLS defines new ITU-T H.248 packages for support of secured TCP bearer traffic according the Transport Layer Security (TLS) technology. In addition, this Recommendation identifies typical network use cases with different ITU-T H.248 media gateway variants, and provides example signalling flows.

# 1	Scope

{Editor’s note (2013-01 meeting): this scope section is still not complete and should be updated as soon as the Recommendation text becomes more mature. Contributions are solicited.}

Transport Layer Security (TLS) is a cryptographic protocol that provides secure communication between two TCP endpoints. This Recommendation defines in general H.248 signalling elements for support of TLS in various options.

This Recommendation provides in more detail information about

- basic exchange architecture of keying information between control plane (incl. H.248 interface) and bearer plane;

- negotiation aspects: indication and determination of cryptographic capabilities between TLS endpoints;

- minimum of H.248 information for H.248 TLS/TCP terminations (NOTE 1);

NOTE 1 – Some information could be provisioned via management and there is also TLS information exchanged via the (TCP) bearer interface with the remote TLS endpoint.

- TLS procedures in detail at the various TLS sublayers, i.e. for support of the

  - TLS record protocol

  - TLS handshake protocol

  - TLS change cipher spec protocol

  - TLS alert protocol

  - TLS application data protocol

- profiling of TLS services;

- specify set of cipher suites;

- consideration of H.248 MG modes of operation and connection models;

- SDP- versus Property-based TLS endpoint control

- {list to be updated before approval/consent …}

- The scope of this Recommendation is limited to {… any limitations?  …}

## 1.1	Applicability statements

Table 1 summarizes all possible TLS-based interfaces of H.248 entities, - under the assumption of an underlying IP network -, and their relevance for this Recommendation.

Table 1 – Principal TLS-based interfaces of H.248 entities 
and their relevance for this Recommendation

| TLS-based transport at: | H.248 entity: | This Recommendation: |
| --- | --- | --- |
| Call control interface (e.g., SIP) | MGC | Out of scope. |
| Gateway control interface (H.248) | MGC, MG | Out of scope. Possible H.248 transport modes are indicated by [ITU-T H.248.67]. Usage of a TLS-based H.248 transport mode would be typically specified by an H.248 profile (as part of clause 6.10 in the profile definition template (see Appendix III in [ITU-T H.248.1])). |
| Bearer interface | MG | In scope. |

[End Proposed Correction]

_________________