|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 18 R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 18 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Clauses 6 to 7 – Update proposals |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

Clauses 6 through 7 are still lacking an initial text proposal. First input text is proposed by this contribution, primarily based on the work item description. An editor’s note shall reflect the still premature state of this Draft.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

# 6	Use case descriptions

Transport security is a network level service and primarily affects H.248 MGs. Basic use cases may be identified from the perspective of

- H.248 MG type: IP-to-IP gateway (see clause 6.2) or IP-to-non-IP gateway (see clause 6.3) in case of usual two-party communication services; and

- Multiparty service types (see clause 6.4).

It may be noted that the various network use cases may be abstracted by a single bearer connection model, see clause 7.2.

## 6.1	Use cases related to TLS transport modes

The TLS protocol is independent of the underlying transport protocol stack. Multiple TLS transport modes are basically useful (Figure X).

Figure X – TLS transport modes

This Recommendation is not dependent on any TLS transport mode, but uses TLS-over-TCP/IP as example.

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

The notion “IP-to-IP” (briefly “IP-IP”) indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 1):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as “non-TLS to non-TLS”);

- Use case #1.2: an end-to-end TCP connection traverses two network domains with different transport security policies (“TLS”, “no TLS”). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as “TLS to non-TLS”);

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as “TLS to TLS”);

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as “TLS to TLS*”);

Figure 1 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor’s note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

## 6.3	Bearer connection network use cases with H.248 TCP-to-Non-TCP gateways

Theoretical scenarios with TCP endpoint location in H.248 MG, but forwarding of application data via a non-TCP bearer.

For further studies.

## 6.4	Bearer connection network use cases with multiparty services using TCP transport (e.g. H.248.69 gateway)

{Editor’s note (2012-02): see also discussion of clause 13.3 in H.248.NATTP2P.}

For further studies.

# 7	Models

## 7.1	Network model from H.248 entity point of view

Following network model (Figure 2) may illustrate the relevant areas covered by this Recommendation. The H.248 MG is basically peering a TLS-capable IP host as remote endpoint. Both TLS endpoints spanning a network transport security domain (here TLS domain).

Figure 2 – Network model from H.248 entity point of view 
(“half call/bearer connection model”)

This Recommendation addresses primarily

- signalling capabilities and procedures at the H.248 interface;

- aspects and control of TCP mode(s) of operation in the H.248 MG; and

- configuration and procedures of the TLS/TCP/IP protocol stack for H.248-controlled bearers.

{Editor’s note (2012-02): there might be an additional network interface in case of dedicated, centralized network servers in the TLS domain with respect to key distribution. This may imply a modified Fig. 2, as well as an extend list above.}

## 7.2	Bearer connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a single ITU-T H.248 termination. In addition, no assumptions are made regarding either the lower layer protocols beneath the TLS level or the upper layer protocols being carried by the TLS/TCP transport. This allows the use of the Recommendation's procedures in various connection models and use-cases.

Figure 3 provides the TCP/IP protocol stack with the suite of TLS protocols and their sub-layer organization.

Figure 3 – TCP/IP protocol stack with TLS protocol

Figure 4 details the generic connection-model where a TLS-enabled termination is connected to a single other termination (either TLS-enabled or not). The generalization to any number of terminations is trivial.

Figure 4 – Two-termination context with a TLS termination

[End Proposed Correction]

_________________