|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 19 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 19 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: clause 6.2 – “TLS to TLS” use case variations |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

It was noted that there are two slightly different use cases behind “TLS to TLS” interworking. Correspondent text is added in a new subclause.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

## 6.2	Bearer connection network use cases with H.248 IP-IP gateways

### 6.2.1	High-level use case categories

The notion “IP-to-IP” (briefly “IP-IP”) indicates an (IP, IP) connection model (as part of clause 6.4 in the profile definition template (see Appendix III in [ITU-T H.248.1])). Such a connection model is described in detail already in clause 13.3 in [ITU-T H.248.84] for TCP-based H.248 streams.

Each H.248 stream endpoint (SEP) may be associated with a non-TLS or different TLS protocol stack variants, leading to various bearer network connection use cases (see Figure 1):

- Use case #1.1: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection without any applied transport security (abbreviated as “non-TLS to non-TLS”);

- Use case #1.2: an end-to-end TCP connection traverses two network domains with different transport security policies (“TLS”, “no TLS”). The H.248 IP-IP MG is located in the middle, at the border of both domains (abbreviated as “TLS to non-TLS”);

- Use case #1.3: an H.248 IP-IP MG located in the middle of an end-to-end TCP connection with applied transport security (abbreviated as “TLS to TLS”);

- Use case #1.4: there is end-to-end transport security, but different TLS profiles (abbreviated as “TLS to TLS*”);

Figure 1 – Bearer connection network use cases with H.248 IP-IP gateways

{Editor’s note 1 (2012-02): we need to define the term “TLS profile”.}

{Editor’s note 2 (2012-02): we need to summarize the different requirements for the H.248 interfaces and the IP bearer interface.}

### 6.2.2	Use case variations

### 6.2.2.1	“TLS-to-TLS transparent forwarding”

FIXTHIS

#### 6.2.2.2	“TLS-to-TLS application aware and decryption-encryption based forwarding”

The MG may be enforced for application awareness in case of e.g. bearer-level ALG support (according [ITU-T H.248.78]). The MG is located within a single security domain, therefore not any change of the TLS profile. However, the MG is located at the edge of two network address domain, thus requested for NAT-T and NAT support.

Example use case:

MSRP bearer-level ALG with MSRP-over-TLS transport and so-called media anchoring in H.248 IP-IP media gateways. See [IETF RFC 6714]; the “TLS-to-TLS” interworking function is termed as “TLS B2BUA”. It may be noted that this use case does not belong to the “TLS-to-TLS*” category. The TLS decryption-encryption process is indicated in clauses 6.5 and 7.4 in [IETF RFC 6714].

[End Proposed Correction]

_________________

Defined as: “A network element that terminates security associations (SAs) from endpoints and establishes separate SAs between itself and each endpoint.“ ([IETF RFC 6714])