|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 21 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 21 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: new Appendix III – Example TLS profiles |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

H.248.TLS/clause 8.7 contains example TLS profile concept information with regards to

- TLS domain profile

- MG TLS profile and

- intersection between both profiles

The TLS domain profile (1) is typically a network level feature, associated with security aspects, service provider policies, supported user equipment types, etc. There are consequently multiple TLS domain profiles around.

It is suggested to move such kind of information in an Appendix.

This contribution provides additional example TLS domain profiles.

Furthermore: the existing TLS domain profile description (see H.248.TLS/Table 2) might be enhanced for better readability. This contribution proposes a slightly different table format, also the partitioning in two tables.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

Appendix III

Example TLS profiles

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some exemplary TLS profiles.

## III.1	Typical Internet TLS profile

### moved from clause 8.7 to this section ###

Table III.1.a – Example of a TLS domain profile – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 1.3 | Compression methods: | NULL | - | DEFLATE, LZS | - |
| 1.4 | Extensions: | None |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 TLS 1.2 | - | - | - |
| 2.2 | Cipher suites: | in Table III.1.b | in Table III.1.b | in Table III.1.b | in Table III.1.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | None |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | - |  |  |  |
| 3.2 | Client authentication required | Yes |  |  |  |
| 3.3 | Certificate types (client): | signed, self-signed |  |  |  |
| 3.4 | Renegotiation: | Yes |  |  |  |
| 3.5 | Resumption: | No |  |  |  |
| 3.6 | Extensions: | - |  |  |  |
| 3.7 | Capability negotiation: | Yes |  |  |  |

Table III.1.b – Example of a TLS domain profile – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA, TLS_RSA_WITH_3DES_EDE_CBC_SHA, TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | - | - |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | - |
|  | SHOULD NOT | - | - |

## III.2	3GPP TLS domain profile

Network domain security is defined in [ETSI TS 133.310] with the normative TLS domain profile in Annex O. Table III.2 summarizes this profile with status of 3GPP Release 11 (2012-12).

Table III.2.a – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | SSL 3.0 | TLS 1.2 | - |
| 1.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 1.3 | Compression methods: | Compression Method.null |  | methods of [IETF RFC 3749] |  |
| 1.4 | Extensions: | allowed, but not specified by 3GPP |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | see client | see client | see client | - |
| 2.2 | Cipher suites: | in Table III.2.b | in Table III.2.b | in Table III.2.b | in Table III.2.b |
| 2.3 | Compression methods: | see client | - | see client | - |
| 2.4 | Extensions: | see client |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | Client authentication required | ? |  |  |  |
| 3.3 | Certificate types (client): | ? |  |  |  |
| 3.4 | Renegotiation: | NOTE 2 |  |  |  |
| 3.5 | Resumption: | NOTE 2 |  |  |  |
| 3.6 | Extensions: |  |  |  |  |
| 3.7 | Capability negotiation: | YES (NOTE 1) |  |  |  |
| NOTE 1 – Target: negotiation up to highest TLS protocol version. NOTE 2 – Not explicitly specified, thus TLS V1.1 [IETF RFC 4346] applies: YES |  |  |  |  |  |

Table III.2.b – 3GPP TLS domain profile (Annex O/[ETSI TS 133.310]) – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA (NOTE 1) TLS_RSA_WITH_3DES_EDE_CBC_SHA (NOTE 2) TLS_RSA_WITH_NULL_SHA | see client |
|  | SHALL NOT | a) Cipher suites with NULL integrity protection or HASH b) “TLS_DH_anon_WITH_” (NOTE 3) | see client |
| O | SHOULD | TLS_RSA_WITH_AES_128_CBC_SHA256 TLS_RSA_WITH_NULL_SHA256 | see client |
|  | SHOULD NOT | Cipher suites with RC4 | see client |
| NOTE 1 – The only mandatory cipher suite from TLS V1.2 [IETF RFC 5246]. NOTE 2 – The only mandatory cipher suite from TLS V1.1 [IETF RFC 4346]. NOTE 3 – Because the “the key exchange method shall not be anonymous”. |  |  |  |

Preferred cipher suites (ordered list): negotiation aspect?

## III.3	OMA TLS domain profiles

The Open Mobile Alliance (OMA) defines two TLS domain profiles [OMA-TS-TLS-V1_0-20080902-A], a “public key TLS domain profile” and a “pre-shared key TLS domain profile”.

FIXTHIS

### III.3.1	OMA public key TLS domain profile

Table III.3.a – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Main part

| Item | Property | Mandatory support |  | Optional support |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SHALL | NOT | SHOULD | NOT |
| 1) Client related: |  |  |  |  |  |
| 1.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 1.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 1.3 | Compression methods: | methods of [IETF RFC 3749] |  |  |  |
| 1.4 | Extensions: |  |  |  |  |
| 2) Server related: |  |  |  |  |  |
| 2.1 | TLS versions: | TLS 1.1 | - | TLS 1.0 | - |
| 2.2 | Cipher suites: | in Table III.3.b | in Table III.3.b | in Table III.3.b | in Table III.3.b |
| 2.3 | Compression methods: | methods of [IETF RFC 3749] |  |  | - |
| 2.4 | Extensions: |  |  |  |  |
| 3) Common: |  |  |  |  |  |
| 3.1 | Root certificates: | ? |  |  |  |
| 3.2 | (Server ¦ Client) authentication required | YES (Server authentication both) |  | YES (Client authentication by server) |  |
| 3.3 | Certificate types (client): |  |  |  |  |
| 3.4 | Renegotiation: |  |  |  |  |
| 3.5 | Resumption: | YES (NOTE z1) |  |  |  |
| 3.6 | Extensions: | [IETF RFC 4366] |  | TLS tunneling |  |
| 3.7 | Capability negotiation: |  |  |  |  |
| NOTE 1 – #. NOTE z1 – The longer session life (e.g., 12 hours) SHOULD be used. |  |  |  |  |  |

Table III.3.b – OMA TLS domain profile [OMA-TS-TLS-V1_0-20080902-A] – Cipher Suites

| Support |  | To 1.2: Client related cipher suites | To 2.2: Server related cipher suites |
| --- | --- | --- | --- |
| M | SHALL | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_NULL_SHA | TLS_RSA_WITH_AES_128_CBC_SHA TLS_RSA_WITH_3DES_EDE_CBC_SHA TLS_RSA_WITH_NULL_SHA |
|  | SHALL NOT | None specified. | see client |
| O | SHOULD | TLS_RSA_WITH_3DES_EDE_CBC_SHA | None specified. |
|  | SHOULD NOT | All others | see client |
| NOTE 1 – # |  |  |  |

### III.3.2	OMA pre-shared key TLS domain profile

see [OMA-TS-TLS-V1_0-20080902-A].

## III.4	IETF Minimum TLS domain profile

The IETF noted the value of TLS protocol profiling. The IETF itself does not (yet) define any specific TLS profile itself, rather provides guidelines and decision criteria for TLS profiling, see [b-IETF tls-minimal].

[End Proposed Correction]

[Begin Proposal]

Bibliography

[b-IETF tls-minimal]	IETF draft-tschofenig-lwig-tls-minimal (2012), A Minimal (Datagram) Transport Layer Security Implementation.
URL: http://tools.ietf.org/html/draft-tschofenig-lwig-tls-minimal-01

[End Proposed Correction]

_________________