|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 22 R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 22 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Appendix I – Text input for scope section |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

The proposed changes shall enhance the readability of Appendix I. Figure I.1 is slightly modified. Some editor’s note could be deleted, new editor’s note are added due to some links with the present package update process.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

Appendix I

Sample use-cases of TLS bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ TLS bearer encryption.

## I.1	Use-case #1:   Terminal-to-MG TLS session, provisioned TLS service negotiation

This use case illustrates the bearer path TLS security between a UE and the MG. Such a configuration is characterized by the following aspects:

- UE and MG reside in the same TLS-domain; the MGC could be outside (as shown) or inside this TLS-domain, but the MGC is in any case aware of the applied TLS domain profile for the secured bearer traffic.

- IP transport bearer type: ‘TCP’ in this example (but might be also e.g. SCTP).

- Call/session control signalling: SIP with SDP Offer/Answer and SDP support for TCP connection control (based on [IETF RFC 4145]) as well as SDP support for connection oriented media transport over TLS (based on [IETF RFC 4572]).

- The SIP signalling path between the UE and the SIP-server is integrity protected, e.g. by transport options SIP/SCTP/IPsec, SIP/DTLS/UDP/IP, SIP/TLS/TCP, etc.

- H.248 MG type: (IP,IP) connection model, TLS to non-TLS

- Bearer path protocol layering: Application/TLS/TCP/IP (i.e., not any tunnelled transport).

- UE: TCPclient and TLS client, MG: TCP server and TLS server (NOTE: this assumption could be satisfied with SDP “a=setup:” attribute)

- H.248 MG TCP-mode: TCP-proxy.
{Editor’s note: Any other TCP-modes and potential transitions between different TCP-modes are to be studied. See also work on H.248.TCP }

- Mutual TLS-authentication via certificates, MG’s certificate is signed, UE’s certificate is self-signed

- TLS-related settings (TLS-version, cipher suites, compression methods, certificate, etc) provisioned in MG

Figure I.1 – Use-case #1: Bearer path TLS security between UE and MG

## I.2	Use case #2: Terminal-to-MG TLS session, MGC-controlled TLS service negotiation

This use case is an extension of the first use case (clause I.1) by allowing the MGC to influence the TLS service negotiation at bearer level. Use case #1 is based on preconditions which are partially subject of network provisioning. Use case #2 is motivated by the fact that there are a number of reasons which demand for a MGC involvement, such as to bind the TLS service parameters, to limit the negotiation time, etc.

{Editor’s note: this clause will be updated when the design and definition of H.248 packages for TLS will be more mature …}

## I.3	Use case #3: WebRTC to NGN/IMS interworking function with DTLS-to-TLS support

To be inserted.

{Editor’s note (2013-01): this use case is also considered from TCP perspective, thus covered by Appendix I.1/H.248.TCP. I.e., there might be a reference added later on in this clause …}

[End Proposed Correction]

_________________