|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 25 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 25 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLSPROF: Clause 6 – Update requirements |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLSPROF. The proposed changes are marked up against the proposed input draft to this meeting TD 21 (WP 2/16).

# Discussion

TLS transport mode related requirements are added.

Proposals

It is proposed to accept the marked up changes below. 
Changes are marked up against the input draft to this meeting TD 21 (WP 2/16), taking into account the editorial update proposals by C-24.

[Begin Proposal]

# 6	Requirements for control the MG mode of operation

## 6.1	TLS transport mode

TLS is designed as a transport-independent protocol, making just the assumption of “reliable transport” of the underlying protocol (see clause 1/[IETF RFC 5246]).

R-6.1/1: Transport mode “TLS-over-TCP” is required to be supported.

NOTE 1 – This requirement would be applicable for the very majority of use cases.

R-6.1/2: Transport mode “TLS-over-SCTP” [IETF RFC 3436]) can optionally be supported.

R-6.1/3: Transport modes other than TCP or SCTP is not recommended to be supported.

It may be noted that TLS protocol procedures needs to be decoupled from protocol procedures of the underlying transport protocol.

## 6.2	MG mode of operation for case of TLS-over-TCP transport

A particular mode of operation (of the MG) is given by

- H.248 SEP view: given by a particular configuration of the TLS/TCP/IP protocol stack (e.g., TCP client, TLS server, etc);

- H.248 Context view: given by the connection model, typically by two associated SEPs (e.g., TCP proxy or relay mode).

See also clause 13.5 “Indication of 'TCP mode' for H.248 MG” in [ITU-T H.248.84].

R-x.y/1: The MGC is required to control the TCP/TLS specific characteristics of the H.248 stream. This comprises:

- the mode of operation in the MG, TCP-proxy vs. TCP-relay.

- the TCP-connection mode for each TCP-SEP of the H.248 Context (TCP-server vs. TCP-client)

- the TLS-connection mode for each TCP/TLS-SEP of the H.248 Context (TLS-server vs. TLS-client). Note: the TLS-connection mode of a SEP may be set independently of its related TCP-connection mode.

R-x.y/2: The TCP-mode of operation to be applied between two SEPs within a MG depends on the processing of the TCP-byte-stream within the MG. In case an entity located above the TCP-layer within the MG is TCP-payload aware, the TCP-layer is required to deliver the byte stream in a reliable and ordered manner to that entity.

R-x.y/3: Thus the presence of at least one TLS entity is a sufficient condition to require the TCP-proxy mode. Refer to Figure 2.

NOTE 1 - That there might be multiple entities present which require the TCP-proxy mode, e.g. in case both terminations of a Context are TLS-enabled, or if another application-aware interworking function in performed by the MG.

NOTE 2 - The term “application” is widely used in TCP-related Contexts, but in this Recommendation it is used as a synonym for the term “media”.

Figure 2 – TCP-byte stream processing entities located in the MG

Although the requirement above seems to require a “full TCP-proxy” mode for the entire lifetime of the TCP-session, there might be use cases for which at least for a dedicated period of the session a simplified TCP-proxy mode makes sense, e.g. from system resource perspective. Such a “lightweight TCP-proxy” mode is for further study.

Besides the termination of a TLS-session the MG may be required to perform application data interworking function. Thus the MG may operate in an “application agnostic TCP-proxy” mode as depicted in Figure 3, or as an “application aware TCP-proxy as shown in Figure 4.

Figure 3 – “application agnostic” TCP-proxy with TLS to non-TLS interworking

Figure 4 – “application aware” TCP-proxy with TLS to non-TLS interworking

The indication of the TCP-proxy mode is defined by [ITU-T H.248.84], but the procedures to control a SEP in the TCP proxy modes must be revised and extended.

R-x.y/4: The MGC shall have the capability to control the setup of the TCP-connection.

R-x.y/5: The MG is required to conform to the TCP protocol according to [IETF RFC 793].

[End Proposed Correction]

_________________