|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 26 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 26 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLSPROF: Clauses 8.1 and 8.2 update proposals |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLSPROF. The proposed changes are marked up against the proposed input draft to this meeting TD 21 (WP 2/16).

# Discussion

This contribution addresses

- editor’s note in clause 8.1 by proposing a figure and a correspondent revision of requirements; and

- updates the requirements related to TLS client/server assignment in clause 8.2.

Proposals

It is proposed to accept the marked up changes below. 
Changes are marked up against the input draft to this meeting TD 21 (WP 2/16), taking into account the editorial update proposals by C-24.

[Begin Proposal]

# 8	Requirements on TLS procedures

## 8.1	Selection of the TLS domain

A MG may be connected to multiple network domains with different TLS profiles. Figure x illustrates such a use case (with scope on TCP bearer traffic) at the example of an H.248 IP-IP MG located at the edge between access network (AN) and core network (CN) domains. The CN belongs to a trusted domain, hence transports unencrypted TCP traffic. Multiple different, TLS-enabled AN domains may be the result of multiple different access technologies, trust models, service provider models, etc.

Figure x – Use case: MG connected to multiple, different TLS domains

R-8.1/1: The MGC is required to provide the TLS domain towards the MG upon creation of a TLS endpoint, under the condition of multiple TLS domains connected to the MG.

R-8.1/2: The MGC is not required to provide TLS domain related information to the MG, under the condition that just a single TLS domain would be served.

R-8.1/3: The MG is required to apply the TLS profile based on the provided TLS domain for the created TLS endpoint.

R-8.1/4: The MG is required to use the certificate for authentication that is associated with the provided TLS domain, if certificate based authentication if applicable.

## 8.2	Client/Server Mode

R-8.2/1: The MG is required to support both the TLS server and TLS client mode.

The TLS standards do not make any correlation between the initiation of the underlying transport connection and the TLS server/client mode. Thus this mode is decoupled from the (TCP) SDP “a=setup:” attribute, as used for TLS-based SDP indications (see clause 4/[IETF RFC 4572]).

R-8.2/2: The MGC is required to indicate the TLS server/client role to the MG.

NOTE – There is not any existing TLS related SDP defined which represents TLS client/server semantic. [IETF RFC 4572] intentionally delegates such control information up to the application (layer).

R-8.2/3: The MG is required to select the TLS client or TLS server role based on the MGC request.

____________________

This use case is a variation of use case #1.4 from clause 6.1/[ITU-T H.248.TLS].