|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 27 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | November 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 27 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLSPROF: clause 8 “Self Signed Certificates” |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLSPROF. The proposed changes are marked up against the proposed input draft to this meeting TD 21 (WP 2/16).

# Discussion

Purpose of this contribution is related to the Editor’s comment concerning careful usage of self-signed certificates. The comment may be addressed by explicit applicability requirements.

Proposals

It is proposed to accept the marked up changes below. 
Changes are marked up against the input draft to this meeting TD 21 (WP 2/16), taking into account the editorial update proposals by C-24.

[Begin Proposal]

#### 8.3.3.2	Self Signed Certificates

Self signed certificates used for authentication can be protected against tampering by using fingerprints of the certificate. Those fingerprints consist of a hash algorithm (e.g. SHA-1) and the corresponding hash value of the certificate.

##### 8.3.3.2.1	Applicability requirements

Self signed certificates can be transported in a signalling information element over IP signalling paths (see Figure 4). As long as the integrity on the signalling information is assured, the certificate of the remote TLS endpoint can be verified against the fingerprint. How this information element is defined on the signalling path between the MGC and the remote end point, and how the integrity of that information element is assured (not only between MGC and endpoint but also on the H.248 interface) is out of scope of this Recommendation. For further reference refer to the [IETF RFC 4572].

Figure 4 – Use case: Fingerprint of the self-signed certificate passed via signalling path to MG

R-8.3.3.2.1/1: Self-signed certificates is prohibited from use when ### transport integrity could not be guaranteed.

R-8.3.3.2.1/2: Self-signed certificates can optionally be used at the H.248 interface when the integrity of the H.248 Control Association could be assured.

NOTE – This requirement is derived from “TLS-based SDP” usage from application control signalling, following the argumentation of clause 3.3/[IETF RFC 4572].

##### 8.3.3.2.2	Verifying the remote endpoint’s self signed certificate

R-8.3.3.2.1/1: In case the fingerprint information from the remote TLS-endpoint is received on the signalling path, the MGC is required to pass this information to the MG.

Editor’s note: Requirement only applicable if option is defined by the TLS profile. Clarification needed?

Editor’s note: TBC, layout and content of the fingerprint. Using RFC4572 on the H.248 interface could be an option

R-8.3.3.2.1/2: In case the fingerprint information is received by the MG, the MG is required to verify the received self signed certificate from the remote TLS endpoint against the fingerprint.

R-8.3.3.2.1/3: In case the verification fails, or in case the fingerprint information element has not been received by the MG, the MG is required to regard the remote TLS endpoint as not authenticated.

##### 8.3.3.2.2	Self signed certificate usage for own TLS endpoint authentication

R-8.3.3.2.2/1: In case the local TLS endpoint is to be authenticated by a self signed certificate the MG is required to include the fingerprint information of the certificate in the command reply that is generated as a response of the TLS endpoint creation.

R-8.3.3.2.2/2: The hash algorithm used to generate the fingerprint in the MG is required to be defined by the TLS-profile.

R-8.3.3.2.2/3: The MGC is required to include the fingerprint received in the H.248 command reply from the MG into the appropriate information element on the signalling path.

____________________