|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 28 R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 28 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.Supp.Prio: Clause I.2 – Resolution of editor’s notes |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.Supp.Prio. The proposed changes are marked up against the output draft of the last meeting TD 43 (Brisbane, 2012-09 meeting).

# Discussion

The description of the various H.248 signalling options in Appendix I.2 is fairly brief. More text was solicited at the last meeting. The correspondent editor’s notes are addressed by this contribution.

It was noted that the terms “direct signalling” and “indirect signalling” might be misleading, should be therefore explained in more detail.

The “principal H.248 signalling options” (Appendix I.2) are given by the combination of following characteristics:

- Translation function

  - at MGC level: call control protocol related codepoints are translated to H.248 codepoints (‘native’ or ‘generic’) format; or/and

  - at MG level: translation of H.248 codepoints to final IP bearer path format;

- H.248 signalling format

  - ‘native’, i.e., format as used in IP bearer packets;

  - ‘generic’, i.e. (I.b) translation would be always required.

Figure A summarizes all principal options:

Figure A – Appendix I.2/H.SuppPrio 
– Complete overview of “principal H.248 signalling options”

Present Appendix I.2 considers 3-out-of-8 theoretical options. It could be concluded that the existing notion of “direct signalling” relates to characteristic (II.a), and “indirect” to (II.b).

# Conclusion

Appendix I.2 could be updated based on this clarification. Editor’s notes might be then deleted in our understanding.

Further – Update Appendix I.1:

The editor’s note could be resolved. Below the status of 3GPP Rel-11 specifications.

Status:

- Ix – 3GPP TS 29.238 V11.0.0 (2012-09):

Table 5.5.1: Context Attributes

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| Topology | Yes (NOTE 1) | See clause 5.7.9 |
| Priority Indicator | Optional (NOTE 2) | 0-15 (NOTE 3) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Indicator | No | NA |
| ContextAttribute Descriptor | No | NA |
| ContextIdList Parameter | No | NA |
| AND/OR Context Attribute | No | NA |
| NOTE 1: Stream ID in Topology Descriptor shall not be supported (because only used for SRVCC service support, which is a monomedia type of call (“voice call”)). NOTE 2: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in 3GPP TS 22.153 [39]. NOTE 3: Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

- Iq - 3GPP TS 29.334 V11.0.0 (2012-09):

- Table 5.5.1: Context Attributes

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| Topology | Yes (NOTE 1) | See clause 5.7.9 |
| Priority Indicator | Optional (NOTE 2) | 0-15 (NOTE 3) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Indicator | No | NA |
| ContextAttribute Descriptor | No | NA |
| ContextIdList Parameter | No | NA |
| AND/OR Context Attribute | No | NA |
| NOTE 1: Stream ID in Topology Descriptor shall not be supported (because only used for SRVCC service support, which is a monomedia type of call (“voice call”). NOTE 2: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in 3GPP TS 22.153 [39]. NOTE 3: Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

# Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the output draft of the last meeting TD 43 (http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-43.zip).

#1:

[Begin Proposal]

## I.1	Status H.248 Profiles

H.248 profile definitions include Context Attribute support information (see profile definition template in [ITU-T H.248.1]). There are many H.248 profile specifications published by standard development organizations (SDOs). In the context of this Supplement are of particular interest are profiles for H.248 IP-IP gateways; such as the 3GPP-defined profiles for their H.248-based reference points Ix and Iq (according [ETSI TS 129 238] and [ETSI TS 129 334]; which are both based on ETSI TISPAN Ia profile [ETSI TS 183 018]).

The following table summarizes the status for Ix and Iq:

Table II.1 – 3GPP Profiles and typical support of Context Attributes (status: Rel-11, 2012-12)

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| … |  |  |
| Priority Indicator | Optional (NOTE 1) | 0-15 (NOTE 2) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Call Indicator | No | NA |
| … |  |  |
| NOTE 1: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in [ETSI TS 122.153]. NOTE 2: Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

It may be noted that there’s actually support of emergency calls only in pure 3GPP environments, and additional priority level support in ETSI TISPAN instantiations.

[End Proposed Correction]

#2:

[Begin Proposal]

## I.2	Discussion of principal H.248 signalling options

H.248 supports basically the provisioning of protocol parameter values, which might be sometimes an alternative to the usual signalling of protocol values. There are further different kind of H.248 decomposed gateway models, such as bearer-independent or bearer-dependent style of operation (see clauses 3.2.2 and 3.2.1). The alternatives lead to various options which are outlined in this clause, using the concrete example of DiffServ-based QoS marking as selected technology for priority traffic support.

### I.2.1	Overview

The “principal H.248 signalling options” are given by the combination of following characteristics:

- Translation function

  - at MGC level: call control protocol related codepoints are translated to H.248 codepoints (‘native’ or ‘generic’) format; or/and

  - at MG level: translation of H.248 codepoints to final IP bearer path format;

- H.248 signalling format

  - ‘native’, i.e., QoS marking codepoints signalled already in the format as used in IP bearer packets;

  - ‘generic’, i.e. (I.b) translation would be always required.

Three meaningful combinations are described subsequently.

### I.2.2	Native H.248 signalling

H.248 signalling (i.e., ADD and MODIFY request commands) carry the native DiffServ codepoint (as e.g. assigned for priority traffic). The MG directly uses that DiffServ codepoint for QoS marking outgoing bearer packets. There are two variants from MGC perspective, see Figure I.1:

Figure I.1 – Controlling of MG-level QoS marking function – Variants 1 and 2

Some observations:

- Variant 1 is based on pure signalling without any provisioning needs at MGC and MG level.

- Variant 2 is typically used when call control signalling either does not support signalling of DiffServ values at all, or when local DiffServ mapping is applied (Note: translation provided by MGC).

### I.2.3	Generic H.248 signalling

The DiffServ codepoint for a particular packet bearer traffic class might be also provisioned at MG level (see Figure I.2).

Figure I.2 – Controlling of MG-level QoS marking function – Variant 3

Some observations:

- Variant 3 does not require the explicit signalling of DiffServ codepoints at the H.248 interface. Another H.248 protocol element could be used instead, serving the ‘generic’ character. The MG has to translate generic H.248 codepoints to the required IP bearer path format..

- Variant 3 needs to take into account that an H.248 profile might support in addition direct DiffServ signalling, which may lead to a mix of both H.248 signalling options (i.e., ‘native’ and ‘generic’ formats used in parallel), and hence the careful consideration of possible interaction issues.

[End Proposed Correction]

_________________