|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 35 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 35 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPMUX: 3GPP-defined RTP multiplexing models |  |  |  |

Summary

This contribution discusses RTP multiplexing models which are supported already by H.248.

# Background

There were at least two activities in the past around multiplexing support for RTP traffic with H.248 control:

- Proposal of H.248.RM “RTP Multiplexing Package” (see 2008-01 meeting, AVD-3360)

- 3GPP 29.414 specification of “Bearer Transport with multiplexing”

Both activities consider 3GPP specific protocol stacks, using a so-called 3GPP framing protocol. Proposal 1 wasn’t accepted in ITU-T. Proposal 2 is specified in 3GPP 29.414.

This contribution focuses on the 2nd proposal.

# Discussion

Relevant section in 3GPP 29.414:

6.4	Bearer Transport with multiplexing	15

6.4.1	Introduction	15

6.4.2	Transport format	15

6.4.2.1	IP	15

6.4.2.2	UDP	15

6.4.2.3	Transport Format for multiplexing without RTP Header Compression	16

6.4.2.4	Transport Format for multiplexing with RTP header compression	17

6.4.3	RTCP	19

6.4.3.1	General	19

6.4.3.2	Multiplex negotiation via RTCP	19

6.4.3.3	RTCP Multiplexing packet	20

Scope: "… sub-clause specifies an optional transport format for the Nb interface and IP transport that allows transporting several RTP/NbFP/codec payload PDUs of different user plane connections within one packet ..."

Thus,

- there are two RTP multiplexing models (with or without header compression support)

- such kind of RTP multiplexing is limited to Nb (CS domain only), but not Mb (IMS domain), - also limited on framing protocol (FP) based IP stacks, but not on native IP stacks such as used for Mb

- 3GPP multiplexing is driven by bandwidth savings, whereas WebRTC/Telepresence consider RTP multiplexing from NAT-T perspective

- 29.414 defines an RTP extension ("multiplex header") which is limited on 3GPP solutions, but not considered for WebRTC (see draft-ietf-rtcweb-rtp-usage)

- the 3GPP approach of a "mux header" relates to the IETF approach of a "SHIM header" (see draft-westerlund-avtcore-transport-multiplexing)

- the SHIM header approach is not considered by IETF, neither for WebRTC nor CLUE (at least at this stage),

# Conclusions

It is proposed to inform about that multiplexing option in an Appendix of H.248.RTPMUX. However, this approach has some inherent deficiencies with respect to usage in services such as telepresence or WebRTC:

- Protocol stack tightly coupled with a specific network architecture (“3GPP CS domain with framing protocols”)

- Call-individual control of multiplexing not supported

- RTP multiplexing model driven by bandwidth efficiency, rather than NAT-T.

# Proposal:

[Begin Proposal]

Appendix I
Existing H.248 support for RTP multiplexing

(This appendix does not form an integral part of this Recommendation)

## I.1	3GPP-defined multiplexing for IP bearers with 3GPP framing protocol

Reference: see clause 6.4/[ETSI TS 129.414].

Motivation: transport capacity reductions (“bandwidth efficiency”).

Multiplexing method: multiple RTP packets mapped into a single PDU augmented by a (3GPP defined) multiplex header (“similar to IETF SHIM approach").

H.248 control: not any signaling support, thus pure provisioning based approach.

[End Proposed Correction]

_________________