|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 37 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 37 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPTOPO: Term "Supplementary RTCP service" |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.RTPTOPO. The proposed changes are marked up against the proposed input draft to this meeting TD 18 (WP 2/16).

# Discussion/conclusion

The editor’s note in clause 3.2.5 might be resolved by the addition of explicit definitions for RTCP service related terms. There are three categories according clauses 8.3 and 9.

The attached proposals are based on the discussions at past meetings.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 18 (WP 2/16).

[Begin Proposal]

3.2.5	RTCP service: A MG service tied to a particular RTCP packet type codepoint. The RTCP PT value may be thus considered as RTCP service identifier.

NOTE 1 – This is the coarsest granularity of an RTCP service definition, but sufficient for this Recommendation. A finer grain RTCP service level could take into account further RTCP packet embedded identifiers such as the block type (BT) codepoint in case of RTCP XR or the feedback message type (FMT) codepoint in case of RTCP FB.

NOTE 2 – Another classification approach could derive generic (because usually PT-independent) RTCP service categories such as related to performance monitoring, media encryption, codec rate control, congestion control, generic feedback, keep alive, etc.

3.2.x1	RTCP basic service: the RTCP services based on RTCP packet types SR (200), RR (201), SDES (202) and BYE (203).

NOTE 1 – These are RTCP services are considered to be the minimum of any RTP implementation according [IETF RFC 3550].

3.2.x2	RTCP profile related service: the RTCP service given by a specific RTP profile.

3.2.x3	RTCP supplementary service: any RTCP service in addition to RTCP basic (clause 3.2.x1) and/or RTCP profile related (clause 3.2.x2) services.

[End Proposed Correction]

_________________