|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 41 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 41 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPMUX: new H.248 Bundle package |  |  |  |

Summary

This contribution proposes a new H.248 package for control of a particular RTP multiplexing model.

# Background/Discussion:

See Annex A of AVD-4272 (http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4272.zip): clause A.4 describes a scenario with a TerminationState-property based multiplexing control.

The direction of RTP multiplexing control might be considered to be still premature, primarily dependent on the progress on correspondent IETF work products.

However, it looks like that additionally new H.248 protocol support is required because the “entire RTP multiplexing control” might be not associated to the “SDP based media description” of a single H.248 stream endpoint, in case of multi-stream-per-termination structures.

A termination-level control element would be required in such scenarios.

In order to have a concrete proposal for that particular mapping scenario and discussion, we’d like to suggest to add a correspondent package fragment to H.248.RTPMUX.

# Proposal:

[Begin Proposal]

# XY	Bundle package

{Editor’s note (2013-01): the package proposal isn’t yet complete, just high lighting the concept of a TerminationState level property for “bundling of MG resources”. Contributions are solicited.}

|  | Package Name: | Bundle package |
| --- | --- | --- |
|  | Package ID: | bundlepkg (0x####) |
|  | Description: | A multimedia related media configuration is typically distributed across multiple streams of a single termination. This package allows the control of common, shared resources for such a “bundle of stream endpoints”. Such resource components would be MG local resources, which are also completely managed by the MG itself. The package defines a property at termination level, which allows the MGC to indicate which stream endpoints of the same termination should be “bundled”. The MG takes this information in his local resource management into account. |
|  | Version: | 1 |
|  | Extends: | None |

## XY.1	Properties

### XY.1.1	Bundle

|  | Property Name: | Bundle |
| --- | --- | --- |
|  | Property ID: | bundle (0x0001) |
|  | Description: | This property FIXTHIS (which resource components are in scope of “bundle” etc) |
|  | Type: |  |
|  | Possible values: | list of StreamIDs |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

## XY.2	Events

None.

## XY.3	Signals

None.

## XY.4	Statistics

None.

## XY.5	Error Codes

FIXTHIS …

## XY.6	Procedures

### XY.6.1	##

[End Proposed Correction]

Copy from AVD-4272: Annex A – Example signalling

…

## A.4	Use case #III: MG local = fully multiplexed, MG remote = fully multiplexed

Characteristics

- communication service:

- same as use case #I

- remote endpoint A:

- same as use case #I

- local endpoint, the H.248 MG:

- H.248 IP-IP MG type, acting as RTP translator

- fully multiplexed mode

- TerminationState-Property controlled multiplexing (so called mapping option II, see clause 2.7.2
=> consideration of new H.248 bundle package (bundlepkg)

- thus, not any “SDP bundling” required at H.248 interface

- single UDP transport connection endpoint

- explicit assignment of SSRC values for support of RTP media multiplexing, also now for local endpoint, beside remote endpoint A

NOTE to SDP “a=ssrc:”: we are using now also the RD for the SSRC-based demultiplexing in incoming direction.

| Use case #III: | H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- | --- |
| MG remote = fully multiplexed, MG local = fully multiplexed | Media{ TerminationState{ bundlepkg/bundle{streams=[1,2]} ; Note 3 }, Stream=1{ LocalControl{ Mode=SendReceive, }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=audio <MG_UDP_port_UDP1> 8 13 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=audio <A_UDP_port_UDP1> RTP/AVP 8 13 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 a=ssrc:… } }, Stream=2{ LocalControl{ Mode=SendReceive, ReservedGroup=OFF, ReservedValue=OFF }, Local{ v=0 c=IN IP4 <MG_IP_addr_IP1> m=video <MG_UDP_port_UDP1> 96 a=rtpmap:96 VP8/90000 a=rtcp-mux ; Note 2 a=ssrc:… }, Remote{ v=0 c=IN IP4 <A_IP_addr_PeerC1> m=video <A_UDP_port_UDP1> RTP/AVP 96 a=rtpmap:96 VP8/90000 a=rtcp: <A_UDP_port_UDP1> IN IP4 <A_IP_addr_PeerC1> a=rtcp-mux ; Note 1 a=ssrc:… }}} | Contributor’s note: As compared to Use Case #II replace “RTP Session Multiplexing” with “Media Description Bundling”, because (according to my understanding) no two (or more) RTP sessions are actually multiplexed onto the same local transport endpoints, but different media description associated (RTP + RTCP) packet flows are multiplexed onto the same local transport endpoint. Note 1: See UC #I. Note 2: See UC #II. Note 3: Here, on termination level (within the TerminationState descriptor) I could imagine a new package property requesting the MG to apply “bundling” when creating the stream endpoints listed as property value. This raises many questions, though. Such a TerminationState “bundle” property would probably always be set already at the point in time a new SEP is created. What exactly is the relationship to RTP/RTCP multiplexing (to the a=rtcp-mux attributes in the Local descriptors)? |

_________________