|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 42 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 42 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPTOPO: New Appendix for "Examples of Transparent Forwarding" |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.RTPTOPO. The proposed changes are marked up against the proposed input draft to this meeting TD 18 (WP 2/16).

# Discussion

This is a follow-up activity of AVD-4248 from previous meeting. The notion of “implicit signalling” in general and “identical media descriptions” in particular (in clause 10.6.3.8) could be illustrated by signalling examples.

# Conclusion

A correspondent proposal for such an Appendix is provided by this contribution, based on the examples already reviewed and discussed at the Brisbane meeting.

# Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 18 (WP 2/16).

#1:

[Begin Proposal]

##### 10.6.3.8.1	Background to “Transparent Forwarding”

There are basically two H.248 signalling options in order to achieve a MG behaviour of “RTP transparent forwarding”:

a)	Implicit signalling

Existing H.248 signalling using LD/RD settings with “identical media descriptions” (see examples in Appendix I). There are two terminations (and two associated stream endpoints), which implies synchronized settings between all four local and remote descriptors (in case of a bidirectional RTP session). The MG would provide a transparent forwarding behaviour (according clause 3.2.6) for all cases of unambiguous H.248 signalling.

b)	Explicit signalling

Explicit indication via rtpt/rtptopo property (see next sub-clause).

[End Proposed Correction]

#2:

[Begin Proposal]

Appendix I
Examples of transparent forwarding

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction

The MG behaviour of “RTP transparent forwarding” may be enforced by either implicit or explicit H.248 signalling (see clause 10.6.3.8.1). The explicit method is semantically straightforward, whereas implicit signalling is often conditional. Some examples shall illustrate the uncontroversial and controversial usage of implicit signalling.

There is an (IP, IP) connection model in all examples. The signalling of a single stream endpoint is shown.

## I.2	Convention

The examples use an abstracted command notation, see Table I.1. There are three key areas of interest in such H.248 commands:

Table I.1 –Implicit signalling – abstracted H.248 command format 
with principal areas related to RTP

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=… a=… }, Remote { m=… a=… } other Descriptors{<”RTP impacting elements Y/N?”>} …} | There are basically three areas in H.248 commands which could contain information indicating the existence of RTP bearer traffic: - SDP media descriptions embedded in LD/RD as part of the Stream Descriptor; relevant information would be primarily subject of the “m=” and “a=” lines; - H.248 properties in the LCD (again embedded in Stream Descriptor); - H.248 properties in other descriptors (e.g., Events Descriptor, Statistics Descriptor). |

## I.3	Examples for implicit signalling

### I.3.1	Example 1 “UDP payload transparent forwarding”

Table I.2 illustrates “UDP payload transparent forwarding”:

Table I.2 – Example 1 “UDP payload transparent forwarding”

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/… Transaction = … { Context = … { Add = ip/$ ; Termination Ta, SEP S1 {Media { Stream = 1 { LocalControl { ... ; NOTE 1 } ... Local { v=0 c=IN IP4 <IP_addr> m=- - udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- - udp – }…}, Add = ip/$ ; Termination Tb, SEP S1 {Media { Stream = 1 { LocalControl { ... } ... Local { v=0 c=IN IP4 <IP_addr> m=- - udp – }, Remote { v=0 c=IN IP4 <IP_addr> m=- - udp – }…}, … | The RTP session shall be configured for transparent forwarding for both traffic directions. Prerequisites: - All four “m=” lines SHALL be identical. Example here: - Transport ‘UDP’ is indicated, which implies “RTP agnostic” to the MG. - The MG is requested for local NAPT, which is a L4-dependent function (thus “UDP” indication). Conclusions: - This setting does not provide “UDP transparent forwarding” (which was also not requested) due to UDP checksum updates. - It leads rather to “UDP payload transparent forwarding” … - … which could be “RTP transparent forwarding” in case of RTP-over-UDP bearer traffic |

### I.3.2	Example 2 “fully RTP protocol unaware forwarding”

Signalling according Table I.3 should lead to a MG behaviour of “fully RTP protocol unaware forwarding”:

Table I.3 – Example 2

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> - - }, Remote { m=- <port> - - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> - - }, Remote { m=- <port> - - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | The so called BGW mode of operation (in TISPAN BGF, 3GPP IMS-AGW, 3GPP TrGW, MSF SBG) with the characteristic of: - media type agnostic - media format agnostic - transport protocol type agnostic and - L4 port aware. The “-“ notation was introduced by TISPAN in border gateway profiles. |

Discussion/conclusions of example 2:

- The MG would implicitly provide “fully RTP protocol unaware forwarding” if there would be not any other signalling elements related to RTP

### I.3.3	Example 3 “additional RTCP awareness”

Example 3: same as example 2, but with additional property “rtcph/rsb = ON” in the LocalControl Descriptor (LCD)

Discussion/conclusions of example 3:

- That’s an explicit indication of RTP bearer traffic, the MG becomes RTP aware.

- However, there are basically three meaningful RTP translator options (PP, TR and TF) with different impact on RTP packets, thus pure implicit signalling would be controversial.

- Ambiguous behaviour might be achieved by explicit assignment of PP or TR topology (with different RTCP handling, but common SSRC/CSRC modifications) in case of non-transparent forwarding.

- Transparent forwarding however could not be achieved via implicit signalling, requires rather the explicit assignment of the TF topology.

### I.3.4	Example 4 “fully specified SDP media description”

Example 4: same semantic aimed as with example 2, but fully specified SDP media description, rather than the “-“ option, see Table I.4:

Table I.4 – Example 4

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … }, Remote { m=audio <port> rtp/avp x1 x2 a=rtpmap:x1 … a=ptime: … } other Descriptors{<”RTP impacting elements Y/N?”>} …} | This option was added by 3GPP profiles (in comparison to TISPAN BGF). The MG has to compare all four SDP media descriptions, and if fully identical then to conclude a correspondent RTP topology behaviour. But which one exactly? |

Discussion/conclusions of example 4:

- This example should be functionally identical as example 2, i.e. leading to the same MG behaviour, but there are subtle difference between examples 2 and 4 when considering the dynamics of H.248 signalling.

- Problem: the final establishment of the H.248 stream might be subject of multiple signalling cycles (ADD, MOD, …). The MG becomes normally already RTP aware (even media aware) with the very first H.248 command request. But which topology? B2BRE, media translator, transport translator, transparent forwarding?

- The MG could of course wait till the stream is finally established, but that would be an unsatisfying situation for performance optimized implementations, which may want to achieve e.g. the minimization of call and bearer establishment delays.

- The explicit indication of the aimed RTP topology (MG, PP, TR or TF) in the first command request could solve the ambiguity.

### I.3.5	Example 5 “transport protocol translation for RTP bearer traffic”

Table I.5 illustrates the example RTP-over-UDP to RTP-over-TCP interworking (but could be also RTP-over-UDP to RTP-over-DCCP, etc.).

Table I.5 – Example 5

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> UDP - }, Remote { m=- <port> UDP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> TCP/RTP/AVP - }, Remote { m=- <port> TCP/RTP/AVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | Stream endpoint SEP Ta/S1 is L4+ agnostic, the MG would be RTP unaware. However, SEP Tb/S1 indicates RTP as L4+ protocol. The complete settings lead to the conclusion of transport protocol translation for RTP bearer traffic. |

Discussion/conclusions of example 5:

- That is a different signalling variant, but similar concerning conclusions as example 3.

- The MG would exclude the PP topology (because PP would be based on value “RTP/AVP” instead of “UDP” at Ta/S1 side.

- However, there are still two meaningful RTP translator options (TR and TF), thus, not unambiguous.

### I.3.6	Example 6 “transparent forwarding of encrypted RTP traffic”

See Table I.6:

Table 6 – Example 6

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Add = ip/$ ; Termination Ta, SEP S1 … Stream = 1 { LocalControl {<”RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …}, Add = ip/$ ; Termination Tb, SEP S1 … Stream = 1 { LocalControl {<” RTP impacting elements Y/N?”>} Local {… m=- <port> RTP/SAVP - }, Remote { m=- <port> RTP/SAVP - } other Descriptors{<”RTP impacting elements Y/N?”>} …} | SRTP traffic is indicated by the “m=” line, in a media agnostic manner. The complete settings lead to the conclusion of transparent forwarding of SRTP bearer traffic. |

Discussion/conclusions of example 6:

- There could be an ambiguous situation without explicit RTP topology indication (e.g. MG could conclude B2BRE (= PP)), which would be controversial.

- Required would be a transparent forwarding behaviour, which would be satisfied by RTPTT and TF topologies. RTPMT might be ambiguous because there shall not be any type of media translation function. However, property value rtpt/rtptopo = TR covers both, RTPTT and RTPMT.

- Solution: unambiguous semantic by explicit indication of rtpt/rtptopo = TF.

[End Proposed Correction]

_________________