|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 44 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 44 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPTOPO: IETF's Media-aware Network Element (MANE) |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.RTPTOPO. The proposed changes are marked up against the proposed input draft to this meeting TD 18 (WP 2/16).

# Discussion

The IETF MANE concept is used in RTP related documents. The definition is contained in RFC 6184:

media-aware network element (MANE): A network element, such as a

middlebox or application layer gateway that is capable of parsing

certain aspects of the RTP payload headers or the RTP payload and

reacting to the contents.

Informative note: The concept of a MANE goes beyond normal

routers or gateways in that a MANE has to be aware of the

signaling (e.g., to learn about the payload type mappings of

the media streams) and that it has to be trusted when working

with Secure Real-time Transport Protocol (SRTP).  The advantage

of using MANEs is that they allow packets to be dropped

according to the needs of the media coding.  For example, if a

MANE has to drop packets due to congestion on a certain link,

it can identify and remove those packets whose elimination

produces the least adverse effect on the user experience.

RTP RFCs with MANE usage indicate that the MANE behaviour is fundamentally based on an “RTP topology” related behaviour plus additional functionality in the RTP bearer path. The media-aware capability implies that a MANE device must be signaling-aware, too.

The MANE function could be perfectly provided by an H.248 gateway, i.e., an MGC (“signaling-awareness”) plus an IP-IP MG for media-aware RTP packet processing plus the execution of associated packet policy rules.

# Conclusion

The MANE concept is a) tightly related to “RTP topologies” and b) in scope of H.248 gateways (profiles) which support correspondent RFCs. It should be documented therefore in H.248.RTPTOPO.

# Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 18 (WP 2/16).

#1:

[Begin Proposal]

## 7.2	Two H.248 ”IP” Termination per Context

### 7.2.1	RFC 5117 defined RTP topologies

[…]

### 7.2.2	Other RTP topologies

#### 7.2.2.1	RTP source translator topology

[…]

#### 7.2.2.2	IETF Media-aware network element (MANE)

The concept of media-aware network elements (MANE) is loosely used in the IETF when looking at general IP traffic handling. MANE behaviour in context of RTP is defined in [b-IETF RFC 6184] (and referred e.g. by [b-IETF RFC 6184], [b-IETF rtp-howto]):

Media-aware network element (MANE): A network element, such as a middlebox or application layer gateway that is capable of parsing certain aspects of the RTP payload headers or the RTP payload and reacting to the contents.

Media-awareness implies signalling-awareness. H.248 IP-IP gateways could realize the IETF MANE behaviour. The MGC is responsible for the signalling path related MANE part (typically “SDP awareness”), the MG for RTP packet processing. The RTP functions provided by a MANE device could be considered to be similar as RTP media translator or RTP mixer topologies. However, “MANE packet policy rules” may contain additional policy actions such as packet discard in case.

MANEs for RTP traffic could be therefore considered as extended RTP topology models.

[End Proposed Correction]

#2:

[Begin Proposal]

Bibliography

[b-ETSI TS 183 068]	ETSI TS 183 068 v3.1.1, Telecommunications and Internet Converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[b-IETF RFC 6184]	IETF RFC 6184 (2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6190]	IETF RFC 6190 (2011), RTP Payload Format for Scalable Video Coding.

[b-IETF RFC 6222]	IETF RFC 6222 (2011), Guidelines for Choosing RTP Control Protocol (RTCP) Canonical Names (CNAMEs).

[b-IETF RFC 6679]	IETF RFC 6679 (2012), Explicit Congestion Notification (ECN) for RTP over UDP.

[b-IETF rtp-howto]	IETF draft-ietf-payload-rtp-howto (2012), How to Write an RTP Payload Format.

[End Proposed Correction]

_________________