|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 46 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 46 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.50 Rev. 1: Indication of Support for Keep-Alive |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.50 Rev. 1. The proposed changes are marked up against the proposed input draft to this meeting C 30.

# Discussion/conclusions

Missing pieces of the “overall NAT-T puzzle” became available after the publication of H.248.50. Such information could be now added to H.248.50. This contribution considers the H.248.50 area of “keep-alive pinhole support”.

Two items are proposed to be added:

- information about MGC stimuli for IP bearer path keep-alive requests, and

- an editor’s note concerning keep-alive for “connection-oriented transports such as TCP or SCTP”.

See attached change proposals.

# Proposals

It is proposed to accept the marked up changes below.

#1:

[Begin Proposal]

## 9.2	Keep alive request package

{Editor’s note (2013-01 meeting): there is a general question still open: the existing keep-alive package did (?) just focus on connectionless transport UDP, but missed the consideration of connection-oriented transports such as TCP or SCTP. To be checked/confirmed.

Two further aspects:

- The “CRLF keep-alive technique“ (RFC 5626) may need to be supported for e.g. TCP or TLS/TCP.

- Connection Reuse in the SIP (RFC 5923)

Both items are driven by NAT-T, and both might be relevant in case of networks with H.248.TCP and/or H.248.TLS. Contributions are solicited.}

…

### 9.2.6	Procedures

#### 9.2.6.1	MGC stimuli for IP bearer path keep-alive requests

The IP bearer path related keep-alive functions are part of overall NAT traversal solutions. The MGC may trigger keep-alive procedures due to

- local policy (provisioned policy, e.g. in case that all calls of an IP network domain require keep-alive support) or/and

- call-individual requests (and parameterization) by application control protocol signalling (see e.g. [b-IETF RFC 6223], [b-IETF RFC 5626])

Example:

A SIP network with usage of [b-IETF RFC 6223]: the MGC could (inter alia) derive the signal parameters “keep alive transmission interval” (kar/skap/ti) and “keep alive packet type” (kar/skap/kapt) from SIP signalling.

#### 9.2.6.2	Triggering keep- alive at MG by MGC

In order to open or maintain a binding, the MGC should send the "Send Keep Alive" (skap) signal to the MG to initiate sending a keep-alive packet to the remote address. This will have the effect of opening a pinhole (or keeping a binding open) allowing the remote end to send packets to the local end. The "Keep Alive transmission interval" (ti) parameter is used in order for the MG to autonomously send a keep-alive packet when no packets have been detected for the address for time interval Tr.

Where a MGC utilizes a Keep alive Request signal with the "RTP packet with unknown payload type", it may be required to communicate this to a peer MGC via the SDP. An MGC on reception of this attribute should then set this attribute in the remote descriptor of the applicable MG Termination/Stream. It indicates to the MG that the remote end will utilize a keep-alive using an RTP packet with an unknown payload type.

[End Proposed Correction]

#2:

[Begin Proposal]

Bibliography

[b-IETF RFC 5626]	IETF RFC 5626 (10/2009), Managing Client-Initiated Connections in the Session Initiation Protocol (SIP).

NOTE 1 – Two keep-alive techniques: CRLF keep-alive technique for TCP and SCTP, and STUN keep-alive technique for UDP.

[b-IETF RFC 6223]	IETF RFC 6223 (04/2011), Indication of Support for Keep-Alive.

{Editor’s note (2013-01 meeting): possible further “keep alive” reference:
“Indication of support for reverse keep-alive” 
(draft-holmberg-sipcore-rkeep). Contributions solicited.}

[End Proposed Correction]

_________________