|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 47 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 47 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.TLS: Living list "DTLS and DTLS-SRTP" |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.TLS. The proposed changes are marked up against the proposed input draft to this meeting TD 20 (WP 2/16).

# Discussion

H.248.TLS scope is related to “transport security”, which covers basically

- TLS for transport protocols ‘TCP’ and ‘SCTP’ and

- DTLS for transport protocol ‘UDP’.

DTLS and TLS are tightly coupled. We expect also a lot of commonalities from perspective of H.248 control of bearer type ‘TLS’ and ‘DTLS’.

DTLS is part of the WebRTC service profile as subject of H.248.WEBRTC.

Conclusion

This contribution proposes to add ‘DTLS’ as a living list item to H.248.TLS. As soon as H.248 control of ‘TLS’ is getting more mature, then DTLS could be progressed.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 20 (WP 2/16).

[Begin Proposal]

new Appendix IV
Living List related to DTLS and DTLS-SRTP

(This appendix does not form an integral part of this Recommendation)

{Editor’s note (2013-01 meeting): This Appendix is related to “DTLS” and “DTLS related topics”

## IV.1	DTLS protocol

DTLS is derived from TLS, could be thus considered as a single security protocol family. DTLS versioning numbers are (somehow) linked to TLS protocol versions.

Figure IV.1 – TLS/DTLS protocol historic

## IV.2	DTLS related services

There are two slightly different applications:

- DTLS as “transport security” means for UDP/IP bearer traffic

- DTLS as “key exchange” means for RTP/UDP/IP bearer traffic using media security according SRTP

## IV.3	DTLS in scope of H.248.TLS?

There are basically following options:

- DTLS covered by H.248.TLS?

- DTLS in a self-contained, new H.248.DTLS?

- DTLS added to H.248.77 due to link with DTLS-SRTP?

- DTLS covered by H.248.WEBRTC due usage by WebRTC service.

Option #2 (or #1) seems to be preferable.

## IV.4	DTLS use cases

Initial list of possible use cases:

## IV.4.1	Use cases from TLS-to-DTLS interworking perspective

Figure IV.2 provides an interworking example (here: DTLS based secure WebRTC data service).

Figure IV.2 – Example TLS-to-DTLS interworking

## IV.4.2	Use cases from SRTP “key exchange” perspective

SRTP-based media encryption, - there are many key exchange options for SRTP. For example the two outlined in below subclauses.

#### IV.4.2.1	Media-path decoupled key exchange using SDP (via SIP & H.248)

Figure IV.3 recalls again the protocol stack for SDES-based key exchange for SRTP, as in scope of H.248.77 or as part of the 3GPP H.248 Iq profile (29.334):

Figure IV.3 – Media-path decoupled key exchange using SDP (via SIP & H.248)

#### IV.4.2.2	Media-path coupled key exchange using DTLS

Figure IV.4 illustrates the alternative of a DTLS-based key exchange for SRTP:

Figure IV.4 – Media-path coupled key exchange using DTLS

## IV.4.3	Two interpretations for „DTLS-SRTP“?

RFC 5764:

This document describes DTLS-SRTP, a SRTP extension for DTLS that

combines the performance and encryption flexibility benefits of SRTP

with the flexibility and convenience of DTLS-integrated key and

association management.  DTLS-SRTP can be viewed in two equivalent

ways: as a new key management method for SRTP, and a new RTP-specific

data format for DTLS.

I.e., there are two possible interpretations from protocol stack point of view, see Figures IV.5 and IV.6.

Figure IV.5 – Interpretation „DTLS only used for SRTP key exchange“

Figure IV.6 – Interpretation „Secure RTP traffic using DTLS“

It has to be noted that these interpretations might be incorrect. To be confirmed.

## IV.4.4	Use cases from WEBRTC perspective

DTLS use cases from WebRTC perspective:

Figure IV.7 – DTLS use cases: „WebRTC data“ & „WebRTC SRTP key exchange“

In Figure IV.7:

1) WebRTC data traffic

2) WebRTC DTLS-based key exchange for SRTP

## IV.4.5	Use cases from Telepresence perspective

Are there any? TBD

[End Proposed Correction]

_________________