|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 48 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 48 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.RTPTOPO: "RTP source translator" topology |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.RTPTOPO. The proposed changes are marked up against the proposed input draft to this meeting TD 18 (WP 2/16).

# Discussion

A new RTP topology was identified in the context of RTP multiplexing, plus optional RTP retransmission format, which is relevant in network scenarios with

- SRTP, i.e. H.248.77 and

- H.248.WEBRTC gateways.

The RTP topology is relevant for H.248 IP-IP media gateways which are involved in

- “SSRC multiplexing” (see RFC 5576) and/or

- “RTP retransmission payload format” processing (see RFC 4588).

Both RTP functions are related to the processing of RTP source specific information (SSRC, sequence numbers). It could be therefore called "RTP source translator" or "RTP source proxy" topology.

It has to be noted that this RTP topology was not (yet) discussed in the IETF. The new RTP topology could be associated to the category of RTP translator types, and is similar like an RTP transport translator topology. However, RFC 5117 limits the RTP transport translator (RTPTT) to the “translation of transport parameters”, which are effectively IP transport addresses and/or payload type codepoints. The translation of RTP source specific parameters is not outlined in RFC 5117.

For discussion – naming proposals for this new RTP topology:

- "RTP source translator" (RTPST) topology; or

- "RTP source proxy" (RTPSP) topology (briefly "RTP proxy" topology); or

- others?

We’ve selected the first one as a working assumption.

# Conclusion

H.248.RTPTOPO should be updated for this new topology. Such a change would affect following clauses:

- new clause 3.2.x concerning new term definition

- update of clause 7.2 with regards to the detailed functionality

- new clause 11 concerning the H.248 signalling methods how this topology may be enforced.
Note: it looks like that a pure SDP-based signalling would be unambiguous, thus not any updated of clause 10 is considered.

Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 18 (WP 2/16).

#1:

[Begin Proposal]

3.2.x	RTP source translator: RTP topology related to the translation of RTP source specific parameters. This topology belongs to the high level category of “RTP translator” topology.

NOTE – This topology is not defined by [IETF RFC 5117].

{Contributor’s note (2013-01 meeting): the name is still under discussion, might be e.g. replaced by “RTP source proxy” topology.}

[End Proposed Correction]

#2:

[Begin Proposal]

## 7.2	Two H.248 ”IP” Termination per Context

### 7.2.1	RFC 5117 defined RTP topologies

See Table 2.

Table 2 – RTP Topologies vs RTCP Handling vs H.248 MG Models
– Two H.248 ”IP” Terminations per Context

| No. | RTP Topology | RTCP Handling | H.248 Connection Model | Example |
| --- | --- | --- | --- | --- |
| II.1 | Back-to-Back RTP Endsystem (B2BRE) | F-1,2,3,4,5,6,7 | (RTP, RTP) | IP-to-IP MG located at the border between two RTP domains (e.g., SRTP-to-RTP interworking) |
| II.2 | RTP Transport Translator (RTPTT) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, without any media processing |
| II.3 | RTP Media Translator (RTPMT) in general (NOTE 1) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | IP-to-IP MG located within a single RTP domain, with processing of the RTP payload |
| II.3.1 | RTP Media Translator without media format change (RTPMTlight) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., transpacketization (changing packetization times), jitter reduction between two IP networks |
| II.3.2 | RTP Media Translator with media format change (RTPMTfull) | F-1,3,4,5,6,7 F-2 if F-6 | (RTP, RTP) | ditto, e.g., audio transcoding |
| NOTE 1 – The RTPMT topology is sometimes further sub-classified here with II.3.1 & II.3.2 (see e.g. [b-IETF RFC 6679]). |  |  |  |  |

NOTE - Some media-level functions may be achieved by more than one RTP topology. For example, audio transcoding may be done in B2BRE or RTPMT topologies. However, there would be different RTCP handling functions.

### 7.2.2	Other RTP topologies

### 7.2.2.1	RTP source translator topology

RTP source translators (RTPST) do not modify the media stream itself, but are concerned with RTP source parameters. RTP source parameters comprise the

- RTP source identifier (parameter  SSRC),

- RTP source/endpoint persistent identifier (parameter CNAME),

- RTP source sequence number space (parameters SN, OSN) and

- RTP source time space (parameter TS).

- {others?}

RTP source translators could modify one or multiple RTP source related parameters.

{Editor’s note (2013-01 meeting): RTCP handling should be described in more detail. Contributions are solicited.}

[End Proposed Correction]

#3:

[Begin Proposal]

# 11	Package-independent procedures for RTP topology control

## 11.1	Topology “RTP source translator”

### 11.1.1	Via SDP attribute "a=ssrc:"

The MGC shall use the SDP attribute "a=ssrc:" (see [IETF RFC 5576]) for the RTP source translator assignment of a particular Stream endpoint. Table 1 defines the allowed roles (i.e. SDP a=ssrc: value settings), given by the 4-tuple {Ta(LD), Ta(RD), Tb(LD), Tb(RD)}.

Table 1 – Indication of topology “RTP source translator”

| H.248 SEP |  | SDP attribute | “RTP source translator” |
| --- | --- | --- | --- |
| Ta | LD | a= ssrc: | wildcarded |
|  | RD | a= ssrc: | not sent |
| Tb | LD | a= ssrc: | not sent |
|  | RD | a= ssrc: | not sent |
| NOTE 1 – The semantic of "not sent" means that the MGC shall not include this SDP attribute in the H.248 Descriptor, despite the fact whether it is used on call signalling level. |  |  |  |

The RTP source translator function is requested to translate the RTP source value with following semantic:

- direction Ta to Tb: not any translation of RTP source parameter values;

- direction Tb to Ta: the received SSRC value in any kind of RTP packets is replaced by the locally used SSRC value for this H.248 SEP;

The MG shall reply with error code 449 in case of incorrect value settings.

### 11.1.2	Via MIME subtype “rtx” in SDP attributes

The MIME subtype “rtx” is defined in [IETF RFC 4588]. The MGC may use this element in SDP attributes for triggering an RTP source translator function with regards to the translation of sequence number spaces.

However, this particular RTP source translator function is conditional, only required when the MG would provide parallel RTP interworking functions which are dependent on sequence number values (such as SRTP to non-SRTP interworking).

The RTP source translator function “sequence number translation” would be then enforced by a correspondent H.248 descriptor information.

[End Proposed Correction]

_________________

OSN: original sequence number