|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 50 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 50 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.77 IG: MG interworking function "SRTP to non-SRTP with local SSRC change" |  |  |  |

Summary

Profile specification guidelines for two SRTP related use cases are proposed to be added to H.248.77.

# Discussion

See discussion in C-48 on RTP source translator topology. The concerned SRTP use case is related to SRTP to non-SRTP interworking, accompanied with a) local SSRC change and b) RTP retransmission usage (“which imply local SN adaptations”) for correct SRTP origination/termination in the MG.

The RTP source translator topology ensures uninterrupted packet flow in non-SRTP to SRTP direction. The reverse direction is not that critical.

# Conclusion

The original SRTP specification RFC 3711 could not yet take into account later RTP technology extensions such as RFC 4588 and RFC 5766. Thus, H.248.77 should be updated in order to complete H.248 related SRTP use cases.

Proposals

It is proposed to accept the marked up changes below.

#1:

[Begin Proposal]

# 2	References

FIXTHIS – to be added:

- [ITU-T H.248.39]

- [ITU-T H.248.RTPTOPO]

[End Proposed Correction]

#2:

[Begin Proposal]

Annex A

Profile guidelines for (IP, IP) connection models

(This annex forms an integral part of this Recommendation.)

## A.1	Motivation

The main body of this Recommendation is considering a connection model with scope on a single termination only, see clause 1.1. However, some SRTP related interworking functions by H.248 IP-to-IP media gateways require the consideration of the overall stream endpoint pair (SEPP). Such use cases are outlined e.g. in Appendix II.2 and II.3.

Purpose of this Annex is the description of high level profile specification guidelines. Such guidelines are typically “SRTP interworking” specific, see following clauses.

## A.2	Profiles with support of "SRTP to non-SRTP interworking with local SSRC change"

This interworking scenario implies the MG local termination/origination of SRTP. The SRTP encryption/decryption function is dependent on RTP source parameters SSRC and SN values. The MG should therefore provide a “RTP source translator” topology, see [ITU-T H.248.RTPTOPO].

The MG is also required to assign local SSRC values. Such a MG level resource management function should be triggered via SDP wildcarding, see clause 6.15.9/[ITU-T H.248.39].

## A.3	Profiles with support of "SRTP to SRTP interworking"

This interworking scenario is outlined in Appendix II.3. The MG shall not change RTP source parameters SSRC and SN, which would violate SRTP encryption/decryption. Such behaviour would be achieved by “RTP transparent forwarding” topology, see [ITU-T H.248.RTPTOPO].

[End Proposed Correction]

_________________