|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 51R1 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 51 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248.WEBRTC: Update of functional requirements for H.248 WEBRTC gateways |  |  |  |

Summary

This contribution provides change proposals for progressing Draft H.248.WEBRTC. The proposed changes are marked up against the proposed input draft to this meeting TD 22 (WP 2/16).

# Discussion

This contribution provides initial material for draft H.248.WEBRTC, primarily with regards to

- use cases as presented at last meeting, and

- document structure proposal related to the items according the charter of the work item.

# Proposals

It is proposed to accept the marked up changes below. Changes are marked up against the input draft to this meeting TD 22 (WP 2/16).

[Begin Proposal]

# 6	WEBRTC core technology overview

The H.248 WEBRTC gateway is peering to WEBRTC endpoints, given by IP host entities with “WEBRTC capable web browsers” as application instance. WEBRTC endpoints are characterized by following mandatory (and optional) capabilities:

- FIXTHIS

# 7	H.248 WEBRTC gateways: example use cases

Some exemplary use cases are described in this clause.

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A is always using WebRTC browser environment.

7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 1:

Figure 1 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a brower-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two WebRTC IP domains, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture by the two WebRTC IP domains, etc.

7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 2 depicts an interworking scenario with NGN/IMS networks:

Figure 2 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed to be not fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 3 outlines the general request in supporting of still existing PSTN/ISDN infrastructures.

Figure 3 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. H.324, H.320).

## 7.2	Multipoint communication

Multiparty calls are in scope of WEBRTC and may demand for “RTP mixer”, as e.g. provided by media servers. Such a network scenario may therefore relate to H.248 profile types with regards to media servers (aka media resource processors).

# 8	Functional requirements for H.248 WEBRTC gateways

There are general and use case specific functional requirements for H.248 WEBRTC gateways. WEBRTC gateway requirements could be classified in subsequent functional areas.

# 8.1	Requirements related to NAT traversal support

FIXTHIS {related to ICE/STUN (H.248.50), H.248.37}

## 8.2	Requirements related to communication topologies

### 8.2.1	H.248 WEBRTC gateway for point-to-point interworking

FIXTHIS

### 8.2.2	H.248 WEBRTC gateway as WEBRTC conferencing point

FIXTHIS

## 8.3	Requirements related to bearer traffic multiplexing

FIXTHIS {related to H.248.RTPMUX, etc}

## 8.4	Requirements related to RTP-to-RTP type interworking

FIXTHIS {related to draft-ietf-rtcweb-rtp-usage in general, interworking between different RTP profiles}

## 8.5	Requirements related to media-aware type of interworking

FIXTHIS {related to media format translations, such as audio transcoding}

### 8.5.1	Requirements related to WEBRTC audio

### 8.5.2	Requirements related to WEBRTC video

### 8.5.3	Requirements related to WEBRTC data

### 8.5.4	Requirements related to WEBRTC text

## 8.6	Requirements related to bearer security

FIXTHIS {related to draft-ietf-rtcweb-security-arch …}

## 8.6.1	Requirements related to transport security

FIXTHIS {related to draft-ietf-rtcweb-rtp-usage on “DTLS”, H.248.TLS…}

### 8.6.2	Requirements related to media security

FIXTHIS {related to draft-ietf-rtcweb-rtp-usage on “SRTP”, H.248.77…}

## 8.7	Requirements related to WEBRTC emergency services

FIXTHIS {related to draft-aboba-rtcweb-ecrit}

## 8.8	Requirements related to QoS support

FIXTHIS {related to draft-ietf-rtcweb-qos, H.248.52, H.248.53}

## 8.9	Requirements related to bearer-path coupled congestion controls

FIXTHIS {related to H.248.82, draft-alvestrand-rtcweb-congestion, …}

## 8.10	Requirements related to performance monitoring

FIXTHIS {related to H.248.48, H.248.RTCPPROF, draft-alvestrand-rtcweb-stats-registry, ..}

## 8.11	Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations

### 8.11.1	Requirements related to supported SDP elements

FIXTHIS

### 8.11.2	Requirements related to SDP Offer/Answer

FIXTHIS {related to H.248.80)}

# 9	H.248 profile specification guidelines

{Editor’s note (2013-01 meeting): this clause would/should follow the structure of clause 8 of H.248.81.}

[End Proposed Correction]

_________________