|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – C 52 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | December 2012 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 |  |  |  |
| STUDY GROUP 16 – CONTRIBUTION 52 |  |  |  |  |  |
| Source: |  | Alcatel-Lucent Deutschland, Alcatel-Lucent Shanghai Bell |  |  |  |
| Title: |  | H.248 control mode: SDP-based vs Property-based H.248 signalling – Deficiencies of the SDP method |  |  |  |

Background:

The discussion of SDP-based versus Property-based H.248 signalling is a well known topic in the H.248 community, but never really documented. The subject is currently again in focus as part of work items H.248.82, H.248.TCP and H.248.TLS.

The Property-based approach is generally admitted due to native H.248 signalling elements. The SDP-based method has limited applicability. Purpose of this contribution is to collect known limitations and boundary conditions for the SDP-oriented H.248 signalling method.

Introduction – H.248 control methods:

Figure 1 depicts a “SIP call server with an embedded MGC function”. There are basically two different H.248 control methods:

- SDP-based H.248 Media Descriptors

- Property-based H.248 Media Descriptors, i.e., the semantic of a SDP element is replaced by an H.248 property (such as Property, Event or Signal)

Method 1 is fairly attractive and preferred under the preconditions of a) SIP as application control protocol and b) H.248 in text encoding format. This approach is advantageous due to efficient translation between application control and gateway control signalling elements (see also Figure 2), being aware that the call server has to follow SDP usage rules according clause 7.1.8.1.1 “Specific syntax for H.248 text encoding” in H.248.1 and to respect other “SDP mapper” conditions.

Figure 1 – Call control level “SDP mapper” function at gateway control interface

# Discussion – preconditions:

Known deficiencies of the SDP-based method:

- Ambiguity of SDP elements (e.g., SDP “m=” line proto element UDP/TLS/RTP/SAVP, TCP/TLS/MSRP or TCP/TLS: which protocol (layer) drives exactly the semantic?).

- Semantical overloading I: if a particular SDP element is used for multiple, different functions during communication establishment (e.g., a bearer type indicator for a) the reservation and preparation of bearer interfaces, and b) as stimuli for triggering bearer connection control procedures). It should be possible to decouple both functions, which implies different kind of H.248 signalling.

- Semantical overloading II: if a particular SDP element is used for multiple, different MG services.

- Bidirectional communication path capability, but unidirectional scope only of concerned SDP element (“SDP in “LD  RD” must be consistent, if not, error”)

- Context level capability: Stream endpoint pair (SEPP) related capability (such as RTP topologies, TCP modes, etc)

- Multi-step bearer preparation and establishment, e.g.,

  - one-stage vs two-stage resource reservation procedures in border gateway profiles,

  - proactive vs reactive media interworking principles, or

  - first step “media-agnostic” settings, final step “media-aware” (after completed SDP Offer/Answer negotiations)

- Dynamics in call establishment: yet missing SDP for remote sides when gateway control procedures already triggered at incomplete SIP level SDP Offer/Answer stage …

- Agnostic media/transport MG modes: imply to omit SDP information, which might be in contradiction when SDP should be sent due to other control purposes.

- Semantical scope: IETF SDP elements are defined for communication endpoints (UEs, SIP UAs, etc), but not for network elements such as gateways or media servers. The semantical scope is typically different, particular in case of interworking functions provided by IP-IP gateways.
[Note: that’s the primary reason why H.248 reuses SDP in a different manner (see clause 7.1.8/H.248.1)].

- Irrelevance of SDP for gateway control: any session description related SDP ….(see Fig. 2)

- Semantic of ignore: different SDP processing styles at client/server or peer-to-peer based interfaces (“IETF semantic of ignore applies”) in contrast to master/slave oriented gateway control (“H.248 semantic of error codes applies”).

- Application control protocol without SDP (e.g., SIP-I, BICC, H.323)

- H.248 binary encoding format (“SDP is text-oriented”)

- others

Figure 2 – SDP processing instances in H.248 decomposed gateway models

# Conclusions:

The SDP-based method is preferable for H.248 profiles with text encoding and SIP network environments, and under the condition that unambiguous semantics could be guaranteed. Such specific H.248 profiles also should take into account evolutionary aspects: an uncontroversial SDP semantic in profile version N may become ambiguous in a later version, e.g. when additional capabilities would be added.

It should be also ensured that the semantic of a specific SDP parameter/value combination in profile X should be identical with other H.248 profiles.

The SDP-based method is conditional: above listed terms should be taken into account at the development of H.248 profiles and/or H.248.x-Recommendations.

# Appendix – Example H.248 technologies with alternative SDP- and Property-based support:

Some examples with debates on H.248 control options, see Table 1:

Table 1 – Example H.248 technologies 
with alternative SDP- and Property-based support

| H.248 control of: | 1) SDP-based | 2) Property-based |
| --- | --- | --- |
| RTCP port allocation | RFC 3605 SDP (NOTE 1) | H.248.57: rtcph package (NOTE 1) |
| SRTP functions | initial SRTP services with SDP | H.248.77: srtp package |
| ECN function | initial ECN service with SDP | H.248.82: ecnrous package |
| TCP functions | under discussion (“SDP usage according H.248.84”) | H.248.TCP: packages for TCP connection control |
| TLS functions | under discussion | H.248.TLS: packages for TLS session control |
| NOTE 1 – Actually both methods are merged by comprehensive port allocation rule semantics in H.248.57. |  |  |

It should be noted that these are not always “either-or” alternatives, rather often complementary.

_________________

Such semantical alignment may be ensured by specifying the SDP semantics in an H.248.x-Recommendation, - e.g. as package-less protocol procedures -, which may be then again referred by profile specifications.