| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4355(Amd1) |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 1 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | New Recommendation H.28x Far End Motorised Control |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This Recommendation covers the elements and procedures to support Far End Motorised Control (FEMC) protocol via as a simplex application mechanism as specified in Recommendation H.224.  The FEMC protocol is designed to operate in both point-to-point and multipoint simplex modes.

# Introduction

The original proposal for Far End Motorised Control was presented to the last Study Group Meeting in Geneva 14 – 25 January 2013 to Q2 it was then referred to Q20 where an Adhoc Group was formed to discuss how to take the work forward. Out of that Adhoc Group meeting [76-PLEN] it was proposed to:

- To keep in scope with SG 16, the new work should be limited to control of motorized conferencing systems;

- The experts present felt that there needed to be at least two documents.

  - An F series document to define the scope and terminology.

  - An H series document to define the procedures related to Multimedia Systems.

- Experts noted the similarity to existing far end camera control protocols in ITU-T H.281. It was also noted that interworking with H.320, H.323, H.324 and SIP might be facilitated with the use of ITU-T H.224. For these reasons, it was envisaged that further contributions be assigned to Q1/16.

# Discussion

The purposed of this document is to support the conclusion 2b and 3 of the adhoc group meeting and to propose procedures of far end motorised controls within multimedia systems.

# Conclusion

A proposed new recommendation for Far End Motorised Control (FEMC) is attached herein.

Proposed New Recommendation ITU-T H.28x

A Far End Motorised Control Protocol using H.224 in Multimedia Systems

# 1	Scope

This Recommendation covers the elements and procedures to support Far End Motorised Control (FEMC) protocol via as a simplex application mechanism as specified in Recommendation H.224.  The FEMC protocol is designed to operate in both point-to-point and multipoint simplex modes.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.224]	Recommendation ITU-T H.224 (2005), A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1   simplex protocol [ITU-T H.224]: A communication protocol that is purely one-way, and where acknowledgements are not part of any application protocol. Error control is either not present, or is accomplished via forward error correction

3.1.2   H.224 client management entity [ITU-T H.224]:  A H.224 data link client that send a complete list of locally registered clients and their optional extra capabilities.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	  FEMC commands: Atomic indexed requests to act on a remote motorised device.

3.2.2   FEMC client: An entity that uses the data transfer services of the Data Link layer to control far end motorised functionality.

3.2.3   FEMC ControlPoint: A unique remote motorised functionality to control.

3.2.4   FEMC Positioning Mode:  The method or format of the remote instruction.

3.2.5   FEMC View Port:  The ControlPoint values for the current video display window.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

CME           Client Management Entity

CP               ControlPoint

FEMC	Far End Motorised Control

# 5	Conventions

–	Session – A session is a logical connection between two or more devices for the purpose of exchanging remote motorised control instructions on a real-time basis.

–	Channel – Reference to a H.224 Simplex Channel as defined in [ITU-T H.224].

6           Control Overview

Each ControlPoint shall be capable of being controlled independently. The limits and positioning shall be defined in terms of representative positive translational linear units. The device containing the motorized function shall advertise in the CME Extra Capabilities an appropriate linear scale representation of the motorised function. The remote is not required to understand the actual units or scale but only the limits, step size and current position appropriate for initialising a user input control such as a dial or slider. The device with the motorized function shall be capable of translating linear represented instruction into appropriate motorized functionality.

# 7	Protocol

This Recommendation only addresses the session contents. The procedures used to establish a session are beyond the scope of this Recommendation.

It is recommended that FECC messages are sent as HIGH PRIORITY datagrams to the underlying Data Link layer.

Each ControlPoint shall operate in one of the positioning modes specified in Table 1.

Table 1/H.28x  FEMC Modes

| Positioning Mode | Details |
| --- | --- |
| Simple | Basic stop/stop/continue instruction |
| Relative | Instruction to step a number for pre-defined unit sizes |
| Absolute | Instruction to move to an absolute position |

## 7.1	CME Extra Capabilities Message

The FEMC Client capabilities are sent by the H.224 Client Management Entity in the form of a CME Extra Capabilities message. The FEMC client at time of registration with the H.224 layer shall submit a data block encoded as per the following field formats.

Each ControlPoint shall advertise a capability definition as specified in Figure 1.

0                                   1                            2                           3

0 1 2 3 4 5 6 7   8   9   0 1 2 3 4 5  6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

| CP | P | V | X | R | reserved | Step |
| --- | --- | --- | --- | --- | --- | --- |
| Min Limit (optional) |  |  |  |  |  |  |
| Max Limit (optional) |  |  |  |  |  |  |
| V Min (optional) |  |  |  |  |  |  |
| V Max (optional) |  |  |  |  |  |  |

Figure 1/H.28x  FEMC Capability Définition

The fields of the capability definition are defined as follows:

CP:            ControlPoint Identifier (Refer Table 2)

P:               Positioning Mechanism 0- Simple and Relative and 1- Absolute

V:              ViewPort. Whether support display viewing window.

X:              Supports current position indication

R:              Protocol Revision shall be set to 1

Step:          Step Size. Set to 0 for Simple and Absolute.

Min Limit: (optional) Used with P=1 to indicate minimum absolute limit

Min Limit: (optional) Used with P=1 to indicate maximum absolute limit

V Min         (optional) Used with V=1 to indicate current view minimum value

V Max      (optional) Used with V=1 to indicate current view maximum value

Table 2/H.28x  ControlPoint Identifiers

| ControlPoint Identifier | Motorised Control |
| --- | --- |
| 0x00 | Zero Reset all Control Points |
| 0x01 | Movement Forward/Reverse |
| 0x02 | Movement Left/Right |
| 0x03 | Movement Up/Down |
| 0x04 | <reserved> |
| 0x05 | <reserved> |
| 0x06 | <reserved> |
| 0x07 | Neck Pan |
| 0x08 | Neck Tilt |
| 0x09 | Neck Roll |
| 0x0a | <reserved> |
| 0x0b | <reserved> |
| 0x0c | Camera Zoom |
| 0x0d | Camera Focus |
| 0x0e | Camera Lighting |
| 0x0f | Auxiliary Lighting |
| 0x10-EF | <reserved> |
| 0xF0 – 0xFF | User defined |

## 7.2	FEMC Instruction Message

0                                  1                            2                           3

0 1 2 3 4 5 6 7   8 9 01 2 3 4 5  6 7 8 9 0 1 2 3   4 5   6 7 8 9 0 1

| CP | Instruction ID | A | <reserved> |
| --- | --- | --- | --- |
| Position (optional) |  |  |  |

Figure 1/H.28x  FEMC Instruction Définition

The fields of the Instruction definition are defined as follows:

CP:                      ControlPoint Identifier (Refer Table 2)

InstructionID:    Identifier to uniquely identify the instruction (values 1 to 65535).

Action:                Action to start/stop a simple instruction. Used as a directional indicator for Absolute and Step positioning (Refer to Table 3)

Step Count:    Number of Steps to increment (ignore for Simple and Relative)

Position:         New absolute position (optional only mandatory for Absolute positioning)

Table 3/H.28x  Motorised Instructions

| Action | Motorised Control |
| --- | --- |
| 0x0 | Stop |
| 0x1 | Start/Move Positive |
| 0x2 | Start/Move Negative |
| 0x3 | Continue |

## 7.3	FEMC Current Position

Where a ControlPoint supports sending current postion indications, the motorised device may send current position indications from time to time to update the remote device as to the current position of the ControlPoint.

0                                   1                            2                           3

0 1 2 3 4 5 6 7   8   9   01 2 3 4 5  6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

| CP | <reserved> |
| --- | --- |
| Current Position (optional) |  |

Figure 1/H.28x  FEMC Current Pointion Indication

## 7.4	Procedures for Simple Positioning Mode

Each Instruction shall contain at a minimum a Start (either positive or negative) and a Stop Action for a given InstructionID. The Step Count shall be set to 0 and the position field omitted. A Continue Action shall be sent at intervals of no more than T1 and if a stop or continue action is not received by T2 it shall be deemed the instruction has ended and a local Stop action shall be invoked. Timer values are specified in Table 4.

Table 4/H.28x  Simple Positioning Timers

| Timer | Duration |
| --- | --- |
| T1 | 250ms |
| T2 | 500ms |

If a new InstructionID is received before the previous InstructionID Stop Action has been received then the previous instruction shall be deemed to have ended and a local Stop Action invoked prior to invoking the new instruction.

## 7.5	Procedures for Step Positioning Mode

Each Instruction shall contain at a Start (either positive or negative) and a step count value greater than 0. The position field shall be omitted. The Step field shall indicate the count of step values as specified in the ControlPoint’s capability definition.

## 7.6	Procedures for Absolute Positioning Mode

Each Instruction shall contain at a Start/Move Positive Instruction with a step count value of 0. The position field shall contain the absolute location to reposition. The absolute value shall be within the bounds specified in Min/Max Limit fields of the ControlPoint’s capability definition. The FEMC client may send multiple instructions with the same InstructionID to ensure at least 1 instruction is received over error prone networks. Duplicate instructions received with the same InstructionID shall be ignored.

7.7      Interaction between ControlPoints

Each ControlPoint may be controlled independently or may be instructed together to perform a desired operation. To instruct ControlPoints as a group, each instruction shall be included in the same H.224 frame.

_________