| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4360 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | H.323 Associative Simple Endpoint Type (ASSET) Definition |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document proposes a new work item to define a new H.323 “Associative” SET (Simple Endpoint Type) to be included as a new Annex to H.323. This document should be read in conjunction with AVD-4359 and AVD-4361.

# Introduction

The proposal to add support to H.323 as a stepping stone to H.325 functionality was first proposed at the last rapporteur meeting in Brisbane. Out of that meeting the first draft of a H.460 based protocol document was put forward at the meeting in Geneva in January. It was noted at that meeting…

[TD-51]

It was suggested that perhaps what we need is a framework document for introducing "H.323 Application Entities" like file transfer applications or whiteboard applications.  This might be a "profile" of H.323 similar to the "Simple Endpoint Type" defined in H.323 Annex F.  Perhaps most important is the removal of the requirement that an H.323 Application Entity support G.711, since most such applications, would not need audio or video support, as the user's primary H.323 terminal would provide that capability.  Use of audio or video should not be precluded, though.  If the H.323 Application Entity is a video display, for example, it would need to utilize video.  The framework document would explain how to use call-independent supplementary service messages to allow association of applications and terminals, etc.  The framework would probably be an Annex to H.323.  Contributions are solicited.

It was decided not to add this as a work item to the work programme at this meeting, though there is interest in progressing this and the framework document once it is prepared. Further, definition of a protocol and application protocol entities are also solicited.

# Discussion

The author has taken note of the experts review and developed this contribution within in the framework of a H.323 “Simple Endpoint Type” as proposed.

# Conclusion

The following contribution proposes the outline definition of an H.323 “Associative” simple endpoint type (ASSET) to be included as a new annex to H.323.

Annex ?

Associative simple endpoint type

(This annex forms an integral part of this Recommendation)

# ?.1	Introduction

An H.323 Associative SET device (ASSET) is a device within a H.323 network that is able to provide additional capabilities beyond those of any regular H.323 terminal devices. For instance, an H.323 device capable of only audio may connect with an H.323 Associative SET (ASSET) on another device to share a H.239 presentation within the context of a call initiated/received on the audio device or a File transfer application maybe invoked on a separate device to send a file within the context of an on-going H.323 call.

## ?.2	Scope

The scope of this annex is to define Associative simple endpoint type.

## ?.3	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.323]	Recommendation H.323 (2009), Packet-based multimedia communications systems.

[ITU-T H.225.0]	Recommendation H.225.0 (2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.235]	Recommendation H.235 (2003), Security and encryption for H-Series 
(H.323 and other H.245-based) multimedia terminals.

[ITU-T H.245]	           Recommendation H.245 (2011), Control protocol for Multimedia Communications.

[ITU-T H.460.1]	Recommendation H.460.1 (2002), Guidelines for the Use of the Generic Extensible Framework.

[ITU-T H.460.18]	Recommendation ITU-T H.460.18 (2005), Traversal of H.323 signalling across network address translators and firewalls.

[ITU-T H.460.19]	Recommendation ITU-T H.460.19 (2005), Traversal of H.323 media across network address translators and firewalls.

[ITU-T H.46x.0]	     Recommendation H.46x.0 (201x), H.323 Associative Set (ASSET) Feature

# ?.4	Definitions

### ?.4.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

?.4.1.1 Simple Endpoint Type: As defined in Annex F.

### ?.4.2	Terms defined in this Recommendation

### ?.4.2.1	Application:  A real-time function which resides on an Associative Simple Endpoint Type.

## ?.5	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ASSET 		    Associative Simple Endpoint Type.

## ?.6	Requirements for Associative Simple Endpoint Type

The requirements for an ASSET can be divided into two parts.

- Interface between the ASSET and the Endpoint.

- Interface between the ASSET and another ASSET.

### ?.6.1 	Endpoint interface requirements

The Endpoint Interface shall support the following:

- Advertise with H.225.0 endpointType of SET (bit 2 set)

- Full H.225.0 RAS support including full registered to a gatekeeper.

- H.225.0 Call Signalling messages SETUP, CONNECT, FACILITY and RELEASECOMPLETE.

- Only support conferenceGoal element set to callIndependentSupplementaryService.

- No H.245 Support.

- No H.450 Support.

- H.460.18 NAT Signalling support.

- H.46x.0 as a required feature.

### ?.6.2 	ASSET interface requirements

The ASSET Interface shall support the following:

- Advertise with H.225.0 endpointType of SET (bit 2 set)

- Full H.225.0 Call Signalling.

- Full H.245 Support.

- No mandatory H.245 Capability requirements.

- H.460.18/.19 NAT Traversal Support.

- H.46x.0 as a required feature.

## ?.7	Procedures for Associative Simple Endpoint Type

Implementation of the Associative Simple Endpoint Type feature is defined in H.46x.0

__________________