| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4372 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.39 (Rev): clause 6.15.9 - Update proposals |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update H.248.39 with scope of missining text in clause 6.15.9. Initial text is suggested to complete that section.

Wildcarding use cases for the last two SDP attributes seems to be questionable (see proposed editor’s notes), but might be formally specified.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 48 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

### 6.15.8	Media description with multiple payload types and multiple attribute lines

Clause 6.11 shows a wildcarding example with multiple list entries in <fmt list> in the "m=" line.

The fourth "m=" line parameter <fmt list> is a list of one or more payload formats. This subclause is considering another wildcarding example with multiple list entries in <fmt list>.

The unambiguous resource selection by the MG may require correspondent "a=" lines in case of wildcarding in the list of formats in the "m=" line. This is reflected in 7.1.8/H.248.1.

NOTE 1 – To avoid ambiguity when requesting the MG to reserve and commit resources, the MGC should supply as much information as needed when using under-specification (i.e., CHOOSE) so that the MG can make an unambiguous selection. For example, when using CHOOSE without specifying the required application type (e.g., "media name" in case of SDP encoding), further information may be needed (e.g., attribute lines in case of SDP encoding).

Such a wildcarding configuration must therefore consider the combination of a single "m=" and multiple "a=" lines.

NOTE 2 – Clause 6.1.1 has examples with scope on "m=" line only.

### 6.15.9	Source-specific media attributes

Source-specific media attributes are defined in [IETF RFC 5576]. These attributes may be used for to describe RTP media sources in SDP, to associate attributes with these sources, and to express relationships among such traffic sources.  It also defines several source-level attributes that can be used to describe properties of media sources.

Structure of [IETF RFC 5576] SDP attributes:

- Media Attributes:

  - the "ssrc" media attribute (see clause 6.15.9.1)

  - the "ssrc-group" media attribute (see clause 6.15.9.2)

- Source Attributes

  - the "cname" source attribute (see clause 6.15.9.3)

  - the "previous-ssrc" source attribute (see clause 6.15.9.4)

  - the "fmtp" source attribute (see clause 6.15.9.5)

{Editor’s comment (2013-06 meeting): we’ve noticed that the two media attributes specify entire “SDP line structures”, whereas the source attributes define just individual fields within a SDP line. We are not sure whether it make sense to indicate wildcarding on isolated SDP line fields, guess H.248.39 should rather define SDP wildcarding at the level of SDP line structures (to be confirmed).
In that case: subclause 6.15.9.3 to 6.15.9.5 would be removed again and examples would be added/merged in clauses 6.15.9.1 & 6.15.9.2. Feedback would be appreciated.}

#### 6.15.9.1	The "ssrc" media attribute

See Table 6-15.9.1:

Table 6-15.9.1/H.248.39 – Source-specific media attribute “ssrc”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 4.1/[IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[IETF RFC 5576]) | a=ssrc:<ssrc-id> <attribute> a=ssrc:<ssrc-id> <attribute>:<value> Formal: ssrc-attr = "ssrc:" ssrc-id SP attribute ; The base definition of "attribute" is in RFC 4566. ; (It is the content of "a=" lines.) ssrc-id = integer ; 0 .. 2**32 - 1 attribute =/ ssrc-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:? ? a=ssrc:? ? ? |  | Are invalid. (TBC) Because there are multiple options for the additional attribute. The MG could theoretically select any, which doesn’t make really sense (?)- |  |
| a=ssrc:? |  | Would give a=ssrc:ssrc-id |  |
| a=ssrc:? cname:? |  | Would give a=ssrc:ssrc-id cname:cname i.e., - MG assigned SSRC value at the level of a stream endpoint or termination and - MG used CNAME value (typically at the level of root), which is either provisioned or locally generated, based on [IETF RFC 6222]. |  |

#### 6.15.9.2	The "ssrc-group" media attribute

See Table 6-15.9.2:

Table 6-15.9.2/H.248.39 – Source-specific media attribute “ssrc-group”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 4.1/[IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[IETF RFC 5576]) | a=ssrc-group:<semantics> <ssrc-id> ... Formal: ssrc-group-attr = "ssrc-group:" semantics *(SP ssrc-id) semantics = "FEC" / "FID" / token ; Matches RFC 3388 definition and ; IANA registration rules in this doc. token = <as defined in RFC 4566> attribute =/ ssrc-group-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc-group:? ? a=ssrc-group:? … ? |  | Are invalid. (TBC) Because there are semantical limitation concerning the group semantics according clause 4.2/[IETF RFC 5576]. E.g., meaningful tokens are "FEC" and "FID", but other tokens such "LS", "SFR", "ANAT" or "CS" are normally not applicable in this service context. |  |
| a=ssrc-group:token ? … ? |  | Would give a=ssrc-group:token list of ssrc-ids |  |

Example:

Wildcarding a=ssrc-group in conjunction with [IETF RFC 4588] specified MIME subtype rtx it as follows:

a=ssrc-group:FID $ $

a=ssrc:$ cname:$

a=ssrc:$ cname:$

where MG then allocates one “original stream SSRC” and one “retransmission stream SSRC” and where is returns the “original stream SSRC” in the first a=ssrc attribute lines and the “retransmission stream SSRC” in the second a=ssrc attribute line like

Replied Local Descriptor:

...

a=ssrc-group:FID <original-stream-SSRC> <retransmission-stream-SSRC>

a=ssrc:<original-stream-SSRC> cname:<MG-CNAME>

a=ssrc:<retransmission-stream-SSRC> cname:<MG-CNAME>

#### 6.15.9.3	The "cname" source attribute

See Table 6-15.9.3:

Table 6-15.9.3/H.248.39 – Source-specific attribute “cname”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.1/[IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[IETF RFC 5576]) | a=cname:<cname> Formal: cname-attr = "cname:" cname cname = byte-string ; Following the syntax conventions for CNAME as defined in RFC 3550. ; The definition of "byte-string" is in RFC 4566. attribute =/ cname-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=cname:? |  | Would give the a=cname:cname ## i.e., - MG used CNAME value (typically at the level of root), which is either provisioned or locally generated, based on [IETF RFC 6222]. |  |

#### 6.15.9.4	The "previous-ssrc" source attribute

See Table 6-15.9.4:

Table 6-15.9.4/H.248.39 – Source-specific attribute “previous-ssrc”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.1/[IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[IETF RFC 5576]) | a=ssrc:<ssrc-id> previous-ssrc:<ssrc-id> ... Formal: previous-ssrc-attr = "previous-ssrc:" ssrc-id *(SP ssrc-id) attribute =/ previous-ssrc-attr |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:? |  | Is invalid. |  |
| a=ssrc:? previous-ssrc:? … |  | Would give the a= ssrc: ssrc-id previous-ssrc: ssrc-id … |  |

{Editor’s note (2013-06 meeting): we are not sure about real world use cases due to SSRC changes based on SSRC collision detection. Hm?}

#### 6.15.9.5	The "fmtp" source attribute

See Table 6-15.9.5:

Table 6-15.9.5/H.248.39 – Source-specific attribute “fmtp”

| SDP Specification: |  |  |  |
| --- | --- | --- | --- |
| Definition: (Clause 6.1/[IETF RFC 5576]) | – |  |
| ABNF: (Clause 10/[IETF RFC 5576]) | a=ssrc:<ssrc> fmtp:<format> <format specific parameters> Formal: not part of [IETF RFC 5576] |  |
| H.248/SDP encoding examples with applied wildcarding: |  | Results: |  |
| a=ssrc:? fmtp:? ? |  | {Editor’s note (2013-06 meeting): we are not sure about real world use cases …. Hm?} |  |
| ## a=ssrc:? fmtp:<format> <format specific parameters> |  | ## Would give a=ssrc:ssrc-id fmtp:<format> <format specific parameters> |  |

Table 6-15.8/H.248.39 – Examples

| H.248/SDP encoding example with applied wildcarding: | Result: |
| --- | --- |
| H.248 request: Local{ v=0 c=IN IP4 $ m=audio$ RTP/AVP $ $ a=rtpmap:$ G729D/8000 a=rtpmap:$ G726-16/8000 a=ptime:10 } | H.248 reply: Local{ v=0 c=IN IP4 11.9.19.65 m=audio 5000 RTP/AVP 98 99 a=rtpmap:98 G729D/8000 a=rtpmap:99 G726-16/8000 a=ptime:10 } |

In this example the MGC specifies the encoding name for the two RTP payload types. Thus the MG cannot choose a different encoding name. If the MGC would not send the "a=" line, the MG could choose the encoding name as well. This latter option is not preferred.

[End Proposed Correction]

____________________