| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4373 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for clauses 1 to 4 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 1 to 4. The concrete updates are related to the parallel contributions for progressing H.248.DPI.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

AAP Summary

Recommendation ITU-T H.248.DPI has in scope ITU-T H.248 media gateways with support of packet inspection capabilities. This Recommendation describes the relation of deep packet inspection (DPI) to existing ITU-T H.248 protocol capabilites concerning packet processing, and defines new ITU-T H.248 packages with the specific scope of DPI.

Summary

Recommendation ITU-T H.248.DPI has in scope ITU-T H.248 media gateways with support of packet inspection capabilities. This Recommendation describes the relation of deep packet inspection (DPI) to existing ITU-T H.248 protocol capabilites concerning packet processing, and defines new ITU-T H.248 packages with the specific scope of DPI.

The proposed ITU-T H.248 packages should serve various different network-level use cases and provide inherent flexibilty due to the core design concept with regards to the separation of encoding and transport of DPI policy rules.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 1	Scope

An ITU-T H.248 media gateway (MG) provides operations on packet traffic in general, related to the ephemeral (and root) termination type. A number of existing H.248 packages includes the inspection of packets as part of the overall defined service (such as gate management and packet filtering services according [ITU-T H.248.43]).

This Recommendation defines additional H.248 capabilities for so called deep packet inspection (DPI) related services. Understanding and definition of DPI in this Recommendation follows principally [ITU-T Y.2770]. The “DPI concept” is fundamentally coupled with the architecture of layered protocol stacks, as expressed by the acronym words ‘deep’ and ‘packet’. Figure 1 depicts DPI at the example of [ITU-T X.200] layered protocol architecture, and also illustrates the historical evolution from shallow packet inspection (SPI) over media depth packet inspection (MPI) towards DPI. It should be noted that SPI and MPI are colloquial terms, whereas [ITU-T Y.2770], clause 3.2.5 defines DPI. Further, the notion of ‘packet’ is used in the broadest sense, typically nowadays focusing on IP packets [IETF RFC 791], but covering non-IP traffic as well (see also clause 1.1/[ITU-T Y.2770]).

Figure 1 – Packet Inspection – Example of DPI, here layered architecture according OSI-BRM [ITU-T X.200] and the example when DPI covers also layer 2

This Recommendation

- clarifies the understanding of DPI versus non-DPI in context of H.248.x-series of Recommendation (clause 5):

- describes the relation of “policy rule” support by existing H.248 packages (clause 6); and

- defines new H.248 packages for flexible DPI support in various network scenarios (clauses 7 to 9).

This Recommendation provides also complementary information as part of some Appendices to various aspects of H.248-supported DPI.

## 1.1	Applicability statements

As outlined by Figure 1, DPI is not a new technology from scratch, rather a continuous evolution of existing packet processing functions. Principle interaction with existing packet inspection technologies needs to be considered when using this Recommendation.

### 1.1.1	This Recommendation with other H.248.x-series of Recommendation with policy rule support

See clause 6.2.

### 1.1.2	This Recommendation with non-H.248 policy specification languages

The usage of non-H.248 policy specification languages (PSL) is a fundamental concept behind the defined H.248 capabilities for DPI support by the H.248 packages of this Recommendation. See also clause 6.4, Appendix III and IV.

However, any normative guidance for the usage of such PSLs is out of scope of this Recommendation.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T X.200]	Recommendation ITU-T X.200 (1994) | ISO/IEC 7498-1:1994, Information technology. Open Systems Interconnection. Basic reference model: The basic model.

[ITU-T Y.2770]	Recommendation ITU-T Y.2770 (12/2012), Requirements for Deep Packet Inspection in Next Generation Networks.

[IETF RFC 791]	IETF RFC 791 (1981), Internet Protocol.

{Editor’s note: add all missing references when preparing the meeting output draft!}

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

# 3	Definitions

This Recommendation defines the following terms:

## 3.1	Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

3.1.1	deep packet inspection (DPI) [ITU-T Y.2770]: Analysis, according to the layered protocol architecture OSI-BRM [ITU-T X.200], of

- payload and/or packet properties (see list of potential properties in clause 3.2.11/[ITU-T Y.2770] deeper than protocol layer 2, 3 or 4 (L2/L3/L4) header information, and

- other packet properties

in order to identify the application unambiguously.

NOTE – The output of the DPI function, along with some extra information such as the flow information, is typically used in subsequent functions such as reporting or actions on the packet.

3.1.2	DPI policy rule [ITU-T Y.2770]: The policy rule pertinent to DPI (see also clause 3.1.2/[ITU-T Y.2770]). In this Recommendation, a DPI policy rule is referred to simply as a rule.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

None.

{Contributor’s note: this term was finally worked out by SG13, initially planed to be part of Y.2770, and in the meanwhile expected to be documented in a Supplement (as part of draft Y.SupTerm). However, this is just background information, the term is presently not foreseen to be used in H.248.DPI.}

[End Proposed Correction]

Change proposal #5:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

| A | Address |
| --- | --- |
| AMR | Adaptive-Multi Rate (codec) |
| Bp | Bucket size of peak token bucket [ITU-T H.248.53] |
| Bs | Bucket size of sustainable token bucket [ITU-T H.248.53] |
| BT | (RTCP) Block Type |
| DA | (IP) Destination Address |
| DPI | Deep Packet Inspection |
| DS | Differentiated Services |
| FD | Flow Dependent |
| FI | Flow Independent |
| GBRA | Generic Byte Rate Algorithm |
| HTTP | Hypertext Transfer Protocol |
| IM | Instant Messaging |
| LD | Local Descriptor (H.248 protocol) Local Destination (H.248 bearer connection endpoint) |
| LxHI | Layer x Header Inspection (see also clause 3.2.19/[ITU-T Y.2770]) |
| LyPI | Layer y Payload Inspection (see also clauses 3.2.21 to 3.2.23 in [ITU-T Y.2770]) |
| Lz+ | Layer(s) above z (e.g. L4+) |
| MIME | Multipurpose Internet Mail Extensions |
| MPI | Medium depth Packet Inspection |
| MSRP | Message Session Relay Protocol |
| OGP | Open Game Protocol |
| P | (L4) Port |
| P2P | Peer-to-Peer |
| PDU | Protocol Data Unit |
| PCI | Protocol Control Information |
| PSL | Policy Specification Language |
| PT | (RTP) Payload Type (RTCP) Packet Type |
| RD | Remote Descriptor (H.248 protocol) Remote Destination (H.248 bearer connection endpoint) |
| Rp | Rate of peak token bucket [ITU-T H.248.53] |
| Rs | Rate of sustainable token bucket [ITU-T H.248.53] |
| RS | Remote Source (H.248 bearer connection endpoint) |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| SA | (IP) Source Address |
| SMTP | Simple Mail Transfer Protocol |
| SIP | Session Initiation Protocol |
| SP | (IP) Source Port |
| SPI | Shallow Packet Inspection |
| TC | (IPv6) Traffic Class |
| TCP | Transmission Control Protocol |
| TFTP | Trivial File Transfer Protocol |
| TLS | Transport Layer Security |
| ToS | (IP) Type of Service |
| TTL | (IP) Time To Live |
| UDP | User Datagram Protocol |
| XML | Extensible Markup Language |
| XMPP | Extensible Messaging and Presence Protocol |

[End Proposed Correction]

____________________