| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4375 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for packages (clauses 6 and 7) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 6 and 7.

Discussion and conclusions

This contribution aims the stabilization of the package designs. The design principles take into account similar package designs as H.248.69, § 13 “Message Filtering Package” and H.248.9.

We’d like to propose following suggestions as a concrete way forward:

- To merge the “pointer to rule” and “container for rule” methods in single package as a base service. Separate packages are not needed because both capabilities could be used in parallel. Both represent just different and disjoint signalling methods.
The base package would be named as inspection rule base package.

- Further, in order to provide some initial tools for observing the policy rule enforcement process in the MG by the MGC, we are proposing an inspection rule operational package as an extension package.
The package defines statistics for some major policy rule actions as well as one statistic and event related to the packet identification process.

- The proposed two packages satisfy the requirements of a basic DPI service, similar as H.248.43 concerning IP traffic filtering.
We’d like to include a new clause 9 concerning the (future) option of grouping DPI policy rules, given the experience with H.248.76.
We’ve not yet the intention to concrete already a correspondent package in the first release of this Recommendation.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 7	Inspection rule base package

|  | Package Name: | Inspection rule base package |
| --- | --- | --- |
|  | Package ID: | irb (0x????) |
|  | Description: | This package allows the MGC to indicate to the MG DPI policy rules. The DPI policy rules and behaviour are out of scope of this Recommendation. Such rules could be e.g. based on "SNORT" (see Appendix IV) as a typical policy rule specification language for DPI. {this info might be moved to clause 6.3?} This package defines a minimum service by two properties, representing two principle rule signalling variants (see more in Appendix III). The properties may be associated with Root and non-Root terminations. Property values may be either signalled by the MGC or provisioned via management actions. The base package capabilities allow therefore a basic service. Example: a very minimum DPI service support (from MGC perspective) might be the provisioning of an irb package property via management plane (i.e., configuration manangement action for DPI policy rule(s)) with rule-embedded management plane reporting only. Such an application is also known as “overlay DPI”. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Pointer to rule conditions

|  | Property Name: | Pointer to rule conditions |
| --- | --- | --- |
|  | Property ID: | ptr (0x0001) |
|  | Description: | This property allows the MGC to indicate which rule conditions should be used for inspection of incoming packets by referring to a pointer to a data object which contains DPI policy rule conditions. The pointer is in a URI format which includes also a generic name (which may represent a globally or locally unique identifier). The applied pointer scheme is subject of an H.248 profile specification. E.g., the URI format could follow a URI scheme such as “file”, “ftp”, “https”, etc. NOTE – The concept of a “pointer to a local or remote data object” is also supported by [ITU-T H.248.9] and [ITU-T H.248.69] (see e.g. clause 13.1.1, property “Incoming Message Filters”) with concrete data object types “media”, “message” and “SIEVE script” respectively. |
|  | Type: | String |
|  | Possible values: | The string is of type "URI". [IETF RFC 3986] See: http://www.iana.org/assignments/uri-schemes.html |
|  | Default: | None |
|  | Defined in: | LocalControl, TerminationState (in case of root termination) |
|  | Characteristics: | Read/Write |

### 7.1.2	Container for rule

|  | Property Name: | Container for rule |
| --- | --- | --- |
|  | Property ID: | # cfr (0x0002) |
|  | Description: | # This property contains a data object that is applied to incoming packets. # The data object represents a DPI policy rule in general (see Appendices I, II and III). Such a DPI policy rule object could internally be structured as simple, compound, hierarchical, etc DPI policy rules. Syntax and semantics of such DPI policy rule objects are out of scope of this Recommendation. # NOTE 1 – Appendix IV indicates possible examples of policy rules for packet inspection. # NOTE 2 – The concept of a “container for a rule data object” is also supported by [ITU-T H.248.69] (see e.g. clause 13.1.2, property “Incoming Message Filters by Reference”) with concrete data object type “SIEVE script”. |
|  | Type: | String |
|  | Possible values: | As per the grammar of the applied policy rule specification language(s). |
|  | Default: | An empty string |
|  | Defined in: | LocalControl, TerminationState (in case of root termination) |
|  | Characteristics: | Read/Write |

## 7.2	Events

## None.

## 7.3	Signals

None.

## 7.4	Statistics

None.

## 7.5	Error Codes

None.

## 7.6	Procedures

### 7.6.1	Enforcement of policy rules for packet inspection

Based on a user profile, service profile or/and MGC policy, the MGC may set packet level inspection rules at an individual termination/stream level or the root termination.

#### 7.6.1.1	via explicit rule signalling in native H.248 syntax

This method is out of scope of this package, and relates rather to existing H.248 tools as indicated by clause 6.2.

#### 7.6.1.2	via explicit rule signalling using container method

The MGC enforces packet inspection by transfering DPI policy rules to the MG via the cfr property.

#### 7.6.1.3	via pointer to rule signalling and MG-locally stored rules

The MGC enforces packet inspection by referring correspondent DPI policy rules via the ptr property.

#### 7.6.1.4	via pointer to rule signalling and MG-remotely stored rules

The MGC enforces packet inspection by referring correspondent DPI policy rules via the ptr property.

#### 7.6.1.5	Parallel usage of explict and pointer based rule signalling

The MGC may use both methods in parallel, taken into account possible rule interactions.

### 7.6.2	Unsuccessful packet inspection process

The enforcement of policy rules for packet inspection might be principally not successful. The base package provides not any means for detailed feedbacks and diagnosis support. The MG should rather reply with existing H.248 error codes.

Applications with requirements for more detailed feedback information from the packet inspection process should additionally use the inspection rule operational package (see next clause 8).

NOTE – Unsuccessful packet inspection operations could also lead to management plane reporting events (e.g. fault management) besides MGC notifications, but this is out of scope of this Recommendation.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 8	Inspection rule operational package

|  | Package Name: | Inspection rule operational package |
| --- | --- | --- |
|  | Package ID: | iro (0x????) |
|  | Description: | This package supports the MGC with information related to the operation of enforced DPI policy rules in the MG. The scope of this package version is limited on operational information concerning principal feedback from packet inspection process with focus on - whether packets are successfully identified (and if, by which policy rule); - possible rule errors; and - high level performance metrics related to some basic packet handling events. |
|  | Version: | 1 |
|  | Extends: | irb v1 |

## 8.1	Properties

None.

## 8.2	Events

### 8.2.1	Satisfied policy rules

|  | Event Name: | Satisfied policy rules |
| --- | --- | --- |
|  | Event ID: | src (0x????)> |
|  | Description: | This event allows the MGC to be notified about satisfied policy rules conditions, as part of an enforced DPI policy rule. It is supposed that DPI policy rules contain an embedded rule identifier (see e.g. Figure III.1). |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	Policy rule filter

|  | Parameter Name: | Policy rule filter |
| --- | --- | --- |
|  | Parameter ID: | prf (0x0001) |
|  | Description: | This parameter allows the MGC to select the list of DPI policy rules. The MG will check all enforced DPI policy rules according the inspection rule base package usage. The MG reporting is limited to successfully checked DPI policy rules according the specified prf filter. |
|  | Type: | Sub-list of String |
|  | Optional: | Yes |
|  | Possible values: | The possible values are determined according to the applied DPI policy rule specification language(s) (PSL). The MG should be inherently aware of that grammar. |
|  | Default: | Empty list (i.e., the MGC shall not be notified) |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	<Parameter Title>

|  | Parameter Name: | Detected policy rules |
| --- | --- | --- |
|  | Parameter ID: | dpr (0x????)> |
|  | Description: | This parameter indicates the list of successfully checked DPI policy rules. |
|  | Type: | see above |
|  | Optional: | No |
|  | Possible values: | same value range as in clause 8.2.1.1.1 |
|  | Default: | None. |

### 8.2.2	Inspection runtime error

|  | Event Name: | Inspection runtime error |
| --- | --- | --- |
|  | Event ID: | ire (0x0002) |
|  | Description: | This event allows the MG to send a notification to the MGC indicating that an error has occurred during the execution of DPI policy rule(s) against incomping packet flows. |

#### 8.2.2.1	EventsDescriptor parameters

None.

#### 8.2.2.2	ObservedEventsDescriptor parameters

##### 8.2.2.2.1	Unsuccessfull policy rule

|  | Parameter Name: | Unsuccessfull policy rule |
| --- | --- | --- |
|  | Parameter ID: | upr (0x????) |
|  | Description: | There might be one or multiple DPI policy rules enforced at the MG. This parameter indicates which particular DPI policy rule(s) generated an error. |
|  | Type: | Sub-list of String |
|  | Optional: | Yes |
|  | Possible values: | same value range as in clause 8.2.1.1.1 |
|  | Default: | None |

## 8.3	Signals

None.

## 8.4	Statistics

### 8.4.1	Identified packets

|  | Statistic Name: | Identified packets |
| --- | --- | --- |
|  | Statistic ID: | idpa (0x????)> |
|  | Description: | Provides the number of successfully identified packets (from the ingress traffic flow) on the termination or stream since the statistic has been set. The packets represent the ingress protocol data units (PDU) of all packet flows of an H.248 Stream (or root termination). The semantic of “packet” is related to the applied “packet inspection” process, which is given by the enforced DPI policy rule(s). This statistic is therefore inherently accumulative across all applied rules. Example: “IP packet” in case of policy rule conditions related to IP traffic. Statistic idpa may be then used in conjunction with the “IP packets received” statistics (clause 7.4.2/[ITU-T H.248.61]), e.g., when the packet ratio of successfully identified packets would be interested. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 8.4.2	Discarded packets

|  | Statistic Name: | Discarded packets due to rule actions |
| --- | --- | --- |
|  | Statistic ID: | dipa (0x????)> |
|  | Description: | Provides the number of packets discarded on the termination or stream since the statistic has been set. The packets represent the ingress protocol data units (PDU) of all packet flows of an H.248 Stream (or root termination). The semantic of “packet” is related to the applied “packet inspection” process, which is given by the enforced DPI policy rule(s). The event of packet discard would be the result of correspondent DPI policy rule actions. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 8.4.3	Modified packets

|  | Statistic Name: | Modified packets due to rule actions |
| --- | --- | --- |
|  | Statistic ID: | mopa (0x????)> |
|  | Description: | Provides the number of packets modified on the termination or stream since the statistic has been set. The packets represent the ingress protocol data units (PDU) of all packet flows of an H.248 Stream (or root termination). The semantic of “packet” is related to the applied “packet inspection” process, which is given by the enforced DPI policy rule(s). The event of packet modification would be the result of correspondent DPI policy rule actions. Packet modification could related to packet header and/or payload information. Excluded is the packet destination address information, because subject of statistic repa (clause 8.4.4). |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

### 8.4.4	Redirected packets

|  | Statistic Name: | Redirected packets due to rule actions |
| --- | --- | --- |
|  | Statistic ID: | repa (0x????)> |
|  | Description: | Provides the number of packets redirected on the termination or stream since the statistic has been set. The packets represent the ingress protocol data units (PDU) of all packet flows of an H.248 Stream (or root termination). The semantic of “packet” is related to the applied “packet inspection” process, which is given by the enforced DPI policy rule(s). The event of packet redirect would be the result of correspondent DPI policy rule actions, which relates typically to a change of the packet destination address information. |
|  | Type: | Double |
|  | Possible values: | Any 64-bit integer 0 and up |
|  | Level: | Either |

## 8.5	Error Codes

None.

## 8.6	Procedures

### 8.6.1	General operation

### This package should be used in conjunction with the inspection rule base (irb) package (clause 7) due to the precondition of enabled DPI policy rules in the MG.

### 8.6.2	Results of policy rule enforcement

#### 8.6.2.1	Notification about policy rule detections

The MGC subscribes to this notification service by arming the src event. The list of policy rules could be qualified by the MGC via event parameter prf.

#### 8.6.2.2	Performance monitoring of policy rule enforcement process

Related to the processing of rule conditions:

- Number of successfully identified packets are counted by statistic idpa.

Related to the processing of rule actions:

- Number of discarded packets are counted by statistic dipa.

- Number of modified packets are counted by statistic mopa.

- Number of redirected packets are counted by statistic repa.

### 8.6.3	Runtime errors

The MGC should also set the "inspection runtime error" (iro/ire) event on the applicable stream or termination in order to determine if there are any errors when running DPI policy rules against incoming packet traffic.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 9	Inspection rule group package

The previous packages allow a basic support for DPI. This service may be augmented in future by e.g. an inspection rule group package, similar to the constellation between [ITU-T H.248.43] (“basic filter service”) and [ITU-T H.248.76] (“advanced filter service by filter rule grouping”). A future inspection rule group package could then provide similar properties (as [ITU-T H.248.76]) with respect to the introduction of a “inspection rule context” concept, an “inspection rule group identifier” and a “relative rule order” capability.

Such an advanced DPI support service is out of scope of the initial release of this Recommendation.

[End Proposed Correction]

____________________