| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4376 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for Appendix I |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to keep Appendix I, which serves presently as a living list.

Discussion and conclusions

The information of Appendix I is useful and wasn’t questioned since the inclusion as living list item. Clause 6/H.248.DPI may refer to this Appendix (as proposed by parallel contribution AVD-4374).

That informative text provides a useful description on DPI policy rule aspects from MGC point of view in our opinion.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix I

MGC aspects of DPI policy rules

(This appendix does not form an integral part of this Recommendation)

## I.1	Introduction

Policy rules enforced in a MG are basically related to an associated communication instance, such as a call, session, application, service, dialogue, etc., which may be abstracted as “context” (NOTE 1). There are consequently context-dependent policy rules, whether of type DPI or non-DPI. Context-independent policy rules would be associated with the H.248 Root termination in MGs.

The MGC may principally enforce both, - context-aware and context-unaware policy rule processing -, however, typically the MGC’s prime responsibility is the control of context-aware MG behaviour.

This general background is also valid with regards to DPI policy rule enforcement.

NOTE 1 – The general context concept is here identical to the H.248 (protocol) context element. The “context awareness” of MGs (and MGCs) is synonym to the correspondent capability according clause 3.2.5/[ITU-T Y.2201].

## I.2	MGC-locally vs MGC-remotely generated DPI policy rules

DPI policy rules signalled to the MG by the MGC via H.248 could be either completely produced by the MGC itself, or prepared and provided by other network elements. Below Figure I.1 illustrates these two options of MGC-locally and MGC-remotely generated DPI policy rules. The applied model in a specific network environment may impact the question on the used policy rule format or which signalling option (“container” versus “pointer”) at the H.248 interface.

Figure I.1 – Possible Roles of a Policy Control Server (with embedded MGC entity) - 
MGC-locally (A) vs MGC-remotely (B) generated DPI policy rules
(Example with SIP as application control protocol at MGC level)

There might be a mix of both (A and B) in real networks, and even the additional policy rule enforcement in MGs via management plane actions (see also Figure II.2).

[End Proposed Correction]

____________________