| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4377 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for Appendix II |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to continue with Appendix II.

Discussion and conclusions

The information of Appendix II is refered by the main body of the draft Recommendation. The examples illustrate and serve multiple aspects of DPI-enhanced H.248 MGs. The contribution proposes a transformation into an regular Appendix.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal:

[Begin Proposed Correction]

Appendix II

Examples of DPI and non-DPI policy rules

(This appendix does not form an integral part of this Recommendation)

## II.1	Introduction

### II.1.1	Purpose and overview

Accompanying material for the illustration of typicall DPI use cases. Table II.1 provides an overview of illustrated example policy rules.

Table II.1 – Overview of examples in this Appendix

| No. |  | Name | Clause |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| a) Non-DPI scenarios: |  |  |  |  |  |
| I |  | Packet size range policing | II.2.1.1 |  |
| II | Flow-level byterate policing | II.2.1.2 |
| III | Well-known port SIP message policing | II.2.1.3 |
| IV | General IP 5-tuple policing | II.2.1.4 |
| V | Extensive filtering | II.2.1.5 |
| b) DPI scenarios: |  |  |
| VI | Media type/media format policing | II.2.2.1 |
| VII | Detect application X of an "application-agnostic" bearer | II.2.2.2 |
| VIII | Detect application X = 'BITTORRRENT' | II.2.2.2.1 |
| IX | Detect application X = 'SKYPE' | II.2.2.2.2 |
| X | Detect application X = 'OPEN GAME PROTOCOL' | II.2.2.2.3 |
| XI | Detect application X = 'audio channel in a multi-channel media application' | II.2.2.2.4 |
| XII | Detect & measure application X = 'greek Jabber traffic' | II.2.2.2.5 |
| XIII | TCP attack detection | II.2.2.3 |
| XIV | Remove invalid MIME attachments from Instant Messaging | II.2.2.4 |
| XV | RTCP Block Type Filtering | II.2.2.5 |
| XVI | Application-specific traffic handling | II.2.2.6 |
| XVII | Abnormal packet size detection | II.2.2.7 |
| XVIII | Delete old packets of application X | II.2.2.8 |

### II.1.2	Specification level of rules

Packet inspection may be considered as a packet policing function in general. The particular “inspection function” may be thus formulated as policy rule (see also [ITU-T H.248.79]). The specification depth may differ in various respects:

- High-level

- Low-level rules (using informal specification language, like a prose language)

- Low-level rules (using formal specification grammar)

This Recommendation defines H.248 protocol elements for either transporting complete DPI policy rules embeeded in H.248 messages (“container principle”), or a reference method (“pointer principle”). The particular encoding of DPI policy rules in either case is out of scope of this Recommendation. Thus, the policy rule examples of this Appendix are inherently generic, i.e., using a kind of pseudo policy specification language.

### II.1.3	Generic rule format

In order to use a common description format, - for DPI and non-DPI) policy rules -, the examples here following below generic rule format (see Figure II.1), comprised of

- a rule header (name, identifier, precedence, etc.) and

- a rule body (for conditions, actions).

NOTE - The examples focusing on the condition/action part only.

Figure II.1 – Generic high-level format of a (DPI and non-DPI) policy rule

{Editor’s note (2013-06): avoid overlappings with Appendix III.}

Such a generic format is actually independent of H.248, suited for all existing policy control and policy management protocols.

NOTE - A filter rule is just a dedicated type of a policy rule. In scope of this Recommendation the terms filter rule and policy rule are synonym (however, in general there are also other policy actions than filtering (of packets, flows, traffic, etc)).

Figure II.2 illustrates the principle framework: policy rules are assigned to H.248 contexts, terminations (non-Root or Root) or/and streams. Policy rules may be signalled, but correspondent H.248 property values may be also provisioned. There is thus a signalling and management aspect (or policy control and policy management), see also clause 6.x.y.

Figure II.2 – General principle for (DPI) policy rules

## II.2	Examples of generic DPI and non-DPI Policy Rules

### II.2.1	Non-DPI scenarios

This category refers to “legacy packet inspection” use cases (classified as non-DPI policing scenarios in clause 5.1).

#### II.2.1.1	Example I: "Packet size range policing"

This example illustrates packet size range policing (Table II.2):

Table II.2 – "Non-DPI" Example I – "Packet size range policing"

| Policy Rule ("Packet size range policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L3 DA = (H.248 LD(A))?" AND { C2: "L3 PDU size > (H.248 pacs/M)?" OR C3: "L3 PDU size < (H.248 pacs/mpu)?"} | Then: A1: "discard IP packet" AND A2: "Update Statistic H.248 pacs/dp" |

The policy rule is classified as non-DPI because only layer 3 packet header and payload properties needs to be “inspected”. This example supposes a wildcard ALL L4 port value (thus LD(P) = ALL; see [ITU-T H.248.39], clause 6.11). The received H.248 IP packets must match a particular range concerning a given packet size distribution function, which is bounded by a maximum and minimum size value (using the [ITU-T H.248.53] packet size (pacs) package). Violating packets are not silently discarded, rather counted by the pacs/dp statistic.

It should be reminded that such a policy rule could be already achieved solely on basis of [ITU-T H.248.53].

#### II.2.1.2	Example II: "Flow-level byterate policing"

This example, - also fully based on [ITU-T H.248.53] -, illustrates flow-level byterate policing (Table II.3). The notion of 'flow' relates to the H.248 'flow', i.e. a traffic level above the H.248 stream (here: see clause 6.6.2/[ITU-T H.248.53]).

Table II.3 – "Non-DPI" Example II – "Flow-level byterate policing"

| Policy Rule ("Flow-level byterate policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L3 DA = (H.248 LD(A))?" AND C2: "L4 DP = (H.248 LD(P))?" AND C3: "L3 SA = (H.248 RD(A))?" AND C4: "L4 SP = (H.248 RD(P))?" AND { C5: "GBRA(Rp, Bp) = not OK?" OR C6: "GBRA(Rs, Bs) = not OK?"} | Then: A1: "discard IP packet" AND {/** if peak-rate violation A2,1: "Update Statistic H.248 tmanr/pvp" A2,2: "Update Statistic H.248 tmanr/pvo" } {/** if sustainable-rate violation A3,1: "Update Statistic H.248 tmanr/svp" A3,2: "Update Statistic H.248 tmanr/svo" } |
| NOTE – Package identifier tmanr refers to the [ITU-T H.248.53] traffic policing statistics package. |  |

This example supposes incoming IP traffic on a local transport connection endpoint (given by transport address LD(A,P)) plus the inspection of the remote source transport address (i.e. RS(A,P)), which is supposed to be equal here to the remote destination transport address (i.e. RD(A,P)) due to a symmetry assumption.

A 2-stage policing shall be enforced (peak- and sustainable rate level; see e.g. model in clause I.2.1.3.1/[ITU-T H.248.53]).

The traffic policer conditions (for GBRA algorithm) are derived from tman v2 properties: e.g. the first list item values for flow 1.

Violating packets counted by the correspondent tmanr statistics, on packet and byte level.

#### II.2.1.3	Example III: "Well-known port SIP message policing"

This example is based on clause III.1.1/[ITU-T H.248.43] (Table II.4):

Table II.4 – "Non-DPI" Example III – "Well-known port SIP message policing"

| Policy Rule ("Well-known port SIP message policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If NOT: C1: "L3 DA = (H.248 LD(A))?" AND C2: "IP PCI 'protocol' = 'UDP' OR 'TCP'?" AND C3: "L4 DP = well known port for 'SIP' OR 'SIP-TLS'?" | Then: A1: "discard IP packet" |

This example uses compound policy conditions, i.e. a combination of multiple simple policy conditions (see also [ITU-T H.248.43]). However, this aspect is minor (because every compound condition may be equally transformed in a number of simple conditions).

#### II.2.1.4	Example IV: "General IP 5-tuple policing"

The considered 5-tuple relates here to the 4-tuple of the IP transport connection endpoints plus the 1-tuple of protocol type. [ITU-T H.248.43] supports "general IP 5-tuple policing" by specifying a set of such 5-tuples in parallel. The example here is based on clause III.1.3/[ITU-T H.248.43](Table II.5):

Table II.5 – "Non-DPI" Example IV – "General IP 5-tuple policing"

| Policy Rule ("General IP 5-tuple policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| Incoming IP packet: If NOT: C1: "L3 SA = [101.0.*.0]?" /* wildcard IPv4 OR C2: "L4 DP = 23 OR 14 OR [1442 - 1490] OR 19999 OR 25000?" /* port range OR C3: "IP 'protocol' = 'FTP' OR 'TELNET' OR 'SMTP' OR 'TFPT' OR 'FINGER' OR 'SIP' OR 'SIP-TLS'?" | Then: A1: "discard IP packet" |
| Outcoming IP packet: If: C1: "L3 DA = "[12.8.3.0]/12?" /* wildcard AND C2: "L4 DP = 153 OR 155 OR 157 OR 159 OR 161 OR [732 - 789]?" /* port range AND C3: "IP 'protocol' = '…' OR '…' … OR '…'?" | Then: A1: "discard IP packet" |

The policy rule is classified as "non-DPI" because the policy conditions are based on L3 and L4 PCI information elements only (see also clause 5.2).

#### II.2.1.5	Example V: "Extensive filtering"

[ITU-T H.248.76] provides a tool for defining extensive set of filter rules per context.

### II.2.2	DPI scenarios

Scenarios which contain at least one policy rule condition related to higher protocol layers above the transport layer.

#### II.2.2.1	Example VI: "Media type/media format policing"

There are H.248 profile definitions for IP-to-IP gateways which support media type/media format policing. Such kind of policy rules may be addressed by specifying allowed SDP values (in the H.248 profile section on SDP information elements).

For instance, the profile could not allow a particular list of RTP payload type values (related to a specific RTP profile type (like e.g. 'AVP' [IETF RFC 3551])). The H.248 MG must then inspect the RTP header field "payload/packet type" against correspondent values. Not supported media formats shall lead to the blocking of such RTP packets (see Table II.6). That is an example of an implicit policy rule because not explicitly signalled over the H.248 interfaces, but enforced due to the successful registration of this H.248 profile by the MG.

Table II.6 – "DPI" Example VI – "Media type/media format policing"

| Policy Rule ("Media type/media format policing") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "L4+ protocol = RTP?" AND { C2: "RTP PT = NOT {…allowed values…}?" | Then: A1: "silently discard IP (= RTP) packet" |

#### II.2.2.2	Example VII: "Detect application X of an "application-agnostic" bearer"

Figure II.3 abstracts the general framework. The created context may be e.g. transport protocol type aware, but application agnostic, according the H.248 signalling information (from MGC to MG) for "regular IP user plane processing".

Figure II.3 – General framework for rule category of
"detect application X of an "application-agnostic" bearer"

Such a context could be then overlaid by a policy rule for the inspection of a particular application or a number of applications. Justified e.g. by security concerns, or to check whether users are really using that IP transport connection for the initially requested application.

The principle rule is here (Table II.7) just denoted, but not detailed:

Table II.7 – "DPI" Example VII – "Detect application X of an "application-agnostic" bearer"

| Policy Rule ("Detect application X of an "application-agnostic" bearer") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND { C2: "… transport protocol = …?" AND { C3: "'application' = X?" | Then: A1: "…" + possibly other actions |

It may be noted that the notion of "application" shall be not limited to (IP) application protocol types only, rather every kind of "application" which may be unambiguously detected based on correspondent conditions (Note: such an understanding of “application” is consistent with [ITU-T Y.2770]).

Just some random examples for 'X':

- BitTorrent

- Skype

- Open Game Protocol (OGP)

- media format (e.g. video layer)

The choosen application types related to peer-to-peer (P2P) filesharing (a), proprietary conversational service (b), multi-user distributed P2P (c) and e.g., an HTTP-based streaming service (d).

More detailed policy rules for above scenarios are indicated in subclauses II.2.2.2.1 to II.2.2.2.5, which illustrate that the underlying policy conditions may be multifaceted, e.g.

- "simple" by searching for a particular bit pattern,

- "complex" by executing a regular expression on each packet,

- deterministic vs stochastic (e.g. heuristic) rule type,

- stateless vs statefull.

##### II.2.2.2.1	Example VIII: "Detect application X = 'BitTorrrent'"

This is a HTTP/TCP-based application, which may be roughly detected by looking for specific keywords (e.g. control commands), see Table II.8.

Table II.8 – "DPI" Example VIII: "Detect application X = 'BitTorrrent'"

| Policy Rule ("Detect application X = 'BitTorrrent") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'TCP'?" AND C3: "L4+ protocol type = HTTP" AND C4: "http control = GET request header" AND C5: "http message content contains element uploaded with a value > 0" AND C6: "…" | Then: A1: "…" + possibly other actions |

##### II.2.2.2.2	Example IX: "Detect application X = 'Skype'"

Table II.9 presents an example for a bidirectional, statefull DPI policy rule. The attribute ‘bidirectional’ indicates that the packet flows of each communication direction needs to be inspected.

NOTE – The used rule itself illustrates just the principal proceeding, but is not necessarily sufficient for a real deployment. There are some policy rule proposals for detecting this application. This is just one out of many.

The details of the rule are minor. Relevant here is the fact that this application has an inherent session concept in terms of a state machine. The DPI policy rule here is related to that state model and associated to the session establishment phase.

Table II.9 – "DPI" Example X: "Detect application X = 'Skype'"

| DPI Policy Rule ("Detect Skype session establishment") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| State S1: If: C1,1: "L4 protocol = UDP" AND C1,2: "L4 payload size = 18" AND C1,3: "3rd payload byte = 0x02" | Then: A1,1: "save source IP transport address" A1,2: "save destination IP transport address" A1,3: "save first two bytes of L4 payload" Comment: the FlowID is based here on the 5-tuple for the UDPoIP transport connection and locally stored. |
| State S2: If: C2,1: "FlowID in reverse direction = … (see A1,1 / A1,2)" AND C2,2: "L4 payload size = 11" AND C2,3: "first two bytes of L4 payload = … (see A1,3)" AND C2,4: "3rd payload byte: lower nibble = 7" | Then: none |
| State S3: If: C3,1: "FlowID in initial direction = … (see A1,1 / A1,2)" AND C3,2: "L4 payload size = 23" AND C3,3: "first two bytes of L4 payload = … (see A1,3)" AND C3,4: "3rd payload byte: lower nibble = 3" | Then: none |
| State S4: If: C4,1: "FlowID in reverse direction = … (see A1,1 / A1,2)" AND C4,2: "L4 payload size = 18" AND C4,3: "3rd payload byte = 0x02" | Then: A4,1: "report "successfully Skype detected" |

##### II.2.2.2.3	Example X: "Detect application X = 'Open Game Protocol'"

This is a UDP-based application, which may be simply detected by L4+ Header Inspection (L4+HI), see Table II.10:

Table II.10 – "DPI" Example X: "Detect application X = 'Open Game Protocol'"

| Policy Rule ("Detect application X of an "application-agnostic" bearer") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'UDP'?" AND C3: "L4-SDU 1st 32-bit-Word = '0xFF…F'?" AND C4: "L4-SDU 2nd 32-bit-Word = '"OGP"'?" | Then: A1: "…" + possibly other actions |

##### II.2.2.2.4	Example XI: "Detect application X = 'audio channel in a multi-channel media application'"

Chosen example: detect 2nd audio channel in a multi-channel AMR application and remove channel, see Table II.11.

Table II.11 – "DPI" Example XI: "Detect application X = 'audio channel in a multi-channel media application'"

| Policy Rule ("Detect application X = 'audio channel in a multi-channel media application'") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "…" AND C2: "…" AND C3: "Packet contains the hexadecimal string = "0x2321414d525F4D43312E300a"" AND C4: "next 32-bit word contains the hexadecimal string = "0x00000002"" | Then: A1: "Discard Audio Frame within SDU" A2: "Update Statistic" + possibly other actions |

##### II.2.2.2.5	Example XII: "Detect & measure application X = 'greek Jabber traffic'"

This is a XMPP/TCP-based application, which may be simply detected by looking for XML specific control elements, see Table II.12.

Table II.12 – "DPI" Example XII: "Detect & measure application X = 'greek Jabber traffic'"

| Policy Rule ("Detect & measure application X = 'greek Jabber traffic'") Rx: "…", Id = …, precedence = … |  |
| --- | --- |
| Condition(s) | Action(s) |
| If: C1: "… IP transport connection endpoint values = 4-tuple {…}?" AND C2: "IP PCI 'protocol' = 'TCP'?" AND C3: "L4+ header type = XMPP stream header" AND C4: "XMPP message content contains XML element = "xml:lang='gr'" | Then: A1: "…" + possibly other actions |

For the subsequent examples, the DPI policy rule indications are omitted.

#### II.2.2.3	Example XIII: "TCP attack detection"

There are existing H.248 profiles for IP-to-IP gateways which support TCP-based applications (e.g., with [ITU-T H.248.84] NAT travesal support for TCP traffic). Such a gateway may principally located at the edge of a security domain, and then requested to enforce some policy rules for well-known TCP attack scenarios (see also clause 9/[ITU-T H.248.79]).

This is not necessarily a DPI scenario ("the very majority of TCP threat scenarios are related to L3/L4 header inspection, thus rather “non-DPI"), but the correspondent policy conditions are typically "statefull" ("because the inspection may cover multiple TCP/IP packets and the MG may need to drive through the TCP connection state machine").

#### II.2.2.4	Example XIV: "Remove invalid MIME attachments from Instant Messaging"

Similar another use case: a user may be subscribed for an Instant Messaging (IM) service, realized here as "MSRP-over-TCP" application. The IM service shall allow the transmission of "attachments", which may be encapsulated via MIME ("the low level details are not important here"). The user may be not allowed to issue via his IM client dedicated attachment types. The IM service may be "end-to-end", i.e. without a MSRP relay or switch node. The H.248 MG may be thus requested for MIME attachment type inspection (because e.g. the only "IP hop" instance where all bearer traffic is routed across).

This example relates to a DPI policy rule because besides the TCP header verification, there might be policy conditions related to the

- MSRP header,

- MSRP body 'text',

- MSRP body embedded MIME attachment types,

- etc ("e.g., MIME itself is recursive, which allows to consider MIME-over-MIME …").

#### II.2.2.5	Example XV: "RTCP Block Type Filtering"

That's an example, which may come along with [ITU-T H.248.48] based network applications (see clause 3.2.1/[ITU-T H.248.48]). Appendix I in [b-ITU-T H.248.RTCPPROF] illustrate a number of example use cases with “RTCP filtering”. E.g., provider A wants to block the distribution of own RTCP-based measurement data to other, peering provider partners.

Such DPI policy conditions for measurement data filtering may look like:

- RTCP report: PT = x1, BT = x2, Field = "..."

In other words, firstly packet type value is checked, then block type value, then a particular field within a block, and if all conditions are true, then the action may be to remove the correspondent metric or to reset the value to default, and to recalculate PCI on all affected protocol layers (e.g., UDP checksum recalculation).

#### II.2.2.6	Example XVI: "Application-specific traffic handling"

The notion of 'handling' shall indicate other possible action types like just "packet accept" or "packet discard" actions.

For instance, action types like

- create packet copy and …;

- sent alert (e.g., H.248 Notify to MGC);

- modify IP header DS/ToS/TC values ("[ITU-T H.248.52] like");

- etc.

A principal classification of DPI policy rule actions is described in clause 6.3.3 in [ITU-T Y.2770], which are all relevant as well for this Recommendation.

#### II.2.2.7	Example XVII: "Abnormal packet size detection"

This example is similar to example I (clause 2.2.1), but the difference may be related to another protocol layer (> L4). E.g., multiple application layer PDUs may be assembled in a single IP packet.

The example may be generalized to "abnormal traffic detection", with correspondent policy conditions for specific traffic characteristics. Related information is provided by [b-ITU-T X.Sup#] and clause 6.4.3 in in [ITU-T Y.2770].

#### II.2.2.8	Example XVIII: "Delete old packets of application X"

The notion of 'old' shall be related to the IP header TTL value. The specific example here may illustrate that the condition

"IP PCI "TTL" < y?"

would be tight to a pre-condition of

"'application' = X?".

It may be observed that policy rules itself may be hierarchical, like e.g. conditions embedded in another condition, or an action may refer again to another policy rule, etc.

[End Proposed Correction]

____________________

= ASCII character string: "#!AMR_MC1.0\n" (see [IETF RFC 4867], the magic number for multi-channel AMR)

= 32 bit channel description field(see [IETF RFC 4867])