| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4379 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Update proposal for Appendix IV |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

It is suggested to revise Appendix IV from living list material to regular text.

Discussion and conclusions

H.248.DPI defines capabilities for support of DPI policy rules in non-native H.248 encoding, i.e., H.248.DPI supports a wide range of PSLs. It would be beneficial to illustrate at least one example PSL by an Appendix.

There were already quite intensive evaluations with regards to PSL ‘SNORT’ and their applicability in the context of H.248 gateways. Thus, existing material of Appendix IV serves already as a good baseline for such information in our opinion.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix IV

Discussion of Policy Specification Language

(This appendix does not form an integral part of this Recommendation)

## IV.1	Introduction

This appendix provides complementary information on Policy Specification Languages (PSL), also known as Policy Expression Language (PEL) of Filter Specification Language (FSL). The definition of a PSL is out of scope of this Recommendation. However, the question of PSLs is related here to multiple network interfaces, like a control plane policy control interface between a remote policy decision function and the DPI node function and a management plane policy management interface between network (policy) management functions and the DPI node function. There are thus inherent protocol requirements across multiple, different network interfaces concerning the “transport of policy rule sets” down to the DPI-FE (see also Figure IV.1).

## IV.2	PSL for Policy Control and Policy Management Interfaces

Figure IV.1 provides a summary of a typical network scenario. The policy operations by the control plane and network plane addressing both the same objects of the policy enforcement path in the user plane. Thus, an aligned PSL usage across all relevant interfaces would be crucial for efficient DPI node functions.

Figure IV.1 – Policy Specification Language (PSL) – PSL for Policy Control and Policy Management Interfaces

## IV.3	Survey of possible PSLs (non-exhaustive list)

(DPI) policy rules are enforced on Protocol Data Units (PDU) in general, briefly called packets in this Recommendation. The objects of DPI are therefore parts of or entire PDUs. A PSL must consequently provide specification means for the definition of such objects (“data structure”) and methods executed on these objects (i.e., “operations”, related to policy conditions and policy actions). Table IV.1 provides a list of example standardized protocols (NOTE 1), which may be candidates for DPI-capable PSLs. The example PSLs provide initial support for the specification of such data objects or/and considered operations.

NOTE 1 –	There might be also proprietary protocols around, particularly for management interfaces (like command line interfaces (CLI) or man-machine interfaces (MMI)).

Table IV.1 – Example list of Policy Specification Languages (PSL)
(aka Policy Expression Language (PEL), Filter Specification Language (FSL))

| No | Policy Specification Language |  | Reference |
| --- | --- | --- | --- |
| 1 | SNORT |  | [b-SNORT] |
| 2 | SAML – Security Assertion Markup Language (SAML 2.0) |  | [b-ITU-T X.1141] |
| 3 | XACML – eXtensible Access Control Markup Language (XACML 2.0) |  | [b-ITU-T X.1142] |
| 4 | Open Service Access (OSA) Application Programming Interface (API); Part 13: Policy management Service Capability Feature (SCF) |  | [b-3GPP 29.198-13] |
| 5 | SIEVE – An Email Filtering Language |  | [b-IETF RFC 5228] |
| 6 | BPEL – Business Process Expression Language |  | [b-OASIS BPEL] |
| 7 | BPML – Business Process Modeling Language |  | [b-OMG BPML] |
| 8 | SAML – Security Assertion Markup Language (SAML 2.0) |  | [b-ITU-T X.1141] |
| 9 | XACML – eXtensible Access Control Markup Language (XACML 2.0) |  | [b-ITU-T X.1142] |
| 10 | XCAP – XML Configuration Access Protocol |  | [b-IETF RFC 4825] |
| 11 | PEEM Policy Expression Language (by Open Mobile Alliance) |  | [b-OMA OMA-TS-PEEM_PEL-V1] |
| 12 | PacketTypes |  | [b-PacketTypes] |
| 13 | APF – A Packet Filter |  | [b-APF] |
| 14 | RTAG – Real-Time Asynchronous Grammars |  | [b-RTAG] |
| 15 | TAP/APC – Timed Abstract Protocol & Austin Protocol Compiler |  | [b-TAP] |
| 16 | GAPAL – Generic Application-Level Protocol Analyzer and its Language |  | [b-GAPAL] |
| 17 | Perl (Compatible) Regular Expressions |  |  |
| 18 | POSIX Regular Expressions |  |
| 19 | others |  |

## IV.4	PSLs on different network levels

It may be worth to consider PSLs on different network levels. There might be very high-level PSLs with focus on behavioural policy definitions, using natural languages. On the other side could be low-level PSLs, close to the program code (“e.g., configurations of policy rules at API level”) of packet-path processing components for policy enforcement (e.g., ASIC, FPGA, network processor, general purpose CPU), using a formal specification approach, which is also a prerequisite for the detection of possible rule interaction problems.

Figure IV.2 illustrates just some examples for policy rule specifications.

Figure IV.2 – Policy Specification Languages – Example (DPI) Policy Rules 
(on different network levels)

## IV.5	Recommendations for selected PSLs

Figure IV.3 outlines a possible PSL architecture concept, which would satisfy the requirements of

- single, aligned PSL for policy control and policy management;

- PSL decoupled from a control plane signalling protocol, thus PSL-independent of a dedicated signalling protocol;

NOTE – Concept is already well established in many protocols, principle is equal to the “MIME concept for electronic mail”, i.e. a multipurpose extension capability by the “carrier protocol”. A “multipurpose extension” mechanism would also allow different PSL types.

- ditto for management protocol;

- the specification of a Policy Rule set RDPI (but also Rnon-DPI) would be embedded in a container of the underlying signalling or management protocol;

- alignment of object models and information bases (e.g., between PIBs on PEF-level and policy decision/management entities).

Figure IV.3 – Policy Specification Languages – Possible PSL Architecture Concept

Figure IV.3 shows an abstract rule specification protocol P (as PSL), which is preferably used by network entities in the control and management plane. Any aligned PSL leads to aligned PIBs. Any Policy Rule set RDPI is carried by signalling (X) or management (Y) protocols.

## IV.6	Discussion of Policy Specification Language candidates

Appendix III highlights possible mechanisms for signalling the policy rules to the MG. The principal options are again:

1	Explicit rule signalling

1.1	Explicit rule in native H.248 syntax

1.2	Explicit rule encapsulated in container over H.248

2	Signalling of pointer to rule and rule MG-locally provisioned

3	Signalling of pointer to rule and rule MG-remotely provisioned

From an analysis of the DPI scenarios (Appendix II) it can be determined that the breadth and complexity of the scenarios do not lend themselves to option 1.1. Using multiple properties and multiple Contexts/Termination would be problematic and overly complex.

This leaves options 1.2, 2 and 3 as a possible means to signal policy rules for the illustrated scenarios. In terms of signalling, option 2 and 3 are essentially the same as they are simple pointers to an existing rule set.  These three options are all supported by the instruction rule base package (clause 7).

In order to make this choice one must consider how the rule set is specified (i.e. what is the policy specification language (PSL)?) and what framework it is specified in. In order to determine an appropriate PSL the candidate PSLs must be compared to the scenario list to see if the functionality can be supported.

Preferentially a single PSL should be used by H.248 applications using this Recommendation. This would increase interoperability between implementations. It is expected that H.248 profile specification will define allowed PSLs for the particular network operation.

An example of an open source system is SNORT [b-SNORT], see next clause.

## IV.7	Example PSL “SNORT” – Analysis and comments

SNORT is considered to be a good candidate for the PSL for H.248-based DPI applications. SNORT has many benefits:

- SNORT is open source and is already used in many products.

- There is a large base of existing rules that may be applied to address known vulnerabilities.

- There is a large address space for proprietary/custom rules (~1,000,000 IDs).

- There is an extensive community updating and developing new rules.

The SNORT PSL can be found in section 3 of the SNORT Manual [b-SNORT].

### IV.7.1	Signalling

SNORT is a text based PSL and thus lends itself to being potentially carried in H.248 commands as a string (i.e., using property irb/cfr (see clause 7.1.2)). It also defines a Snort Rule ID (sid) that allows a unique identification of each rule, which lends itself to being used as a pointer in a H.248 command (e.g., information used in property irb/ptr (see clause 7.1.1) or reported via event iro/src (see clause 8.2.1)). Thus SNORT would not prevent the signalling options 1.2, 2 and 3 (indicated above) from being supported.

### IV.7.2	Structure

Figure III.1 outlines a generic high-level format of a policy rule, identifier/name part, condition part and action part.

The SNORT PSL provides several elements that enable clear identification and version handling. The Snort Rule identifier (sid) as mentioned above allows a unique identification of SNORT rules. For version handling the Revision (rev) element can be used. The Reference (reference) element can provide further information regarding the definition of a set of rules. The Priority element (priority) may be assigned to the rule in order to prioritise one rule over another.

SNORT rules consist of a rule header and rule options. As indicated by the manual:

"Snort rules are divided into two logical sections, the rule header and the rule options. The rule header contains the rule's action, protocol, source and destination IP addresses and netmasks, and the source and destination ports information. The rule option section contains alert messages and information on which parts of the packet should be inspected to determine if the rule action should be taken."

So whilst the rule structure itself does not follow Figure III.1 strictly it does allow conditions and actions to be specified.

### IV.7.3	Rule conditions: comment to address information

SNORT currently supports the TCP, UDP, ICMP and IP protocols. The IP address, port and source and destination (direction indicator) information is carried in the SNORT rule header. H.248 supports the sending of this information via Termination/Stream configuration and the Local and Remote Descriptors. This implies certain alignment of such address information between enforced SNORT rules and H.248 non-root terminations.

### IV.7.4	Rule actions: general SNORT concept

SNORT defines a rich set of actions (see section 3.2.1 / SNORT Manual [b-SNORT]), including:

- alert - generate an alert using the selected alert method, and then log the packet

- log - log the packet

- pass - ignore the packet

- activate - alert and then turn on another dynamic rule

- dynamic - remain idle until activated by an activate rule , then act as a log rule

- drop - block and log the packet

- reject - block the packet, log it, and then send a TCP reset if the protocol is TCP or an ICMP port  unreachable message if the protocol is UDP.

- sdrop - block the packet but do not log it.

SNORT allows the logging and alerting of events. The served user instance for such information might be basically a policy server/controller/manager in the network control or/and management plane (see also clause ###). These SNORT events would be part of performance management and alarm/security management in case of management plane usage. In case of of control plane, then there are relations to H.248 statistics and H.248 event reporting (see clause 8). Any detailed mapping between SNORT and H.248 is out of scope of this Recommendation (e.g., part of a H.248 profile specification).

### IV.7.5	Rule conditions: principal inspection process

SNORT defines many detection rule options. These are effectively classified into three groups:

- Content Detection Rule Options

- Non-Payload Detection Rule Option

- Post-Detection Rule Options

The elements contained in these groups allow complex filtering behaviour to be specified.

Appendix II outlines a number of DPI scenarios. Table IV.2 summarizes a brief discussion of SNORT support:

Table IV.2 – Examples (from Appendix) versus principle SNORT support

| No. | Example name | SNORT |
| --- | --- | --- |
| VI |  | Media type/media format policing |  | The "content" element with associated content modifier allow would allow a rule to be specified to look at the bits associated with the payload type field and to see whether they matched an allowed value/s. |
| VII |  | Detect application X of an "application-agnostic" bearer |  | The example indicates several different methods for determining applications. In the generic example it can be seen that SNORT allows simple bit pattern searching. SNORT also allows complex regular expression matching utilising the "pcre" element (section 3.5.23 in [b-SNORT]). Heuristic analysis is supported via multiple rules and dynamic rule specification. Note: there is an active community defining rules for well known and emerging attack signatures. |
| VIII |  | Detect application X = 'BITTORRRENT' |  | SNORT has several elements that deal with searching and matching in HTTP bodies. I.e. "http_method" (section 3.5.13 / [b-SNORT]) and "http_client_body" (section 3.5.8 / [b-SNORT]). |
| IX |  | Detect application X = 'SKYPE' |  | SNORT allows a stateful based rule definition approach. It is possible that one rule triggers another. It is also possible to extract information via one rule and save it for use in another rule (i.e. through the use of the "byte_extract" (section 3.5.29 / [b-SNORT])). |
| X |  | Detect application X = 'OPEN GAME PROTOCOL' |  | SNORT may achieve this through simple pattern matching. |
| XI |  | Detect application X = 'audio channel in a multi-channel media application' |  | Whilst SNORT may allow the detection of the condition it is currently unclear whether it can support the removal of an audio frame within an SDU. SNORT does have the "replace" element that can be utilised for simple replacement of data (where length can be maintained). However removal of payload data has implications for checksums etc. Further investigation is needed. |
| XII |  | Detect & measure application X = 'greek Jabber traffic' |  | SNORT may achieve this through simple pattern matching. |
| XIII |  | TCP attack detection |  | SNORT allows the detection of TCP attacks. It has several elements dedicated to TCP checks, i.e. the elements "flags", "flow", "flowbits", "seq", "ack" and "window" from section 3.6 / [b-SNORT]). |
| XIV |  | Remove invalid MIME attachments from Instant Messaging |  | As per example XI. SNORT support for selective removal of payload information is for further study. |
| XV |  | RTCP Block Type Filtering |  | As per example XI. SNORT support for selective removal of payload information is for further study. |
| XVI |  | Application-specific traffic handling |  | As discussed in section IV.7.4 above SNORT has several defined actions but also allows user defined actions. |
| XVII |  | Abnormal packet size detection |  | The SNORT element "dsize" (section 3.6.7 / [b-SNORT]) allows the testing of packet payload size. |
| XVIII |  | Delete old packets of application X |  | The SNORT element "ttl" (section 3.6.2 / [b-SNORT]) allows the testing of the time to live for a packet. |

### IV.7.36	Disclaimer

This Appendix only illustrates how PSL ‘SNORT’ might be used in combination with this Recommendation, but does not specify concrete usage.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# Bibliography

[b-3GPP 29.198-13]		3GPP Open Service Access (OSA) Application Programming Interface (API), Part 13: Policy management Service Capability Feature (SCF).

[b-APF]		H. D. Lambright & S. K. Debray. APF: A modular language for fast packet classification. Dept of Computer Science, University of Arizona, Tucson, August 30, 1996. http://www.cs.arizona.edu/~debray/Publications /filter.html .

[b-ETSI TS 123 203]	ETSI TS 123 203 (2011), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Policy and charging control architecture (3GPP TS 23.203 version 10.4.0 Release 10).

[b-ETSI TS 124 229]	ETSI TS 124 229 (2009), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE;IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3 (3GPP TS 24.229 version 9.4.0 Release 9).

[b-ETSI ES 282 003]	ETSI ES 282 003 (2011), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control Sub-System (RACS): Functional Architecture.

[b-GAPAL]		N. Borisov, D. J. Brumley, and H. J. Wang, Generic Application-Level Protocol Analyzer and its Language. Proceedings of the Network and Distributed System Security Symposium, NDSS 2007, San Diego, USA, 2007.

[b-IETF opsec]	IETF draft-ietf-opsec-filter-caps (expired, 2008), Filtering and Rate Limiting Capabilities for IP Network Infrastructure. http://tools.ietf.org/html/draft-ietf-opsec-filter-caps-09.

[b-IETF RFC 1950]IETF RFC 1950 (1996), ZLIB Compressed Data Format Specification version 3.3.

[b-IETF RFC 2474]	IETF RFC 2474 (1998), Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Header.

[b-IETF RFC 2748]	IETF RFC 2748 (2000), The COPS (Common Open Policy Service) Protocol.

[b-IETF RFC 2784]		IETF RFC 2784 (2000), Generic Routing Encapsulation (GRE).

[b-IETF RFC 3198]	IETF RFC 3198 (2001), Terminology for Policy-Based Management.

[b-IETF RFC 3320]	IETF RFC 3320 (2003), Signaling Compression (SigComp).

[b-IETF RFC 3871]	IETF RFC 3871 (2004), Operational Security Requirements for Large Internet Service Provider (ISP) IP Network Infrastructure.

[b-IETF RFC 4011]		IETF RFC 4011 (2005), Policy Based Management MIB.

[b-IETF RFC 4268]	IETF RFC 4268 (2005), Entity State MIB.

[b-IETF RFC 4778]		IETF RFC 4778 (2007), Current Operational Security Practices in Internet Service Provider Environments.

[b-IETF RFC 4825]		IETF RFC 4825 (2007), The Extensible Markup Language (XML) Configuration Access Protocol (XCAP).

[b-IETF RFC 4867]		IETF RFC 4867 (2007), RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs.

[b-IETF RFC 5102]	IETF RFC 5102 (2008), Information Model for IP Flow Information Export.

[b-IETF RFC 5103] 	IETF RFC 5103 (2008), Bidirectional Flow Export Using IP Flow Information Export (IPFIX).

[b-IETF RFC 5189]		IETF RFC 5189 (2008), Middlebox Communication (MIDCOM) Protocol Semantics.

[b-IETF RFC 5228]		IETF RFC 5228 (2008), Sieve: An Email Filtering Language.

[b-IETF RFC 5424]	IETF RFC 5424 (2009), The Syslog Protocol.

[b-IETF RFC 5426]	IETF RFC 5426 (2009), Transmission of Syslog Messages over UDP.

[b-IETF RFC 5476]	IETF RFC 5476 (2009), Packet Sampling (PSAMP) Protocol Specifications.

[b-IETF RFC 5840]		IETF RFC 5840, Wrapped Encapsulating Security Payload (ESP) for Traffic Visibility.

[b-IETF RFC 5879]		IETF RFC 5879, Heuristics for Detecting ESP-NULL Packets.

[b-IEEE GLOBECOM] IEEE GLOBECOM 2008, “Online Identification of Applications Using Statistical Behavior Analysis”.

[b-ITU-T X.734]	Recommendation ITU-T X.734 (1992), Information technology - Open Systems Interconnection - Systems Management: Event report management function.

[b-ITU-T X.1141]		Recommendation ITU-T X.1141 (2008), Security Assertion Markup Language (SAML).

[b-ITU-T X.1142] 	Recommendation	ITU-T X.1142 (2008), Extensible Access Control Markup Language (XACML).

[b-ITU-T X.1243] 	Recommendation ITU-T X.1243 (2010), Interactive gateway system for countering spam.

[b-ITU-T X.abnot]		Recommendation Draft ITU-T X.abnot (2011), Abnormal traffic detection and control guideline for telecommunication network.

[b-ITU-T X.sips]		Recommendation ITU-T X.sips (2011), A framework for countering cyber attacks in SIP-based services.

[b-OASIS BPEL]		OASIS Standard BPEL (2007), Web Services Business Process Execution Language Version 2.0. http://www.oasis-open.org/specs/index.php #wsbpelv2.0.

[b-OMG BPML]		Object Management Group (OMG) BPML (2002), - Business Process Modeling Language Version 1.0. http://www.bpmi.org/bpml-spec.htm.

[b-OMA OMA-TS-PEEM_PEL-V1]	Open Mobile Alliance OMA-TS-PEEM_PEL-V1 (2007), PEEM Policy Expression Language Technical Specification”, Draft Version 1.0.

[b-PacketTypes]		P. J. McCann, S. Chandra, PacketTypes: Abstract Specification of Network Protocol Messages. In SIGCOMM ’00: Proceedings of the conference on Applications, Technologies, Architectures, and Protocols for Computer Communication, pages 321–333, New York, NY, USA, 2000. ACM Press.

[b-RTAG]		D. P. Anderson & L. H. Landweber, A grammar-based methodology for protocol specification and implementation. In SIGCOMM ’85: Proceedings of the ninth symposium on Data communications, pages 63–70, New York, NY, USA, 1985. ACM Press.

[b-Subhabrata]	Subhabrata S., Spatscheck O. and Wang D. (2004), Accurate, Scalable In­Network Identification of P2P Traffic Using Application Signatures, The 13th International World Wide Web Conference (WWW2004, http://www.www2004.org/), May 17 - 22, 2004.

[b-TAP]		T. McGuire, The Austin Protocol Compiler.  http://apcompiler.sourceforge.net/.

[b-SNORT]		SNORT Manual: http://www.snort.org/assets/166/snort_manual.pdf

[End Proposed Correction]

____________________

e.g., via high-level push mode or pull mode operations between policy decision entities and the policy enforcement processing path.