| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4381 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.DPI: Proposal for new Appendix on DPI rule emulation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add a new Appendix on DPI policy rule emulation.

Discussion and conclusions

This contribution is based on the discussion of following contributions at previous meetings:

- Meeting 2012-02 – AVD-4168 “H.248.DPI: Alignment with other policy control protocols” and

- Meeting 2012-09 – AVD-4245 “H.248.DPI: DPI Policy Rule Emulation Package”.

Purpose of these contributions was to show, how the capabilities of H.248.DPI could be applied on a policy control interface with DPI support, which is not H.248 based. Thus, a correspondent H.248 profile would provide a “DPI policy rule emulation service”, in case of H.248 usage for such policy control interfaces.

The purpose of the new proposed Appendix to H.248.DPI is to provide some guideline for H.248 profile specifications concerning such a DPI policy rule emulation service.

Thus, we are proposing to get rid of the idea in adding normative text to the main body of H.248.DPI, such as a new DPI policy rule emulation package (as discussed by AVD-4245). Such an H.248 package would be e.g. policy control technology specific.

The alternative approach could be the description of guidelines, how existing H.248.x capabilities plus the new proposed H.248 DPI packages (see AVD-4375) could be used to emulate the aimed DPI policy control behaviour.

We are suggesting to add a correspondent Appendix to H.248.DPI.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 17 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix V
Emulation of DPI policy rule control interfaces

(This appendix does not form an integral part of this Recommendation)

## V.1	Purpose

## V.2	Guidelines for H.248 profile specifications at the example of native SNORT interfaces

### V.2.1	###

## V.3	Guidelines for H.248 profile specifications at the example of 3GPP Diameter interfaces

### V.3.1	###

[End Proposed Correction]

____________________