| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4383 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal due to RFC 6776 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 2 and 8.4, due to consideration of RFC 6776 “Measurement Identity and Information Reporting Using a Source Description (SDES) Item and an RTCP Extended Report (XR) Block A” related requirements.

Discussion and conclusions

There’s an editor’s note with regards to RFC 6776. This RFC 6776 is relevant for RTP-based performance monitoring and was already discussed (as draft-ietf-xrblock-rtcp-xr-meas-identity) in former work item H.248.48.

RFC 6776 defines two information elements, which may be beneficial for correlating and post-processing of measurement data. The two information elements have to be provided by measurement points and would be reported via the IP bearer interface.

It is therefore suggested to capture RFC 6776 capabilities under clause 8.4.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

...

[IETF RFC 6776]	IETF RFC 6776 (10/2012), Measurement Identity and Information Reporting Using a Source Description (SDES) Item and an RTCP Extended Report (XR) Block.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 8.4	H.248 MG as reporting point at IP bearer interface (RPRTCP)

This interface is used for the exchange of measurement data and information for the production of measurements via RTCP reports.

### 8.4.1	Measurement data

R-8.4.1/1:	The MG is required to send RTCP basic reports (SR and/or RR)..

R-8.4.1/2:	The MG is required to receive RTCP basic reports (SR and/or RR).

R-8.4.1/3:	The MG is recommended to send RTCP extension reports (XR; NOTE 1).

R-8.4.1/4:	The MG is recommended to receive RTCP extension reports (XR; NOTE 1).

NOTE 1 – This requirement could be further detailed to the block type (BT) level or below the BT level down to individual metrics.

### 8.4.2	Measurement configuration

R-8.4.2/1:	The MG is recommended to send measurement identity related SDES item (such as the Application-Specific Identifier (APSI) according clause 3.1/[IETF RFC 6776]).

R-8.4.2/2:	The MG is recommended to receive measurement identity related SDES item (such as the Application-Specific Identifier (APSI) according clause 3.1/[IETF RFC 6776]).

R-8.4.2/3:	The MG is recommended to send measurement information according XR block type 14 (clause 4/[IETF RFC 6776]).

R-8.4.2/4:	The MG is recommended to send measurement information according XR block type 14 (clause 4/[IETF RFC 6776]).

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

| APSI | (RTCP) Application-Specific Identifier (SDES item) |
| --- | --- |
| SDES | (RTCP) Source Description |

[End Proposed Correction]

____________________