| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4384 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal clause 3.2.4 (loopback path options) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 3.2.4 concerning loopback path options

Discussion and conclusions

The editor’s note was added in the 2012-09 meeting (AVD-4262). There are two aspects related to H.248.RTCPPROF:

- Principal interaction between loopback path usage and dedicated performance metrics.

E.g., Traffic volume related performance measurements may differ, dependent on the fact whether loopback traffic is routed via a MP or not.

- Principal difference between IETF Media Loopback and H.248 Media Loopback function.

The LPH.248 relates to a pure redirect function, whereas the LPIETF generates also packet copies.

However, it is not planned to evaluate more detailed “loopback” use cases in H.248.RTCPPROF. Thus, a note in clause 3.2.4 might be sufficient.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal:

[Begin Proposed Correction]

3.2.4	loopback point (LP) (adapted from [ITU-T H.248.85]): Location of a loopback function. In the context of this Recommendation, a loopback function comprises a copy and forward operation at the level of protocol data information, with primary scope on "IETF Media Loopback" (according clause 3.2.3/[ITU-T H.248.85]).

NOTE 5 – A loopback function is different in comparison to a redirect function, which does not copy any protocol data.

NOTE 6 – The loopback function is called "loopback mirror" in [b-IETF RFC 6849].

NOTE 7 – The "H.248 Media Loopback" (see clause 3.2.2/[ITU-T H.248.85]) does relate to a redirect function only in contrast to the "IETF Media Loopback". Might be suject of consideration when used in performance monitoring solutions.

[End Proposed Correction]

____________________