| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4386 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal clause 8.8 (RTP topologies) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 8.8 concerning RTP topology related requirements.

Discussion and conclusions

It is suggested to indicate the basic RTP topologies as explicit requirements.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

...

[IETF RFC 6776]	IETF RFC 6776 (10/2012), Measurement Identity and Information Reporting Using a Source Description (SDES) Item and an RTCP Extended Report (XR) Block.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

## 8.8	Support of specific RTP topologies

The relation between RTCP services and RTP topologies in H.248 entities is described in [b-ITU-T H.248.RTPTOPO]. Performance monitoring based on RTCP XR belongs to supplementary RTCP services which are basically an overlay function and hence independent of RTP topologies, whereas performance monitoring using RTCP basic reports (such as SR, RR) would depend on RTP topologies.

R-8.8/1:	The MG is recommended to support RTP topology “RTP endsystem” (e.g. in use case 1 (see clause I.2)).

R-8.8/2:	The MG is recommended to support RTP topology “Back-to-back RTP endsystem” (e.g. in use case 6 (see clause I.7) with implicit filtering of RTCP measurement reports).

R-8.8/3:	The MG is recommended to support RTP topology “RTP transport translator” (e.g. in use case 2 (see clause I.3) with support of RTP-aware transport level metrics according RTCP basic reports).

R-8.8/4:	The MG is recommended to support RTP topology “RTP media translator” (e.g. in case audio transcoding within an end-to-end RTP session).

[End Proposed Correction]

____________________