| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4387 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal clause 9 (profile guidelines) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 9 concerning profile guidelines.

Discussion and conclusions

Profile specification guidelines are subject of clause 9. However, there’ll be not any single set of guidelines, rather use case dependent recommendations. This is different to the profile specification guidelines by H.248.81, which satisfies only a fixed set of requirements.

The principal proceeding for the derivation of profile specification guidelines in H.248.RTCPPROF is described in clause 1.1 (Fig. 1). Clause 9 could provide example profile specification guidelines at the instance of a few use cases.

We’d like to suggest to consider two use cases, - performance monitoring by a TGW and by a BGW at a peering interface -, and then the illustration of profile specification guidelines.

Proposal

A new Appendix V is proposed to describe the example capability sets (related to performance monitoring) for the two scenarios.

Clause 9 is then updated by referring to the two examples.

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 9	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case (see examples in Appendix I). The guidelines in this clause are therefore basically conditional. Two exemplary use cases are considered (as described in Appendix V), termed as capability set CSA and CSB.

## 9.1	Profile Identification

Subject to profile specification.

## 9.2	Summary

Subject to profile specification.

## 9.3	Gateway Control Protocol Version

Subject to profile specification.

Examples:

IF CSA THEN “no specific H.248.1 version required”.

IF CSB THEN “H.248.1 V3 required due to support of stream-level statistics”.

## 9.4	Connection Model

| Maximum number of contexts: | N/A |
| --- | --- |
| Maximum number of terminations per context: | Examples: IF CSA THEN “2” (TGW). IF CSB THEN “2” (BGW). |
| Allowed termination type combinations in a context: | Context (one or more IP terminations) Examples: IF CSA THEN “(IP, phy) connection model”. IF CSB THEN “(IP, IP) connection model”. |

NOTE – The scope of this Recommendation is only applicable to H.248 gateways in IP networks.

## 9.5	Context Attributes

Subject to profile specification.

## 9.6	Terminations

Subject to profile specification.

Examples:

IF CSA THEN “RTP termination” (for the ephemeral termination).

IF CSB THEN “IP terminations”.

## 9.7	Descriptors

### 9.7.1	TerminationState Descriptor

Subject to profile specification.

### 9.7.2	Stream Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

Examples:

IF CSA THEN “single, RTP-based stream only”.

IF CSB THEN “one or multiple streams”.

### 9.7.2	Events Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.3	EventBuffer Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.4	Signals Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.5	DigitMap Descriptor

Subject to profile specification.

### 9.7.6	Statistics Descriptor

NOTE – H.248-based performance monitoring functions at RTP stream/terminations may demand for support of statistics in case of a RPH.248 requirement.

Examples:

Are statistics supported on terminations, streams or both?

| Statistics supported on: | Examples: IF CSA THEN “termination-level statistics only”. IF CSB THEN “both”. |
| --- | --- |

Are statistics to be reported?

| Statistics reported on Subtract: | IF CSA OR CSB THEN “YES” |  |
| --- | --- | --- |
| If yes | StatisticIDs reported: | IF CSA OR CSB THEN “ALL” (according package usage details by clause 5.14.3). |

IF CSA THEN “RTP termination” (for the ephemeral termination).

IF CSB THEN “IP terminations”.

### 9.7.7	ObservedEvents Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.8	Topology Descriptor

Subject to profile specification.

NOTE – H.248-based performance monitoring functions at RTP stream/terminations does not have implications on this Descriptor.

### 9.7.9	Error Descriptor

Which H.248.8 and package defined error codes are supported?

Error Codes sent by the MGC:

| Supported H.248.8 Error Codes: | Subject to profile specification |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification Examples: IF CSA OR CSB THEN “(additional) support of error codes 485, 486 and 487” (see clause 6.5/[ITU-T H.248.48]). |

Error Codes sent by the MG:

| Supported H.248.8 Error Codes: | All H.248.8 error codes or a list of the individual error code numbers |
| --- | --- |
| Supported Error Codes defined in packages: | Subject to profile specification Examples: IF CSA OR CSB THEN “(additional) support of error codes 485, 486 and 487” (see clause 6.5/[ITU-T H.248.48]). |

## 9.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 9.8.1	Add

Subject to profile specification.

NOTE – H.248-based performance monitoring functions: usage of this H.248 command in principal operations on statistics is illustrated in Appendix IV/[ITU-T H.248.1].

### 9.8.2	Modify

Subject to profile specification.

NOTE – H.248-based performance monitoring functions: usage of this H.248 command in principal operations on statistics is illustrated in Appendix IV/[ITU-T H.248.1].

### 9.8.3	Subtract

Subject to profile specification.

NOTE – H.248-based performance monitoring functions: usage of this H.248 command in principal operations on statistics is illustrated in Appendix IV/[ITU-T H.248.1].

### 9.8.4	Move

Subject to profile specification.

### 9.8.5	AuditValue

Subject to profile specification.

NOTE – H.248-based performance monitoring functions: usage of this H.248 command in reading operations on statistics is illustrated in Appendix IV.6/[ITU-T H.248.1].

### 9.8.6	AuditCapabilities

Subject to profile specification.

### 9.8.7	Notify

Subject to profile specification.

NOTE – H.248-based performance monitoring functions: usage of this H.248 command would be required for “conditional reporting” support, see [ITU-T H.248.47].

### 9.8.8	ServiceChange

Subject to profile specification.

### 9.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 9.9	Generic command syntax and encoding

Subject to profile specification.

NOTE – There is no impact on encoding.

## 9.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 9.11	Messages

Subject to profile specification.

NOTE – There is no impact on the number of transaction per message.

## 9.12	Transport

Subject to profile specification.

NOTE – No requirement on any particular H.248 transport modes. However, given the nature of these services, profile specifications could consider the use of a transport that provides large message size handling ("dependent on the amount of H.248 statistics").

## 9.13	Security

Subject to profile specification.

NOTE – There is no impact on H.248 transport security. However, encryption of H.248 messages may be considered due to the sensitivity of measurement data.

## 9.14	Packages

Although this Recommendation does not mandate any H.248 package for profile specifications, there may be some useful packages for supporting H.248-based performance monitoring of RTP networks. The following is a non-exhaustive list of available H.248 capabilities:

- H.248 controlled MPs (inclusive RP capability):

  - [ITU-T H.248.1], Annex E.12.4 for some basic RTP performance metrics according [IETF RFC 3550];

  - [ITU-T H.248.30] for support of [IETF RFC 3611] in specific use cases (see clause 6.1 and Appendix IV.2);

  - [ITU-T H.248.48] as generic tool for all kind of RTCP XR defined performance metrics;

  - [ITU-T H.248.58] for support of RTP application data related metrics (see also clause III.3.1);

  - [ITU-T H.248.61] for support of RTP-agnostic transport level metrics (see also clause III.2.1);

- H.248 specific remote MPs (inclusive RP capability):

  - [ITU-T H.248.71], clause 7 for collection and reporting for remote measurements of some, basic RTP performance metrics according [IETF RFC 3550];

- H.248 advanced RP:

  - [ITU-T H.248.47] as generic tool for reporting control

- H.248 controlled LP:

  - [ITU-T H.248.85]

NOTE – The above Recommendations contain many different properties, signals, events however, as they are optional to simplify the definition of this profile specification guideline, these impacts are not listed.

### 9.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| <name> | <xxxx, (0x00xx)> | <1, 2, 3, …> | <Describe> |
| Examples: IF CSA THEN - “RTP application data package” [ITU-T H.248.58] - “RTCP XR block reporting package” [ITU-T H.248.48] | rtpad, (0x00cb) xrbr, (0x00af) | v1 v1 | RTP RTP |
| Examples: IF CSB THEN - “RTP package” [ITU-T H.248.1] - “Statistic conditional report-ing pack.” [ITU-T H.248.47] - “RTCP XR block reporting package” [ITU-T H.248.48] - “Received RTCP package” [ITU-T H.248.71] | rtp, (0x000c) scr, (0x00ae) xrbr, (0x00af) recrtcp, (0x00f5) | v2 v2 v1 v1 | IP IP IP IP |

### 9.14.2	Optional Package

Subject to profile specification.

### 9.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 9.14.3.1	RTP Package

(if [ITU-T H.248.1], Annex E.12 approach)

Examples:

IF CSB THEN “usage details for the five statistics”:

| … |  |  |  |  |
| --- | --- | --- | --- | --- |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: | Termination/Stream Types Supported: |
| ps, (0x0004) | M | ADD, MOD, SUBTRACT, NOTIFY | ALL | IP |
| pr, (0x0005) | M |  | ALL | IP |
| pl, (0x0006) | M |  | ALL | IP |
| jit, (0x0007) | Not required (NOTE 1) |  | - | - |
| delay, (0x0008) | M |  | ALL | IP |
| Error Codes | … |  |  |  |
| NOTE 1 – Assumption: support of RTCP XR packet delay variation metrics according [b-IETF RFC 6798], as part of the xrbr package (see clause 9.14.3.7). |  |  |  |  |

#### 9.14.3.2	Received RTCP Package

(if [ITU-T H.248.71] approach)

Examples:

IF CSB THEN “usage details for the five statistics”:

| … |  |  |  |  |
| --- | --- | --- | --- | --- |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: | Termination/Stream Types Supported: |
| rps, (0x0001) | M (NOTE 1) | ADD, MOD, SUBTRACT, NOTIFY | ALL | IP |
| ros, (0x0002) |  |  | ALL | IP |
| rpl, (0x0003) |  |  | ALL | IP |
| rcpl, (0x0004) |  |  | ALL | IP |
| rjit, (0x0005) |  |  | ALL | IP |
| Error Codes | … |  |  |  |
| NOTE 1 – Assumption: there are two types of UEs in IP network domain Xd (see Figure I.6): a) UEs with support of RTCP basic report related performance metrics only, and b) UEs with additional support of RTCP extension report defined performance metrics. The statistics of the recrtcp package are only used for terminal of type (a). |  |  |  |  |

#### 9.14.3.3	RTCP XR Base Package

(if [ITU-T H.248.30] approach)

#### 9.14.3.4	RTCP XR Burst Metrics Package

(if [ITU-T H.248.30] approach)

#### 9.14.3.5	Received RTCP XR Package

(if [ITU-T H.248.30] approach)

#### 9.14.3.6	Received RTCP XR Burst Metrics Package

(if [ITU-T H.248.30] approach)

#### 9.14.3.7	RTCP XR Block Reporting Package

(if [ITU-T H.248.48] approach)

Examples:

IF CSB THEN “usage detail for the five statistics”:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| scpo, (0x0001) | M (NOTE 1) | ADD, MOD | ALL | “N” | IP |
| sfpo, (0x0002) | M (NOTE 2) | ADD, MOD | ALL | “N” | IP |
| srb, (0x0003) | M (NOTE 3) | ADD, MOD | ALL | “N” | IP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| xrpkt, (0x0001) | M | a) ADD & MOD (NOTE 4), b) SUBTRACT, NOTIFY c) Not used in: MOVE, AUDITVALUE& AUDITCAP | ALL |  | IP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #485 | M |  |  |  |  |
| #486 | M |  |  |  |  |
| #487 | M |  |  |  |  |
| NOTE 1 – Required for reporting control of remote measurements by UEA. NOTE 2 – Required for FP configuration at both terminations. NOTE 3 – Required for reporting control and configuration of local measurements. NOTE 4 – Required if all operations on statistics (as described by Appendix IV/[ITU-T H.248.1]) would be supported (e.g., inclusive enable/disable and initialization/reset). |  |  |  |  |  |

#### 9.14.3.8	RTCP Source Description Package

(possibly, if [ITU-T H.248.48] approach)

Examples:

IF CSB THEN “usage detail according following table …”:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| lssrc, (0x0001) | O (NOTE 1) | same as clause 9.14.3.6 | ALL |  | IP |
| rssrc, (0x0002) | M | same as clause 9.14.3.6 | ALL |  | IP |
| lcname, (0x0003) | M | same as clause 9.14.3.6 | ALL |  | IP |
| rcname, (0x0004) | M | same as clause 9.14.3.6 | ALL |  | IP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Local SSRC usage is conditional on the applied RTP topology (see Table V.2). |  |  |  |  |  |

#### 9.14.3.9	further performance monitoring related packages

Subject to profile specification.

## 9.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

Examples:

IF CSA OR CSB THEN “support of RTCP XR”, THEN “support of XR related SDP”:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=rtcp-xr:" | not supported (in this example) | Supported values for attribute parameter <xr-format> are dependent on the concrete supported RTCP XR defined performance metrics. (NOTE 1) |
| NOTE 1 – Usage of this SDP attribute is defined in clause 6.6/[ITU-T H.248.48]. |  |  |

## 9.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

## 9.17	Procedures

### 9.17.1	Measurement-related procedures

#### 9.17.1.1	Enabling of MPs

Subject to profile specification.

Examples:

IF CSA OR CSB

THEN “measurements should always start immediately with creation of the termination or stream end point”,

THEN “enabling of MPs” should conceptually follow the procedures of clauses IV.2.1, IV.2.2 or IV.2.3 according [ITU-T H.248.1].

#### 9.17.1.2	Disabling of MPs

Subject to profile specification.

Examples:

IF CSA OR CSB

THEN “the deactivation of measurements should be supported”,

THEN “deactivation of MPs” should conceptually follow the procedures of clauses IV.3.3 or IV.3.5 according [ITU-T H.248.1].

### 9.17.2	Reporting-related procedures

#### 9.17.2.1	Reporting point at gateway control interface (H.248 statistics)

##### 9.17.2.1.1	Reporting/reading of statistics (RPH.248)

Subject to profile specification.

##### 9.17.2.1.2	Resetting of statistics (RPH.248)

Subject to profile specification.

NOTE – Conceptual procedure is described in clause IV.4/[ITU-T H.248.1].

##### 9.17.2.1.3	Configuration of conditional reporting

Subject to profile specification.

#### 9.17.2.2	Reporting point at IP bearer interface (RTCP reports)

Subject to profile specification.

### 9.17.3	Filtering-related procedures

Subject to profile specification.

### 9.17.4	Loopback-related procedures

Subject to profile specification.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix V

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## V.1	Overview

Profile content is mainly use case dependent as outlined in clause 1.1. In order to demonstrate profile specification guidelines in clause 9, this Recommendation considers two exemplary use cases:

- Capability set ‘A’ (CSA): a trunking gateway performance monitoring scenario according use case 1 (clause I.2); and

- Capability set ‘B’ (CSB): a border gateway performance monitoring scenario according use case 6 (clause I.7).

Tables V.I and V.2 illustrate exemplary set of requirements (based on clause 8) for profiling the two gateway types:

Table V.1 – TGW example for capability set ‘A’ (CSA)

| Functional area: | Capabilities: |  |
| --- | --- | --- |
| - H.248 MG as general measurement point (MP): | R-8.1/1, R-8.1/3, R-8.1/7 |  |
| - H.248 MG as location-dependent measurement point (MP): |  | R-8.2.1/1, R-8.2.2/1, R-8.2.2/2 |  |  |
| - H.248 MG as collection point (CP): |  | R-8.3/1 |  |  |
| - H.248 MG as reporting point at IP bearer interface (RPRTCP): |  | R-8.4.1/4 |  |  |
| - H.248 MG as reporting point at H.248 interface (RPH.248): |  | Data: R-8.5.1/3, R-8.5.1/4 Control: R-8.5.2/1 |  |  |
| - H.248 MG as filtering point (for RTCP traffic) (FP): |  | - (NOTE 1) |  |  |
| - H.248 MG as loopback point (for RTP/RTCP traffic) (LP): |  | - |  |  |
| - Support of specific RTP topologies: |  | R-8.8/1 (NOTE 2) |  |  |
| - Others: |  | - |  |  |
| NOTE 1 - TGW terminates any RTCP traffic. NOTE 2 - TGW represents inherently (in any case) the “RTP endsystem” topology. |  |

The performance monitoring scope of the BGW example (Table V.2) is limited to local measurements for network GoS only, - i.e., typical RTP-agnostic transport level metrics as well as round-trip delay (as RTP-aware transport level metric) -, plus the reporting of all performance measurements by remote UEA.

Table V.2 – BGW example for capability set ‘B’ (CSB)

| Functional area: | Capabilities: |
| --- | --- |
| - H.248 MG as general measurement point (MP): | R-8.1/1, R-8.1/2, R-8.1/4, R-8.1/7 |
| - H.248 MG as location-dependent measurement point (MP): | R-8.2.1/1, R-8.2.2/1, R-8.2.2/2 |
| - H.248 MG as collection point (CP): | R-8.3/1, R-8.3/2 |
| - H.248 MG as reporting point at IP bearer interface (RPRTCP): | R-8.4.1/1, R-8.4.1/2, R-8.4.1/3, R-8.4.1/4 |
| - H.248 MG as reporting point at H.248 interface (RPH.248): | Data: R-8.5.1/1, R-8.5.1/3, R-8.5.1/5 Control: R-8.5.2/1, R-8.5.2/3 |
| - H.248 MG as filtering point (for RTCP traffic) (FP): | R-8.6/1, R-8.6/2, R-8.6/3, R-8.6/4, R-8.6/8 |
| - H.248 MG as loopback point (for RTP/RTCP traffic) (LP): | - |
| - Support of specific RTP topologies: | R-8.8/2 OR R-8.8/3 (NOTE 1) |
| - Others: | - |
| NOTE 1 – Conditional, dependent on the demanded round-trip delay (RTD) metric. E.g., the RTD “UEA to Nk to UEA“ would require topology “Back-to-back RTP endsystem”, and the RTD “UEA to UEB to UEA“ e.g. topology “RTP transport translator”. |  |

[End Proposed Correction]

____________________