| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4390 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP¦H.248.TLS: Semantical SDP issues with GCP control of IP transport connections and IP transport security sessions |  |  |  |
| Purpose: | Discussion |  |  |  |

# Summary:

Purpose of this contribution is to continue the discussion of this topic, which was started at last meeting.

---------------

# 1	Background

Continuation of discussion from last (2013-01) meeting, - see C-0052 “H.248 control mode: SDP-based vs Property-based H.248 signalling – Deficiencies of the SDP method” -, were we got general agreement on the conclusions of that contribution:

The SDP-based method is preferable for H.248 profiles with text encoding and SIP network environments, and under the condition that unambiguous semantics could be guaranteed. 
Such specific H.248 profiles also should take into account evolutionary aspects: an uncontroversial SDP semantic in profile version N may become ambiguous in a later version, e.g. when additional capabilities would be added.

It should be also ensured that the semantic of a specific SDP parameter/value combination in profile X should be identical with other H.248 profiles.

The SDP-based method is conditional: see listed terms in C-0052 evaluation

Thus, we’d like to (re-)use SDP from SIP signalling after the MGC-level “SDP mapping” function at H.248 (i.e., as LD/RD-embedded SDP) whenever possible.

NOTE: the SDP-based approach, - for TCP, TLS (, DTLS) H.248 Streams -, is indicated in clause 1 of H.248.TCP and H.248.TLS, and clauses 12 in these Recommendation are foreseen to define protocol procedures (when SDP is applicable), see parallel contributions AVD-4404 and AVD-4421 with initial input proposals.

Past contribution C-0052 did provide a generic discussion.

Purpose of this contribution is to focus on specific SDP arround work items H.248.TCP, H.248.TLS, H.248.WEBRTC (and new H.248.DTLS).

# 2	Discussion

## 2.1	Overview

There are following typical IP user plane protocol stacks for H.248 Stream endpoints in case of transport security support:

- MSRP/TLS/TCP

- BFCP/TLS/TCP

- T.38/UDPTL/DTLS/UDP

- WEBRTC “Data”/SCTP/DTLS/UDP

- WEBRTC “SRTP key exchange”/DTLS-SRTP/UDP

- “Application agnostic data”/TLS/TCP or “Application agnostic data”/TLS/SCTP

- “Application agnostic data”/DTLS/UDP

- …/DTLS/DCCP (RFC 5238)

- etc

Figure 1 illustrates the IP user plane layering. An H.248 IP-IP MG may be requested for different handling of such streams, such as protocol termination (e.g., TLS-to-non-TLS interworking), transparent forwarding, etc. Furthermore, the MG could be requested for NAT-T support in order to get successfully established end-to-end communication paths. There are two principal NAT-T areas:

- “L4/L3 NAT-T” and

- “L4+ NAT-T” (also known as bearer level application gateway (B-ALG)).

There are correspondent RFCs from IETF, related to the IP control plane, which define (e.g., SDP based) signalling for user plane communication (Fig. 1, right hand side).

Figure 1 –IP user and control plane

However, the control plane “SDP RFCs” are primarily designed for

- terminal to terminal communication (e.g., IMS UE to IMS UE), rather than terminal to network equipment (such as H.248 media gateways/servers) and

- NAT-less environment (as starting point).

Following deficiencies in “SDP RFC” might be observed, e.g.:

- Transport selection: 
i.e., bearer type indication, the well-known semantical issue with the SDP “m=” line <proto> element, which provides “protocol stack” information (i.e., single to multiple protocol layer(s)).

- End-to-end user plane connectivity: 
dedicated L4/L3 NAT-T methods require the capability of flexible control of “establishment directions” of the IP transport connection (e.g., forward direction, backward direction, simultaneous from both endpoints).

- Security aspects: 
support of all kind of security models require the flexibility in controlling the “establishment direction” of a IP transport security session.

Figure 2 provides an attempt of an inventory of related SDP RFCs, with focus on TLS, DTLS, TCP and application MSRP:

Figure 2 – IETF RFC for IP control plane (“open embedded PDF-file”)

The primary purpose of SDP is to control terminal configurations, but not network-located interworking function (media gateway) and support functions (media server). When SDP is also applied at the gateway control interface, then we need to consider a “service expansion”model, see Figure 3.

Figure 3 – Service expansion model due to value added network services by H.248 gateways
(Example ‘MSRP’)

The “terminal requested” communication service needs to be transformed (by the MGC entity) in a “gateway service”, taking into account service related policies, interworking models, network topology information, etc.

In the example of Figure 3: an incoming SDP Offer (as part of a SIP INVITE) carries at a very high level information with respect to the application, transport, possible NAT-T support and possible security support. The information relates to the end-to-end communication service. An interim SIP server/MGC entity is involved in the end-to-end SDP Offer/Answer negotiations at SIP level, i.e. providing a SIP B2BUA function.

With respect to work items H.248.TCP, H.248.TLS (, H.248.DTLS), a future safe design of an H.248 interface should consider (at least):

- Applications: WebRTC suite; instant messageing (MSRP), floor control (BFCP), telepresence;

- Transport: TCP, SCTP/UDP, UDP (, SCTP);

- NAT-T support: latching (H.248.37), ICE/STUN (H.248.50), BALG (H.248.78), P2P (H.248.84);

- Security (media|transport): SRTP, TLS, DTLS, DTLS-SRTP.

Further aspect:

- SDP O/A model (SIP interface): RFC 5939, which leads to consideration of H.248.80 at the H.248 interface.

## 2.2	Discussed issues (by this contribution)

We’d like to focus on two items at this meeting.

### 2.2.1	Item “Bearer type indication”

Issues:

- Firstly, the two control steps of “bearer type resource indication” and “establishment of bearer connection” could be separated and combined in SDP as well.

  - “Combined approach”: the SDP “m=” line <proto> provides the implicit semantic of bearer establishment as well (e.g., the underlying TCP connection in H.248.69 );

  - “Separated approach”: the additional SDP “a=setup:” attribute usage (besides the “m=” line) in case of TCP

- Secondly, it would be better to have dedicated “m=” line codepoints for “IP transport protocol type” and “IP media/transport security method” (e.g., by an extended SDP “m=” line)

Proposed proceeding:

- the issues could be principally solved by using explicit, non-SDP based H.248 signalling (such as an extended H.248 BNCCharacteristics package);

- however, we’d like to continue H.248.TCP and H.248.TLS by using the SDP “m=” line as bearer type indicator …

- … and explicit bearer establishment could be then based on correspondent H.248 packages (see AVD-4393 for TCP and AVD-4400 for TLS).

### 2.2.2	Item: “establishment control of IP transport connections and IP security sessions”

Starting point: syntax/semantics from SIP/SDP side are

- Direction of SIP call establishment

- SDP attribute for TCP role assignment, i.e., “a=setup:” and

- SDP attribute for TCP-supportive ICE, i.e., “a=candidate:”

Issues:

- these SDP attributes are TCP specific

- correspondent SDP usage for other L4 and L4+ is unclear (see also Fig. 2)

- these SDP attribute are part of H.248 LD/RD rather than LCD

Proposed proceeding:

- Use explicit H.248 packages (in H.248 profiles) in “ambiguous SDP” situations

# 4	Summary

The core H.248 packages for TCP and TLS bearer establishment are proposed to be updated according the discussion of this contribution.

The specific update proposals may be found under:

- H.248.TCP:

- AVD-4393	H.248.TCP: Clause 9 – Update of TCP basic connection control package

- AVD-4394	H.248.TCP: Clause 10 – Update of TCP extended connection control package

- H.248.TLS:

- AVD-4400	H.248.TLS: Clause 8 – Update of TLS session control package

- AVD-4401	H.248.TLS: Clause 9 – Update of TLS capability negotiation package

- AVD-4417	H.248.TLS: additional TLS extended session control package

____________________

Such semantical alignment may be ensured by specifying the SDP semantics in an H.248.x-Recommendation, - e.g. as package-less protocol procedures -, which may be then again referred by profile specifications.

Not to be misinterpreted: there’s support for basic scenarios, but not a consistent and complete set of capabilities.

Quote from IETF MMUSIC list: “Unfortunately SDP has a pretty confusing idea of "transport". The proto field identifies a whole stack of protocol layers.