| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4393 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Clause 9 – Update of TCP basic connection control package |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes package updates according the agreements of the 2013-01 meeting.

1	Introduction

Contributions C-4 and C-13 from last meeting (2013-01) resulted in an agreement in drafting three individual H.248 packages for TCP bearer control.

Copy of COM 16 – C 13/Figure 1 – H.248.TCP – Package redesign proposal – Overview

This contribution continues the update process as initiated by C-11 of the TCP basic connection control package.

2	Discussion

The updates are tightly related to the design of the Q.1950 gb package. However, some Q.1950 heritage seems to be minor or even superfluous for TCP bearer connection control. Correspondent package elements are indicated.

Some items in more detail:

- usage of event g/cause? Removed (only for unsuccessful scenarios)

- Signal parameters for (9.3.2) RelBNC signal. Not needed for TCP in our opinion (but perhaps for (D)TLS?), thus removed-
Note: we suppose that this mechanism was used in BICC to transfer some information between call control and bearer control signalling, with the purpose of transfering information to the remote endpoint (via BCP). This capability seems to be not needed between SIP and TCP (“we’ve discussed the option of ‘Reset’, which could be translated to a TCP RST flag setting, however, could identify any use case where the MGC would “reset TCP” …).

- Property for TCP role assignment added again (due to the semantical issue of event BNCChange, which would be context-dependent, whether a tcpbcc signal would be additionaly used or not …)

- New property called “Bearer connection establishment blocking”

  - motivated by event BNCChange, in order to avoid semantical overloading

  - related somehow to BICC/Q.2630 capability of blocking/unblocking

  - related to RFC 4145 “holdconn” semantic

  - aligned with parallel usage of H.248.84 TCP merge/relay mode

  - consideration of similar property for TLS/DTLS

  - compatible with tcpecc package capabilities

- updated signalling flows

- updated state model and state naming

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

References & abbreviations needs to be updated according agreed changes.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 9	TCP basic connection control package

|  | Package Name: | TCP basic connection control package |
| --- | --- | --- |
|  | Package ID: | tcpbcc (0x####) |
|  | Description: | This package provides the functionality to be able to establish and /release a TCP bearer connection. NOTE – The package design follows conceptually the generic bearer connection (gb) package according clause A.6/[ITU-T Q.1950] by using an event/signal-based approach for connection establishment/release control. Exceptions: a) bearer modification is not required for TCP; b) bearer release: using a package-defined codepoint instead of element g/cause. |
|  | Version: | 1 |
|  | Extends: | None |

## 9.1	Properties

## 9.1.1	Bearer connection establishment blocking

|  | Property Name: | Bearer connection establishment blocking |
| --- | --- | --- |
|  | Property ID: | bceb (0x0001) |
|  | Description: | This property defines the MG behaviour before and during TCP bearer connection establishment. NOTE 1: See Annex A concerning a state model based illustration. NOTE 2: This capability might be relevant for the prevention of other TCP processing behaviour such as “TCP relay” or “TCP merge” modes (TBC). |
|  | Type: | Enumeration |
|  | Possible values: | See Table X Table X – Bearer connection establishment preparedness (of TCP endpoints) Value; MG behaviour:. “blocked”, (0x0001); - Bearer resources are prepared (i.e., resources for TCP endpoint are reserved and allocated to H.248 termination/SEP); - TCP role assignment: still open, not yet decided; - TCP connection state = “CLOSED” (Figure A.1); - H.248 counterpart state = “Blocked” (Figure A.3); - any incoming TCP packet shall be silently discarded (e.g., TCP-SYN requests shall be not acknowledged), not any state transitioning from “CLOSED” (NOTE 1).. “unblocked”, (0x0002); - MG shall be ready for incoming or/and outgoing TCP bearer connection control procedures; - TCP role assignment: relates to an (initial) TCP server role effectively (“TCP Passive Open”) (NOTE 2); - TCP connection state: transitioning from “CLOSED” to “LISTEN” (Figure A.1); - H.248 counterpart state = “Idle” (Figure A.3); TCP bearer connection control options: - Incoming TCP-SYN: TCP server role confirmed, 3-way handshake proceeded; - Signal …: start 3-way handshake by sending of TCP-SYN; TCP client role assigned. NOTE 1: This semantic is similar (identical?) to SDP codepoint “a=setup:holdconn”, see [IETF RFC 4145]. NOTE 2: This semantic could be correlated with SDP codepoint “a=setup:actpass”, see [IETF RFC 4145].. |
|  | Default: | None. Subject matter of H.248 profile specifications. Examples: H.248 profiles for - MGs with support of “TCP proxy” mode only may choose default value “blocked” due to … (TBC); - MGs with additional support of “TCP relay” and “TCP merge” modes may choose default value “unblocked” due to (TBC) |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 9.1.1	TCP connection control endpoint role

|  | Property Name: | TCP connection control endpoint role |
| --- | --- | --- |
|  | Property ID: | csr (0x0001) |
|  | Description: | This property defines the MG behaviour for the TCP connection setup. |
|  | Type: | Enumeration |
|  | Possible values: | “notset” (0x0001) This value indicates that the MGC has not yet provided any information whether the MG shall act as active or passive entity concerning TCP bearer connection establishment or release.. “client” (0x0002) This value indicates that the MG will initiate the TCP connection setup as a TCP-client. “server” (0x0003) This value indicates that the MG will act as a TCP-server. Table X – TCP endpoint role assignment (at establishment phase) Value; MG behaviour:. not yet assigned; Any incoming TCP packet shall be silently discarded. TBD. |
|  | Default: | “notset” |
|  | Defined in: | Local |
|  | Characteristics: | Read/Write |

{Contributor’s note (2013-06): we think that we may delete that property proposal …}

## 9.2	Events

### 9.2.1	TCP connection state change (“BNC change”)

|  | Event Name: | TCP connection state change |
| --- | --- | --- |
|  | Event ID: | BNCChange (0x0002) |
|  | Description: | This event occurs whenever a change to a (TCP) bearer network connection occurs. For example a bearer has been established or a bearer has been released. The event is related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A. NOTE – The event design is aligned with event gb/BNCChange according clause A.6.2.1/[ITU-T Q.1950] by using an event/signal-based approach for connection establishment/release control. |

#### 9.2.1.1	EventsDescriptor parameters

##### 9.2.1.1.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | The type of state transitioning, given by the state after the transition. |
|  | Type: | Sublist of Enumeration {Contributor’s note: in order to serve scenario Events=1{tcpbcc/BNCChange{Type=[Est,Rel]}} |
|  | Optional: | No (E.g., in network scenarios with H.248 MGs always as originating point of TCP bearer connection establishment and release, i.e., only TCP client role assignments, and under the condition that the MGC don’t want to be notified about state changes.) |
|  | Possible values: | ### Value; ### Name; ### Final state (Annex A.2); ### Semantic. Est, [0x01]; Bearer Established; Established; a) activate MG for detection of incoming bearer control information (here: TCP SYN); b) notify MGC successful TCP bearer establishment.. Rel, [0x05]; Bearer Released; ### Idle; ### a) activate MG for detection of incoming bearer control information (here: TCP FIN); b) notify MGC successful TCP bearer release.. NOTE 1 – For release indication see: General Package E.1.2.1/[ITU-T H.248.1] g/cause Event. This method is also used by bearer control procedures according [ITU-T Q.1950].; IDLE; Bearer control procedures may fail. The g/cause event is used for unsuccessfull bearer establishment, unsuccessfull bearer release or unplaned bearer release. {Contributor’s note: codepoint g/cause=”NR” relates to normal release, i.e. overlaps with above value “Rel”. Hm?}. NOTE 3 – The numerical codepoint assignments are aligned with /[ITU-T Q.1950].. NOTE 1 – For release indication see: General Package E.1.2/H.248.1 Cause Event. |
|  | Default: | None |

#### 9.2.1.2	ObservedEventsDescriptor parameters

##### 9.2.1.2.1	Type of state change

|  | Parameter Name: | Type of state change |
| --- | --- | --- |
|  | Parameter ID: | Type (0x0001) |
|  | Description: | This is used to indicate what change has occurred to the TCP bearer connection. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | Est, [0x01] Bearer Established Rel, [0x05] Bearer Released |
|  | Default: | None |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”):

Package-less mode NOT possible. Any notification mechanism implies the explicit usage (and definition) of an H.248 event.

Conclusion: event as such should be kept …).}

## 9.3	Signals

## The signals are related to a simplified state transitioning model, - in contrast to [IETF RFC 793] -, which is described in Annex A.

### 9.3.1	Bearer Connection Control Role

|  | Signal Name: | Control Role BNC |
| --- | --- | --- |
|  | Signal ID: | CoRoBNC (0x0001) |
|  | Description: | This signal triggers |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 9.3.1.1	Additional parameters

##### 9.3.1.1.1	Role behaviour

|  | Parameter Name: | Role behaviour |
| --- | --- | --- |
|  | Parameter ID: | Role, 0x01 |
|  | Description: |  |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | "N" Not set (0x0001) "A" Active (0x0002) "P" Passive (0x0003) The semantic in detail is specified by Table #: |
|  | Default: | "P" |

### 9.3.1	Establish (BNC)

|  | Signal Name: | Establish BNC |
| --- | --- | --- |
|  | Signal ID: | EstBNC (0x0003) |
|  | Description: | This signal triggers the bearer control function to start bearer establishment (i.e., this signal is used to establish the TCP bearer connection: the MG takes the TCP client role and sends a TCP SYN segment). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 9.3.1.1	Additional parameters

#### None.

### 9.3.2	Release (BNC)

|  | Signal Name: | Release BNC |
| --- | --- | --- |
|  | Signal ID: | RelBNC (0x0004) |
|  | Description: | This signal triggers the bearer control function to send bearer release (i.e., this signal is used to close the TCP connection: the MG sends a TCP FIN segment). |
|  | Signal Type: | Brief |
|  | Duration: | Not applicable |

#### 9.3.2.1	Additional parameters

None.

{Contributor’s note (2013-06): below three signal parameters are just copied from A.6.3.3/Q.1950. It is yet unclear whether the parameters are required or not …}

##### 9.3.2.1.1	General Cause

|  | Parameter Name: | General Cause |
| --- | --- | --- |
|  | Parameter ID: | Generalcause, 0x01 |
|  | Description: | This indicates the general reason for the Release. It is optional to send this parameter in the normal case. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | "NR" Normal Release (0x0001) "UR" Unavailable Resources (0x0002) "FT" Failure, Temporary (0x0003) "FP" Failure, Permanent (0x0004) "IW" Interworking Error (0x0005) "UN" Unsupported (0x0006) |
|  | Default: | "NR" |

##### 9.3.2.1.2	Failure Cause

|  | Parameter Name: | Failure Cause |
| --- | --- | --- |
|  | Parameter ID: | Failurecause, 0x02 |
|  | Description: | The Release Cause is the value generated by the Released equipment, i.e. a release network connection. The concerned value is defined in the appropriate call control protocol. |
|  | Type: | OCTET STRING |
|  | Optional: | Yes |
|  | Possible values: | - |
|  | Default: | None |

##### 9.3.2.1.3	Reset

|  | Parameter Name: | Reset |
| --- | --- | --- |
|  | Parameter ID: | Reset, 0x03 |
|  | Description: | This parameter indicates whether or not the bearer should be reset. |
|  | Type: | Boolean |
|  | Optional: | Yes |
|  | Possible values: | 0 Indicates that the bearer release was not generated by a call level reset. (Default) 1 Indicates that a reset was generated on the call level. The MG must clear any resource associated with the bearer by sending bearer release signalling. |
|  | Default: | 0 |

{Discussion (related to clause 11) – Package-less semantic (“default MG behaviour”) concerning close/release:

Option 1:

The trigger for bearer release would be tight to the H.248 Command request SUB.req for stream or termination. Any support of TCP connection reuse would be excluded.

Other options?

None!?

Conclusion: a packageless semantic could be defined.}

## 9.4	Statistics

None.

## 9.5	Error Codes

None

## 9.6	Procedures

- incoming protocol procedures

- outgoing protocol procedures

- maintenance protocol procedures

### 9.6.1	TCP endpoint creation

The MGC is responsible for creating a TCP connection enabled SEP in the MG. This will be indicated on the H.248 interface through the use of the SDP “m=” line according the rules as specified in clause 13.4 in [ITU-T H.248.84].

The MG shall reserve “TCP bearer resources” (such as a “TCP connection state machine”) and allocates them to the SEP. The TCP role (client/server) is not (yet) determined by the SDP “m=” line information.

NOTE 1 – This functionality is synonym to the “bearer type indication” semantic as defined by the BNC Characteristics property (BNCChar) of the bearer characteristics package (BCP), see clause A.3.1.1/[ITU-T Q.1950].

### 9.6.2	TCP endpoint role assignment

The TCP role, - client or server -, is primarily relevant during the establishment phase of communication (of perspective of H.248 gateways). Table X summarizes MGC actions and their results in TCP role assignments:

Table X – TCP endpoint role assignment (at establishment phase)

|  |  | TCP role |  | H.248 indication |  | MG behaviour: |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | not yet assigned |  | H.248 state “Blocked” of SEP, either due to explicit signalling of bceb property or correspondent default value |  | Any incoming TCP packet shall be silently discarded. TBD |
| 2 | TCP server | H.248 state “Idle” by “unblocking” the SEP | Expect incoming TCP SYN, subsequent completion of 3-way handshake. |
| 2 |  | TCP client |  | H.248 state “Idle” of SEP plus signal EstBNC enabled. |  | ### Start of 3-way handshake for TCP connection establishment. |
| 4 | both | Not possible in TCP. (TBC) |  |

### 9.6.3	TCP bearer connection establishment

The incoming and outgoing side of TCP bearer connection protocol procedures is given by the assigned TCP role (see above clause 9.6.2) by the MGC.

The MG shall initiate the TCP connection setup according to the indicated role and the TCP connection setup handshake is performed by the MG without any further involvement of the MGC. The end of the TCP bearer connection establishment phase may be summarized according Table Y:

Table Y – TCP bearer establishment - Results

| TCP role | Result of TCP bearer connection establishment | MGC notification? |
| --- | --- | --- |
| Client (i.e., outgoing establishment side) | Successful | MGC dependent on subscription of event and parameter value tcpbcc/BNCChange{Type=[Est]} . |
|  | Unsuccessful | Unsuccessful establishment may be indicated to the MGC via a correspondent error code or event g/cause. |
| Server (i.e., incoming establishment side) | Successful | As client side. |
|  | Unsuccessful | As client side. |

### 9.6.4	TCP application data transfer

#### 9.6.4.1	MG externally

The TCP bearer connection is ready for application data transfer when remote and local TCP endpoints are both transitioned to TCP connection state “ESTAB”.

The StreamMode property of the LocalControl Descriptor affects the application data flow rather than the TCP control information used to establish or close the TCP connection. The requested MG behaviour is defined by Table Z:

{Editor’s note (2012-09 meeting): Consensus required on this behaviour.
Comment (2013-06 meeting): the principle problem relates to the fact that TCP allows to piggyback control and data information, i.e., an individual TCP/IP packet may not always be classified solely as part of the TCP data flow (-> H.248 media flow) or TCP control flow (-> H.248 control flow).
Reminder: H.248.1/7.1.7.1.1 StreamMode affects only H.248 media flow, but not H.248 control flow traffic.}

Table Z – Impact of StreamMode on TCP bearer traffic at external MG interface

| Traffic direction |  | StreamMode settings |  | MG behaviour: |
| --- | --- | --- | --- | --- |
| Outgoing | Sending enabled (i.e., values SendOnly, SendRecv, LoopBack) | all TCP packets sent |
|  | Sending disbled (i.e., values RecvOnly, Inactive) | only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are sent, independent of optional piggybacked data. |
| Incoming | Receiving enabled (i.e., values RecvOnly, SendRecv, LoopBack) | all TCP packets accepted |
|  | Receiving disbled (i.e., values SendOnly, Inactive) | only TCP packets with active usage of TCP SYN, FIN, RST and PSH information are accepted, independent of optional piggybacked data. |

TCP provides the application the capability to affect the data application transfer through the PSH- and the URG-flag. The usage of these flags is application specific and therefore must be specified in the application specific procedures.

#### 9.6.4.2	MG internally

This clause is only relevant for H.248 contexts with TCP SEPP(s), as outlined in clause 6.2.3.

Thus application data may be received at a SEP in time where the other SEP of the context is not yet able to process or send the application data. The behaviour of the MG in such a situation is implementation and/or application dependent. If application independent or unambiguous behaviour would be required, then the SEPP interlinkage property of the tcpecc package (see clause X.1.A.B) should be used.

### 9.6.5	TCP connection release

#### 9.6.5.1	Characteristic of TCP bearer connection release

This package supports bidirectional TCP bearer connection release only. Consequently, two TCP half-close procedures shall be performed, resulting in a four-way handshake. The transitioning from H.248 state Established to Idle of the local TCP bearer connection endpoint is only done after the successful processing of both TCP half-close procedures.

#### 9.6.5.2	Assignment of incoming or outgoing TCP bearer connection release

#### 9.6.5.2.1	Outgoing side

The local TCP bearer connection endpoint shall be explicitly prepared for outgoing TCP bearer connection release via signal RelBNC by the MGC.

##### 9.6.5.2.2	Incoming side

The local TCP bearer connection endpoint shall be able to process incoming TCP bearer connection release requests (TCP-FIN packet) at any point in time.

#### 9.6.5.3	MG bearer plane procedures

##### 9.6.5.3.1	MGC initiated release (outgoing side)

The RelBNC signal is used to initiate Bearer Release signalling (Table X1). The MGC may optionally include a GeneralCause, FailureCause or Reset indication. If a MG receives the reset indication it shall release all resources associated with the bearer.

Table X1 – MG bearer plane procedures (outgoing side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Starts first TCP half-close by sending TCP-FIN. Awaits TCP-FIN-ACK and TCP-FIN of second TCP half-close. Subsequent confirmation by a TCP-FIN-ACK. Transition to connection state H.248 Idle (TCP ‘CLOSED’). | Successful | Optional, via event BNCChange, ObservedEventsDescriptor parameter codepoint “Type=Rel”. The MGC default assumption is a “successfully released bearer connection”. TBD |
|  | Unsuccessful | There are following options: - Via event g/cause (TBD) - Via error code. - Via event adid/ipstop (see [ITU-T H.248.40]) (TBD) |

##### 9.6.5.3.2	Bearer initiated release (incoming side)

The MG is awaiting an incoming TCP-FIN packet as start event for TCP bearer connection release (Table X2)

Table X2 – MG bearer plane procedures (incoming side)

| MG bearer plane procedure | Result | MGC notification? |
| --- | --- | --- |
| Awaits first TCP half-close by detection of incoming TCP-FIN. Subsequent acknowledgment as well as start of second TCP half-close procedures. Transition to connection state H.248 Idle (TCP ‘CLOSED’) after reception of TCP FIN-ACK. | Successful | see Table X1 |
|  | Unsuccessful | see Table X1 |

### 9.6.5	Path MTU Discovery

Path MTU discovery (PMTUD) is normally started after successful establishment of TCP bearer connections, see [IETF RFC 1191] (TCP-over-IPv4), [IETF RFC 1981] (TCP-over-IPv6) and [IETF RFC 2923] (TCP specific problems with PMTUD). It is expected that bearer plane PMTUD procedures are completely hidden to the MGC.

PMTUD procedures may result in error conditions, such as received ICMP errors from remote side. Whether such situations should lead to error notifications of the MGC by the MG is for further studies.

### 9.6.6	Interaction of TCP-retransmissions with traffic control functions

Retransmissions of unacknowledged TCP segment leads to an inherent increase of the consumed transport capacity. There might be therefore possible side effects and interactions with traffic control functions, such as traffic policing or traffic shaping.

#### 9.6.6.1	Interaction with traffic policing: IP byterate policing according [ITU-T H.248.53]

Such a traffic policer entity is enforced at the ingress side of a TCP SEP. The traffic policer function shall consider all incoming TCP packets, without distinction between first sent or retransmitted TCP data.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Annex A

State modelling for 
TCP bearer connection endpoints

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

The TCP basic connection control package (clause 8) is tightly coupled to a state model, because the event (tcpbcc/BNCChange) and signal (tcpbcc/EstBNC) are related to state transitioning behaviour. The detailed user plane state model for TCP is given by [IETF RFC 793]. However, a simplified view is sufficient from perspective of H.248 gateway control procedures.

The underlying state model of package tcpbcc is described by this Annex.

## A.2	Original state model for TCP bearer connection endpoints

The principle model for TCP is comprised of 12 states and defined by Figure 6/[IETF RFC 793], see Figure A.1:

Figure A.1 – [IETF RFC 793] copy: TCP connection state diagram

## A.2	Simplified state model for H.248-based TCP basic connection control

The TCP basic connection control package makes a couple of assumptions, primarily:

- Bearer establishment:

  - neglects TCP simultaneous open;

  - interim states of 3-way handshake are not needed;

- Bearer release:

  - oneway closure not considered;

  - all interim states between the transitioning from TCP states ESTAB to CLOSED are not needed.

The resulting simplified model is depicted by Figure A.2:

Figure A.2 – Simplified state model for H.248-based TCP basic connection control

There are two remaining states, called Idle and Established in the H.248 state model. Signal tcpbcc/EstBNC is used for triggering state changes by the MGC, event tcpbcc/BNCChange prepares the MG for the reception of incoming bearer control information at user plane interface, and indicates successfully completed state transitions to the MGC.

The MGC may block the establishment of a TCP bearer connection, e.g., whilst TCP role assignment is yet unclear, ongoing call control signalling procedures, etc. Resulting in a third state Blocked from H.248 perspective, see Figure A.3.

Figure A.3 – Extended state model for H.248-based TCP basic connection control

Comment to the notion of “blocking”:

The concept of blocking/unblocking of bearer connection procedures is a known concept from BICC (see e.g. [b-ITU-T Q.2630]).

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Appendix III

Illustration of protocol semantics 
of the TCP basic connection control package

(This appendix does not form an integral part of this Recommendation.)

## III.1	Overview

The tcpbcc package (clause 8) defines the basic TCP bearer connection control with scope on support for establishment and release. The illustrated use cases are abstracted examples.

## III.2	Conventions

An H.248 context with a single SEPP; only one SEP (labelled as T1(S1)) is considered. The MG bearer interface (TCP) is highlighted besides the H.248 interface.

Furthermore all Figures indicate possible Event notifications to the MGC by the MG. The particular event(s) would be related to state changes of the local TCP bearer connection endpoint.

## III.3	Establishment of TCP bearer connections

### III.3.1	Successful establishment – Terminating side

See Figure III.1, termed as use case (E.1):

Figure III.1 – Successful establishment – Terminating side

### III.3.2	Successful establishment – Originating side

See Figure III.2, use case (E.2)

Figure III.2 – Successful establishment – Originating side

### III.3.3	Unsuccessful establishment

Figure III.3, use cases (E.3) summarizes the principle behaviour of unsuccessful TCP bearer connection establishment attempts:

Figure III.3 – Unsuccessful establishment

FIXTHIS: usage of g/cause still under discussion!

## III.4	Release of TCP bearer connections

### III.4.1	Successful release – Terminating side

See Figure III.4, termed as use case (R.1):

Figure III.4 – Successful release – Terminating side

FIXTHIS: usage of g/cause still under discussion!

### III.4.2	Successful release – Originating side

See Figure III.5, use cases (R.2) …

Figure III.5 – Successful release – Originating side

### III.4.3	Unsuccessful release

Figure III.6, use cases (R.3) summarizes the principle behaviour of unsuccessful TCP bearer connection establishment attempts:

Figure III.6 – Unsuccessful release

FIXTHIS: usage of g/cause still under discussion!

[End Proposed Correction]