| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4398 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TCP: Appendix II – Update proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to the signaling examples in Appendix II.

1	Discussion

The proposed updates are coupled to the package update proposals by AVD-4393 and AVD-4394 with regards to TCP basic and extended connection control. The examples illustrate only a view scenarios. The final update of Appendix II should be done when the package clause become stable.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 19 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

[Begin Proposed Correction]

Appendix II

Example call flows

(This appendix does not form an integral part of this Recommendation.)

## II.1	Assignment of TCP mode of operation

### II.1.1	Mode type: application agnostic TCP-proxy

The MGC needs to assign the TCP mode of operation to an H.248 context. This is fundamentally based on SDP information elements according [ITU-T H.248.84], clause 13.5. An example is provided by clause II.1.1.1.

The MGC could use also additional explicit H.248 signalling elements for the SEP individual assignment of TCP client/server roles, via the indication of incoming or outgoing TCP bearer connection establishment procedures, using the tcpbcc package capabilities as defined by this Recommendation, see example in clause II.1.1.2.

## II.2	Basic TCP bearer connection control

### II.2.1	Network configuration

The call flow is based on the network configuration as shown in Figure II.1:

Figure II.1 – Configuration “application agnostic TCP-proxy”

The use case shows an MG with two SEPs (labelled as T1(S1) and T2(S1)) configured in an application agnostic TCP-proxy mode. The configuration and the call flow are characterized as follows:

- MGC strictly control of the TCP bearer endpoints on both terminations

- Stream endpoint T1(S1): TCP-server, i.e., providing incoming TCP bearer establishment procedures (“TCP passive open” according [IETF RFC 793])

- Stream endpoint T2(S1): TCP-client, i.e., providing outgoing TCP bearer establishment procedures (“TCP active open” according [IETF RFC 793])

- TCP-connection for both terminations are setup in parallel

- TCP-connection released by remote TCP-endpoint of termination T1; application data may still flow from UE-Y to UE-X.

### II.2.2	Establishment

Figure II.2 illustrates a traffic flow example concerning the establishment of an end-to-end TCP connection between UE-1 and UE-2, which relates to two TCP connection segments from H.248 MG perspective (due to TCP proxy mode).

The example traffic flow indicates parts of application control signalling (here SIP/SDP with scope on SDP Offer/Answer information), gateway control signalling (H.248) and bearer plane traffic (here TCP packets for connection establishment).

The MGC follows an early reservation strategy of MG resources, thus initiates the creation of a “TCP-TCP” context in the MG already during still ongoing capability negotiations at application control level. In order to prevent accidental TCP state transitioning in the MG due to “too early TCP media”, the MGC decides to block explicitly TCP bearer connection establishment (via property tcpbcc/bceb).

As soon as application control signalling allows, the MGC enforces incoming (at SEP T1(S1)) and outgoing (at SEP T2(S1)) TCP bearer connection establishment procedures.

The MGC wants to be notified about successfully established TCP bearer connection segments in this example.

The signalling scenario is termed as “MGC strictly controlled” due to the fact that only the TCP basic connection control package is used. The property sepplink of the TCP extension connection control package would tightly coupled both TCP connection control procedures, see Appendix x.y.

Figure II.3 illustrates the traffic flow example for the release of the TCP connection without any involvement of the MGC. The release procedure on the signalling path is not shown.

Figure II.2 – Traffic flow example for an MGC strictly controlled TCP bearer conneciton establishment using the TCP-proxy mode

### II.2.3	Release

Figure II.3 illustrates the traffic flow example. for the use case when the MGC does not know which remote TCP endpoint initiates a TCP CLOSE procedures. Further, the default case of a bidirectional TCP bearer connection release is supposed. The MGC subscribes to event tcpbcc/BNCChange in order to be notified about release activities. It may be reminded that the notification is only issued when the local SEP is successfully transition to TCP connection state IDLE.

Figure II.3 – Traffic flow example for an incoming TCP bearer connection release, explicitly propagated by the MGC in outgoing direction

Figure II.4 provides another release example: the MGC enforces a TCP bearer release by initiating parallel TCP bearer connection release procedures towards both remote TCP endpoints X and Y:

Figure II.4 – Traffic flow example for outoing TCP bearer connection release procedures towards both remote TCP endpoints X and Y

## II.3	Extended TCP bearer connection control

### II.3.1	Establishment

This use case applies to the same configuration than the previous use case as depicted in Figure II.2. There is not any necessity for explicit TCP bearer establishment control by the MGC in this example, because e.g. of lacking NAT traversal support, or application specific, or security dedicated requirements. The direction of TCP bearer establishment is also unimportant from MGC point of view. The  MGC only want to wait with TCP establishment as long as application control level capability negotiations become stable, hence the MGC intially blocks possible establishment. Later on, TCP bearer establishment is unblocked and the interlinkage at the concerned stream endpoint pair is enabled, in both directions at the example of Figure II.5.

The notification of successful TCP bearer establishment is again an optional feature, up to the MGC level application/service control logic.

Figure II.5 – Traffic flow example for an extended TCP bearer connection establishment

### II.3.2	Release

Figure II.6 provides an example for an MG autonomous release of the TCP bearer connection. The MGC enables interlinkage in both traffic directions in this example, e.g. because the MGC does not know which remote TCP endpoint is initiating a TCP release.

Figure II.6 – Traffic flow example for an extended TCP bearer connection release

[End Proposed Correction]

____________________