| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4407 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.TLS: con't DTLS discussion |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution makes a proposal how to proceed with support of DTLS besides TLS.

1	Introduction

The DTLS topic was already subject of discussion at the past meetings. A living list was created and parked in H.248.TLS Appendix IV “Living List related to DTLS and DTLS-SRTP”.

2	Discussion

We did further evaluate H.248-controlled DTLS security sessions since the January meeting, coming to the conclusions that the overlappings are dominating and that there are not any significant differences between the H.248 control of TLS and DTLS bearers from perspective of the MGC entity. The result as such is also not really surprising due to the tight alignment of both protocols (DTLS is based on TLS).

Looking at the three main interfaces in more detail:

- MGC – call control signalling (such as SIP SDP)

  - out of scope of Q.3/16 and H.248.x documents

- Gateway control signalling (H.248):

  - Impact of underlying transport?
DTLS and TLS should be fairly similar, primarily due to the fact that TLS is designed as IP L4 independent protocol, i.e., TLS is independent of transport protocol such TCP or SCTP. DTLS is consequently also L4 independent. E.g., there shouldn’t be any dependencies of UDP.

  - Impact of key exchange methods?
DTLS needs additionally to consider “SRTP” (and the correspondent plethora of SRTP key exchange mechanisms).

- MG – bearer plane:

  - Bearer plane procedures are slightly different between TLS and DTLS, but should be almost hidden to the MGC (e.g., (D)TLS negotiations?, (D)TLS alert handling?, (D)TLS statistics?)

3	Conclusions

Concerning the question of DTLS documentation: we are still tending to option #2 (see H.248.TLS, Appendix IV.3), - DTLS in a self-contained, new H.248.DTLS document -, primarily due to the instantiation character of DTLS vs TLS and the additional DTLS-SRTP topic. We don’t expect any new H.248 syntax specific to DTLS control

The separate H.248.DTLS Recommendation would then mainly provide the following information:

a) DTLS specific use cases;

b) applicability statements of H.248 packages from Recommendation H.248.TLS for their usage with DTLS bearers;

c) procedures for DTLS-SRTP; and

d) MG bearer plane differences of DTLS vs TLS.

4	Proposal

We’d like to start a new work item H.248.DTLS. Correspondent contribution were submitted, see:

- AVD-4418: work item reques;

- AVD-4419: initial skeleton.

____________________