| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4415 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTPMUX: clause 3 – IETF progress on terminology definitions and clarifications |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

Some information concerning the progress of terminology definitions in IETF.

1	Background and recent progress in IETF

An important scope item of H.248.RTPMUX is related to terminology with the purpose of a consistent, aligned view between IETF WGs and ITU-T H.248 in case of multiplexed media configurations.

The IETF concluded (at IETF 85 / 2012-11) to provide a collected view across involved WGs (i.e., RTCWEB, CLUE, MMUSIC, AVTCORE) as a kind of taxonomy document:

Draft: draft-lennox-raiarea-rtp-grouping-taxonomy, “A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources”

For ITU-T Q.3/16 (“the perspective of H.248 gateways (MGC, MG)”) is of crucial importance the relation and coordination of identifier spaces between the (at least) three interfaces of application control, gateway control and IP user plane, as elaborated already in AVD-4272 (2012-09 meeting):

- copied text again:

## 2.2	Identifier spaces (in user, control and application plane)

The data objects (such as e.g. a protocol data unit at a particular protocol layer), with communication information as carried over the network or/and processed by involved network entities, are referred by identifiers. Figures 3 provides a rough view on WebRTC, from perspective on a WebRTC gateways.

Figure 3 – Identifier spaces (in user, control and application plane) at the example of WebRTC

It may be noted that there are multiple, different identifier spaces in use, primarily due to multiple different protocols used (at different interfaces). Secondly, in scenarios with open application programming environments (such as with browsers in WebRTC) due to additional consideration of API specification (e.g., the common work on WebRTC by IETF RTCWEB and W3C WEBRTC imply a correlation and mapping between “WebRTC data objects” at the IP network interface (IETF) and the browser (W3C)).

There are consequently mappings between different identifier spaces, e.g. between SIP-and-H.248 at MGC level and H.248-and-IP stack at MG level.

Such identifier mappings are well known capabilities, but demand for special attention in case of RTP multiplexing models (see later).

Figure 4 outlines the legacy mapping model, as applied by H.248 and IP bearer traffic, between the identifier spaces of H.248 and IP protocol stacks. The illustration is based on the examples of [15], the discussion of “valid media format suites” in the context of H.248.

Figure 4 – H.248 identifier space and media stream/flow model

There is (was?) a 1:1 correlation between an H.248 flow component (media flow or control flow) and IP traffic flows in case of unmultiplexed RTP traffic.

There was some progress on the subject of identifier spaces, and the underlying modelling of information flows and data objects, at the IETF 86 meeting. E.g.

draft-burman-rtcweb-mmusic-media-structure, “Multi-Media Concepts and Relations”

provides a necessary and good attempt on the modelling subject. An initial UML model may be found at slide 7 of www.ietf.org/proceedings/86/slides/slides-86-avtext-5.ppt:

2	Problem focus for H.248 gateway entities:

2.1	Generic view on IP-IP devices

We’d like to suggest a slightly different perspective on the identifier space subject (in contrast to Fig. 3/AVD-4272). Figure A illustrates a generic model (labelled as IETF view due to the high level assumption of generic IP-IP in-media-path devices):

Figure A – Identifying IP media traffic (of multiplexed traffic structures) 
by interim IP-IP in-path devices (IETF view)

The problem statement may be outlined as followed:

The IP-IP in path device is requested to identify incoming IP packets in order to associate them with an application level data object (e..g., a media format/type and embedded component such as camera X, video codec Y, codec layer Z, etc).

This problem is also known as IP flow identification (in IETF WG IPFIX) or flow/application identification (in ITU-T SG13; e.g., Q.7/13, see Y.2770, Y.SupTerm).

I.e., there must be a traffic descriptor (e.g., flow descriptor, application descriptor) which provides sufficient and unambiguous information to the IP-IP in path device concerning the detection capability of correspondent IP packets and embedded multiplexed data structures (as subject of protocol data units at various protocol layers above the network layer).

Such an perspective allows to formulate following requirement on the “terminology definition process”:

The definition of term shall contain following integral parts:

- a name;

- a description in behavioural language and

- a formal specification on basis of an identifier tight to protocol syntax of the traffic (e.g., a k-tuple of protocol related information elements plus optional value associations).

The third condition would guarantee the technological unambiguity, which is pre-requisite for technical usage.

2.2	Terminology definition from protocol syntactical perspective

Figure B depicts a generic structure of a k-tuple, as possible part of a terminology definion:

Figure B – Terminology definition from protocol syntactical perspective
- General structure of a k-tuple as essential part of a term definion

It should be noted

- that the relation (2) between value (3) and information element (1) if often of type “equality”, but could be principally of any other type as well;

- that the value (3) element could be a single value, a set of values, a value range, unspecified, wildcarded, etc.

2.3	Examples

Some examples shall illustrate the idea of a protocol syntactical based term definition process:

e.g., Table 1 provides the “RTP source” definition (note: still floating, not yet fixed by IETF):

Table 1 – Term “RTP source”

| Term: RTP source |  |
| --- | --- |
| 1. Name: | RTP source |
| 2. Description: | … |
| 3. Protocol based identification |  |
| 3.1 possibility? | Yes |
| 3.2 if yes, by identifer: | RTP source: Traffic Descriptor = { IE1: IP address = y1, IE2: IP L4 port = y2, IE3: IP transport protocol = y3 (= UDP, DCCP or TCP}, IE4: IP L4+ protocol = y4 (= RTP), IE5: RTP SSRC = y5, IE6: RTP CNAME = y6 } |

E..g., Table 2 illustrate a “media source” definition (note: again not sure whether this is inline with latest IETF understanding), which could be based on the “RTP source” definition (because the traffic of multiple media sources may be multiplexed on a single RTP source)

Table 2 – Term “media source”

| Term: media source |  |
| --- | --- |
| 1. Name: | media source |
| 2. Description: | … |
| 3. Protocol based identification |  |
| 3.1 possibility? | Yes |
| 3.2 if yes, by identifer: | Media source: Traffic Descriptor = { IE1-6: TrafficDescriptor{RTP source}, IE7: RTP PT = y7 } |

And Table 3 provides an another example:

Table 3 – Term “RTP endsystem”

| Term: RTP endsystem |  |
| --- | --- |
| 1. Name: | RTP endsystem |
| 2. Description: | … |
| 3. Protocol based identification |  |
| 3.1 possibility? | Yes |
| 3.2 if yes, by identifer: | RTP endsystem: Traffic Descriptor = { IE1: IP address = y1, IE2: IP L4 port = y2,1 (RTP media) AND y2,2 (RTCP control), IE3: IP transport protocol = y3, IE4: IP L4+ protocol = y4 (= RTP) } |

2.4	The correspondent H.248 model

Figure C provides the correspondent model from H.248 perspective. However, there is not any conceptual difference concerning the discussed terminology definition process. The IETF terms as such would be unchanged. However, H.248.RTPMUX could provide the information how the IETF terms would relate to the “identifier space used by H.248 signalling” (see above Fig. 3/AVD-4272).

Figure C – Identifying IP media traffic (of multiplexed traffic structures) 
by interim IP-IP in-path devices (ITU-T Q.3/16 view)

2.5	Classification of terms:

Coming back again to the IETF taxonomy attempt (draft-lennox-raiarea-rtp-grouping-taxonomy), the terms as described by this draft may be classified in:

1) Terms, which may be unambiguously derived from a IP media path, network located traffic identification instance (e.g., an IPFIX entity for flow/application identification):

1.1) Names of protocol elements itself ("PCI elements" (of IP protocol stack identifier space))

e.g., IP SA, IP TTL, RTP PT, RTP SSRC, RTCP SDES CNAME, ...

1.1) (Compound) Terms, builded by a combination of simple terms (1.1), represented by N-tuples

e.g., bidirectional IP transport connection (-> 4-tuple of ...); RTP session (...); RTP endsystem, track, media stream

2) Terms, which may not be derived by IP media path traffic observation (e.g., due to information exchanged in the IP signalling path (e.g., via SIP/SDP), or due to endpoint local only concepts (?))

e.g., RTP receiver, scene?, ...

3	Conclusions:

This contribution presents a concept how the terminology definition process could achieve unambiguous terms, - from perspective of technical systems involved in processing of such packet traffic.

The idea as such is still pre-mature, also the provided examples are only the results of first studies.

It is recognized that possibly not all terms in discussion could be covered. However, we think that the very majority of (H.248 gateway) relevant terms should be possible.

The concept as such should be firstly addressed against draft-lennox-raiarea-rtp-grouping-taxonomy,  i.e., as soon as this draft becomes more mature to work on correspondent protocol based identifiers for each term..

At this stage, we’d like to suggest to add the two new IETF documents to our H.248.RTPMUX living list.

4	Proposal

It is proposed to accept the marked up changes below. 
Changes are marked up against TD 56 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

Appendix II
Living List related to IETF documents under observation

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix would be again deleted as soon as the reference (I.1) and terminology (I.2) discussion becomes stable).

## II.1	Important references

Following IETF documents (from four IETF working groups) are relevant to the RTP multiplexing discussion:

[x1]	A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources
IETF “RAI Areaa” / WG AVTEXT? (still open)
Draft: draft-lennox-raiarea-rtp-grouping-taxonomy
URL: http://tools.ietf.org/html/draft-lennox-raiarea-rtp-grouping-taxonomy-00

[1]	Guidelines for using the Multiplexing Features of RTP
IETF AVTCORE
Draft: draft-westerlund-avtcore-multiplex-architecture 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multiplex-architecture-02.txt

[2]	Multiplexing RTP Data and Control Packets on a Single Port
IETF MMUSIC
RFC 5761

[3]	“BUNDLE”
= Multiplexing Negotiation Using SDP Port Numbers 
IETF MMUSIC
Draft: draft-ietf-mmusic-sdp-bundle-negotiation
URL: http://tools.ietf.org/wg/mmusic/draft-ietf-mmusic-sdp-bundle-negotiation/

[4]	“RTP for WebRTC”
 = Web Real-Time Communication (WebRTC): Media Transport and Use of RTP
IETF RTCWEB
Draft: draft-ietf-rtcweb-rtp-usage
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

[x2]	“Multi-Media Concepts and Relations” 
IETF RTCWEB
Draft: draft-burman-rtcweb-mmusic-media-structure
URL: http://tools.ietf.org/id/draft-burman-rtcweb-mmusic-media-structure-00.txt

[5]	“RTP for Telepresence”
 = RTP Usage for Telepresence Sessions
IETF CLUE
Draft: draft-lennox-clue-rtp-usage
URL: http://tools.ietf.org/id/draft-lennox-clue-rtp-usage-04.txt /

Complementary:

[6]	Source-Specific Media Attributes
IETF MMUSIC
RFC 5576

…

[End Proposed Correction]

____________________