| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4428 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.WEBRTC: clause 7 (use cases) - comment on editor's note |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution comments the editor’s note from clause 7.

1	Background

See clause 7:

{Editor’s note (2013-01 meeting): there might be other potential use cases (e.g., interworking between WebRTC-enhanced IMS networks and existing IMS infrastructures, etc). Contributions are solicited.}

2	Discussion

WebRTC and IMS are related in two principal directions:

- The IMS core network supports interworking with other (i.e., non-3GPP defined) IP networks, using an H.248 gateway (MGC = IBCF; MG = TrGW). 
Such an IP network may be termed as IETF WebRTC domain when SIP signalling and media transport interfaces are solely based on IETF RFCs.

- The IMS is evolving over subsequent 3GPP releases: there’ll be coming 3GPP releases with integrated WebRTC support, but on the other side also still pre-WebRTC 3GPP releases will be in the field.

Figure A illustrates two possible IMS options with WebRTC support, - and there might be also other options. The two scenarios focus on the H.248 WebRTC gateway (as in work by ITU-T Q.3/16):

Figure A – Further use cases: Principle IMS evolution options
(“a pure H.248 WebRTC gateway perspective”)

There’s an SIP-based IP signalling path and a media-over-UDP based IP bearer path in case of WebRTC communication from H.248 WebRTC gateway perspective. The three network domain types in Figure A are given by following characterization:

Table 1 – Network domain types (in Fig. A)

| Name | SIP-based IP signalling path | media-over-UDP based IP bearer path |
| --- | --- | --- |
| IETF WebRTC domain | IETF SIP/SDP profile as currently defined in IETF MMUSIC (primarily SDP O/A for UDP multiplexed “media streams”) | IETF RTP usage profile for WEBRTC, as currently defined in IETF RTCWEB & AVTCORE |
| “Native” 3GPP IMS domain (= 3GPP releases without yes support of WEBRTC) | Abstraction by focus on Gm: 3GPP Rel-x 24.229 SIP profile | Given by Mb: 3GPP Rel-x 26.114 RTP profile |
| “Extended” 3GPP IMS domain (= 3GPP releases with additional support of WEBRTC) | Let’s call it Gm+: 3GPP Rel-x+ 24.229 SIP profile with additions from IETF MMUSIC | Let’s call it Mb+: 3GPP Rel-x+ 26.114 RTP profile with additions related to multiplexed UDP transport conncetions, etc etc |

It may be noted that the various IMS evolution options are dependent on the UE capabilities concerning WebRTC support. E.g., there might be even multi-WEBRTC-mode kind of UEs (as indicated in Fig. A).

H.248 WebRTC gateways are positioned to serve the various network evolution scenarios. Different interworking use cases would be reflected by different “H.248 WebRTC profiles”.

3	Conclusion

Two example WebRTC IMS evolution scenarios were discussed. However, we are not proposing to add them to H.248.WEBRTC because

1st 	IMS dedicated use cases are subject and responsibility of 3GPP (and not ITU-T);

2nd	Both discussed examples are actually just instantiations of existing use case #2, hence already covered by H.248.WEBRTC.

The editor’s note is solved and could be deleted in our opinion.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 22 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal #1:

[Begin Proposed Correction]

# 7	H.248 WEBRTC gateways: example use cases

Some exemple use cases are described in this clause.

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A is always using WebRTC browser environment.

### 7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 1.

Figure 1 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a browser-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two networks where the WebRTC endpoints reside, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture of the networks where the WebRTC endpoints reside, etc.

### 7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 2 depicts an interworking scenario with NGN/IMS networks.

Figure 2 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed to be not fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

### 7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 3 outlines the general request in supporting of still existing PSTN/ISDN infrastructures.

Figure 3 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. H.324, H.320).

## 7.2	Multipoint communication

Multiparty calls are in scope of WEBRTC and may demand for “RTP mixer”, as e.g. provided by media servers. Such a network scenario may therefore relate to H.248 profile types with regards to media servers (aka media resource processors).

[End Proposed Correction]

____________________