| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4437 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June, 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Supp.Prio: Appendix I - Update proposal |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to progress the update discussion of Appendix I.

1	Background

The discussion is lasting since 2, 3 meetings. This contribution tries to continue the discussion from C.28R1 of the last meeting:

Q.3 report (2013/01):

| C.28R1 | H.Supp.Prio: Clause I.2 - Resolution of editor's notes | Alcatel-Lucent Deutschland AG, Alcatel-Lucent Shanghai Bell |
| --- | --- | --- |

This Contribution was discussed and the proposals were agreed with the following changes:

- Rather than use the term "Native" and "Generic" the terms "bearer dependent" and "bearer independent should be used.

- II. b. … translation would always be require <<in the MG>>>.

- It was felt that the Recommendation would benefit from further definition of each variant in Appendix I. As a result it was agreed to add an editor’s note stating that variant 1 and 2 need further description.

This contribution attempts to resolve the raised items.

2	Discussion

The crucial point is related to the clarification of semantics related to “mapping” (or “translation”, “conversion”, etc). It is obvious that the “mapping function” (MF) may be characterized by

- it’s location (i.e., MFMGC, MFMG) and

- kind of changed information-

The 2nd aspect is related to hierarchical protocol information, there we could identify three (or four) relevant levels for our consideration, e.g.:

We’d like to suggest a mapping function (MF) concept following a mapping model with the hierarchy of (a) protocol mapping, (b) type mapping and (c) value mapping (i.e., in general MFa,b,c).

The change proposal provides some examples for mappings functions MFMGC and MFMC.

Further, there is a cost factor associated to MFs (i.e., CMF,MGC and CMF,MG), given by required mapping algorithm complexity, resource consumption (CPU time, memory) and minimal mapping times (as performance objective). Such a cost factor could be also considered as mapping efficiency.

We don’t want to evaluate mapping efficiency in very detail, however, it could be concluded that the less kind of mapping required, the higher the mapping efficiency of a MF.

Coming back to Appendix I/H.SuppPrio, there are variants indicated with primary scope on MFs with type or/and value mapping (MFb,c)- The “principal H.248 signalling options” (Appendix I.2) are then given by the combination of following characteristics:

- Kind of MFb,c?

  - at MGC level: call control protocol related codepoints are mapped to H.248 codepoints (‘BD or ‘BI) format (by the MFMGC(b,c)); or/and

  - at MG level: mapping of H.248 codepoints to final IP bearer path format (by the MFMG(b,c));

- H.248 signalling format

  - ‘bearer dependent, i.e., native format as used in IP bearer packets, hence type mapping not necessary;

  - ‘bearer independent, i.e. a type mapping would be always required.

Figure A summarizes all principal options:

Figure A – Appendix I.2/H.SuppPrio 
– Complete overview of “principal H.248 signalling options”

Figure B highlights the correspondent MF types at MGC and MG level. The “-“ indicates the absence of mapping requirements.

Figure B – Correspondent MF types at MGC and MG level

Present Appendix I.2 considers 3-out-of-8 theoretical options.

An update is proposed by alignment of terminology and the introduction of a defined concept for mapping function-

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the output draft of the last meeting TD 15R1 (WP1/16) (01/2013).

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined Elsewhere

This Supplement uses the following term defined elsewhere:

3.1.1	emergency telecommunications service (ETS) [ITU-T E.107]: A national service providing priority telecommunications to the ETS authorized users in times of disaster and emergencies.

## 3.2	Terms defined in this Supplement

3.2.1	Bearer-dependent decision point: the MGC as the primary decision point with respect to the mapping of call control level information to gateway control signalling and the concerned H.248 context, termination and/or stream entity as bearer level element. The MGC provides all necessary information about bearer plane protocol stack, supported QoS technologies, etc.

NOTE 1 – This is the usual model in the master-slave relationship of H.248 gateways.

NOTE 2 – The synonym of “technology-dependent …” / “technology-independent …” originates from the ITU-T Resource and Admission Control Function (RACF) architecture (see [ITU-T Y.2111]).

3.2.2	Bearer-independent decision point: this is in contrast to clause 3.2.1, the MGC is still the primary decision point with respect to the mapping of call control level information to gateway control signalling but is lacking partially or fully bearer related information. The MGC applies therefore some kind of generic signalling and delegates the final mapping to the MG.

NOTE 1 – The notion of “bearer-independent …” originates from the Bearer Independent Call Control (BICC) architecture (see ITU-T Q.19xx-series of Recommendation); applied e.g. in the H.248 profile “Bearer independent call bearer control protocol” according [ITU-T Q.1950].

3.2.x	Mapping function: function, located in H.248 entities (MGC, MG), for mapping protocol objects between two interfaces (MGC: between call control and gateway control signalling; MG: between gateway control signalling and user plane protocols). The mapping function relates to following, hierarchical information:

a) protocol type (“protocol mapping”) (e.g., H.248, H.323, SIP, IP; protocol type mapping: MGC: SIP to H.248, SDPoSIP to SDPoH.248; MG: H.248 to IP);

NOTE 1 – The MGC located mapping function (MFMGC) could affect the two protocol levels (a1) application control protocol (e.g., H.323, SIP, BICC) and an optional embedded (a2) media description protocol (e.g., SDP).

b) information element type (“type mapping”) (e.g., signalling element type, IP header field) and

c) information element value (“value mapping”).

NOTE 2 – The combination of signalling element/type is also known as codepoint.

The mapping function may affect (or not) all three levels in every combination.

Examples:

- MGC located mapping function (MFMGC):

- the SDP “b=” line is mapped from SIP/SDP to H.248/SDP with a modified “b=” line value: type (a1, c) mapper (because the information element type (SDP “b=” line) is unchanged.

- the SIP Resource Priority Header (RPH) value x1 is mapped to
2.1 H.248 priority indicator with value x2 (see also Table 2/[ITU-T H.Sup9]): i.e., type (a, b, c) mapper;
2.2 H.248 ds/dscp property with value x3: i.e., type (a, b, c) mapper;
2.3 H.248 vlan/pri property with value x1: i.e., type (a, b) mapper (because the Ethernet priority is here identical to the SIP RPH value);

- the SDP “a=qos” attribute is mapped from SIP/SDP to H.248/SDP ####

- MG located mapping function (MFMG):

- the H.248 priority indicator with value x1 is mapped to
4.1 IPv6 traffic class with value x2: i.e., type (a, b, c) mapper;
4.2 IPv6 traffic class with value x1: i.e., type (a, b) mapper; 
4.3 Ethernet priority header with value x1: i.e., type (a, b) mapper;

- the H.248 ds/dscp property with value x1 is mapped to
5.1 IPv6 DiffServ field with value x1: i.e., type (a) mapper;
5.2 IPv6 DiffServ field with value x2: i.e., type (a, c) mapper; 
5.3 Ethernet priority header with value x1: i.e., type (a, b) mapper;

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

Appendix I

H.248 profiles and Context Attributes for priority traffic handling

The purpose of this Appendix is an analysis concerning signalling support for priority traffic handling.

## I.1	Status H.248 Profiles

H.248 profile definitions include Context Attribute support information (see profile definition template in [ITU-T H.248.1]). There are many H.248 profile specifications published by standard development organizations (SDOs). In the context of this Supplement are of particular interest are profiles for H.248 IP-IP gateways; such as the 3GPP-defined profiles for their H.248-based reference points Ix and Iq ( [ETSI TS 129 238] and [ETSI TS 129 334]; which are both based on ETSI TISPAN Ia profile [ETSI TS 183 018]).

The following table summarizes the status for Ix and Iq interfaces:

Table II.1 – 3GPP Profiles and typical support of Context Attributes (status: Rel-11)

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| … |  |  |
| Priority Indicator | Optional (NOTE 2) | 0-15 (Note 3) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Call Indicator | No | NA |
| … |  |  |
| NOTE 2: This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in [3GPP TS 22.153]. NOTE 3: Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

It may be noted that there’s actually support of emergency calls only in pure 3GPP environments, and additional priority level support in ETSI TISPAN and 3GPP instantiations.

## I.2	Discussion of principal H.248 signalling options

H.248 supports basically the provisioning of protocol parameter values, which might be sometimes an alternative to the usual signalling of protocol values. There are further different kind of H.248 decomposed gateway models, such as bearer-independent or bearer-dependent style of operation (see clauses 3.2.2 and 3.2.1). The alternatives lead to various options which are outlined in this clause, using the concrete example of DiffServ-based QoS marking as selected technology for priority traffic support. The different options are also related to kind of mapping function (MF; see clause 3.2.x) involved at MGC and/or MG level.

### I.2.1	Overview

The “principal H.248 signalling options” are given by the combination of following characteristics:

- Kind of mapping function (related to type and/or value mapping, MFb,c)

  - At MGC level: call control protocol related codepoints are mapped to H.248 codepoints (‘bearer dependent’ or ‘bearer independent’) format (by the MFMGC(b,c)); or/and

  - At MG level: mapping of H.248 codepoints to final IP bearer path format (by the MFMG(b,c));;

- H.248 signalling format

  - ‘bearer dependent’, i.e., QoS marking codepoints signalled already in the format as used in IP bearer packets (hence, so-called type mapping (MFb) not necessary);

  - ‘bearer independent’, i.e. (I.b) type mapping (MFb) would be always required in the MG.

Three meaningful combinations are described subsequently.

### I.2.2	Bearer dependent H.248 signalling

H.248 signalling (i.e., ADD and MODIFY request commands) carry the bearer dependent DiffServ codepoint (as e.g. assigned for priority traffic). The MG directly uses that DiffServ codepoint for QoS marking outgoing bearer packets. There are two variants from MGC perspective, see Figure I.1:

Figure I.1 – Controlling of MG-level QoS marking function – Variants 1 and 2

Details:

- Variant 1 could avoid type mapping and value mapping at MGC and MG level.

- Variant 2 is typically used when call control signalling either does not support signalling of DiffServ values or when a local DiffServ translation is applied. In this case, the QoS marking is provisioned at the MGC, and the MGC provides the DiffServ mapping. Thus, there is type/value mapping at MGC level, but not any such mapping additionally necessary by the MG.

### I.2.3	Bearer independent H.248 signalling

The DiffServ codepoint for a particular packet bearer traffic class might be also provisioned at MG level. This implies the necessity of a correspondent mapping function at MG level (MGMF, see Figure I.2) besides the MGC level MF.

Figure I.2 – Controlling of MG-level QoS marking function – Variant 3

Details:

- Variant 3 does not require the explicit signalling of DiffServ codepoints at the H.248 interface. Instead another H.248 protocol element could be signalled to trigger bearer packets marking with appropriate DiffServ codepoints.

- Variant 3 needs to take into account that an H.248 profile might support in addition DiffServ signalling (i.e., bearer dependent), which may lead to a mix of both H.248 signalling options (i.e., ‘bearer dependent’ and ‘bearer independent’ formats used in parallel), and hence the careful consideration of possible interaction issues.

## I.3	IEPS/ETS traffic support: question of signalling (stage 3) solutions?

There are different signalling (stage 3) alternatives as outlined by the previous clause, in order to satisfy requirements (stage 2) for IEPS/ETS traffic support:

- Native approach: bearer dependent signalling (stage 3) method based on the explicit H.248 signalling element for that purpose, the ContextAttribute IEPS call indicator.

- Emulation approach: bearer independent signalling variant by reuse of existing signalling (stage 3) capabilities, which would relate here to the support of ContextAttribute Priority indicator for IEPS/ETS.

Discussion:

Option (A) looks pretty straightforward, but there’s a catch with this approach: this ContextAttribute was just introduced with H.248.1 Version 3, which is often not yet supported by existing H.248 profile as published by SDOs.

For instance, there is reluctance (e.g., in case of the examples of clause I.1) to upgrade from H.248.1 Version 2 (as the core protocol baseline for these profiles) to Version 3, just due to the functionality associated with a single additional protocol element. However, the introduction of H.248.1 V3 might be basically mitigated when a stepwise addition of V3 capabilities through profiles would be considered (“instead of the all or nothing approach”). Appendix III provides a more detailed discussion of such a phased proceeding.

[End Proposed Correction]

____________________