| Rapporteur Meeting of Questions 1, 2, 3, and 5/16 |  |  |  | AVD-4443 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Polycom |  |  |  |
| Title: | H.TPS-AV Use of UltraHD in Telepresence Systems |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

# Introduction

The UltraHD colour gamut is missing from clause 7.1.2.1 H.TPS-AV.  Polycom proposes that it be added.

# Discussion

UltraHD video resolutions are clearly desirable in telepresence systems.  As the example in H.TPS-AV 7.1.2.2.1 suggests, in many telepresence room designs, 1080p offers less resolution than the resolving power of the human eye.  4K or greater resolutions will often match or exceed the eye’s resolving power.

ITU-R standardized parameter values for ultra-high definition television systems in August 2012 [ITU-R BT.2020].  These parameters apply to both 4K (3940x2160 pixels) and 8K (7680x4320 pixels) resolutions.  Tables 3 and 4 recommend a new colour gamut for ultra-high definition broadcast systems, which is “wider” than the colour gamut specified in ITU-R BT.709.

Note also that the recommended coding format uses 10 or 12 bits per component (exceeding the 8 bit format generally used today).

It is proposed that the Video Colour Gamut parameter be revised to include a new value of BT.2020, and that a references to BT.601, BT.1361, and BT.2020 be added to section 2.

Polycom also observes that the pixel bit depth is presently not included in the set of video parameters.  We note that ITU-T H.264 defines separate bit depths for luma (bit_depth_luma_minus8) and chroma (bit_depth_chroma_minus8).

We therefore propose that two parameters be added to H.TPS-AV – Video Luma Bit Depth, and Video Chroma Bit Depth. The possible values are proposed as 8-16 bits (which is consistent with both H.264 and H.265), with a default value of 8.  We also include definitions of luma and chroma for section 3 that are slightly modified from the definitions used in ITU-T Rec. H.264.

We believe that these parameters are needed both to assure the best interoperability and to ensure the best user experience.

# Conclusion

Polycom proposes that the following changes to H.TPS-AV.

Attachment 1

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	ITU-T Recommendation X.yyy (date), Definitions, requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	ITU-T Recommendation X.yyy (date), Telepresence System Architecture.

[ITU-R BT.1361]ITU-R Recommendation BT.1361 (02/1998), Worldwide unified colorimetry and related characteristics of future television and imaging systems.

[ITU-R BT.2020]ITU-R Recommendation BT.2020 (08/2012), Parameter values for ultra-high definition television systems for production and international programme exchange.

[ITU-R BT.601]	ITU-R Recommendation BT.601-7 (03/2011), Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios.

[ITU-R BT.709]	ITU-R Recommendation BT.709-5 (04/2002), Parameter values for the HDTV standards for production and international programme exchange.

[ITU-T G.122]	ITU-T Recommendation G.122 (03/1993), Influence of national systems on stability and talker echo in international connections.

[ITU-T H.245]	ITU-T Recommendation H.245 (05/2011), Control protocol for multimedia communication.

[ITU-T H.264]	ITU-T Recommendation H.264 (01/2012), Advanced video coding for generic audiovisual services.

[ITU-T H.323]	ITU-T Recommendation H.323 (12/2009), Packet-based multimedia communications systems.

[ITU-T P.56]	ITU-T Recommendation P.56 (12/2011), Objective measurement of active speech level.

[ITU-T P.300]	ITU-T Recommendation P.300 (11/2001), Transmission performance of group audio terminals (GATs).

[ITU-T P.501]	ITU-T Recommendation P.501 (01/2012), Test signals for use in telephonometry.

[ITU-T Q.931]	ITU-T Recommendation Q.931 (05/1998), ISDN user-network interface layer 3 specification for basic call control.

[IETF CLUE FW]		IETF draft-ietf-clue-framework-08, Framework for Telepresence Multi-Streams.

[IETF RFC 4796]		IETF RFC 4796 (2007), The Session Description Protocol (SDP) Content Attribute.

[ISO 11664-2]		ISO 11664-2 (2007) / CIE S 014-2/E (2006), Colorimetry - Part 2: CIE standard illuminants.

[ISO 140-4]		ISO 140-4 (1998), Acoustics - Measurement of sound insulation in buildings and of building elements - Part 4: Field measurements of airborne sound insulation between rooms.

[ISO 717-1]		ISO 717-1 (1996), Acoustics – Rating of sound insulation in buildings and of building elements – Part 1: Airborne Sound Insulation.

[ISO 8995]		ISO 8995 (2002), Lighting of indoor work places.

[P.supp 16]		ITU-T Recommendation Series P Supplement 16 (11/1988), Guidelines for placement of microphones and loudspeakers in telephone conference rooms and for group audio terminals (GATs).

# 3	Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	camera-left and right [IETF CLUE FW]: For media captures, camera-left and camera- right are from the point of view of a person observing the rendered media.  They are the opposite of stage-left and stage-right.

## Terms defined in this Recommendation

{Editor's note: The definitions of consumer and provider are consistent with CLUE but are placeholders}

This Recommendation defines the following terms:

3.2.1	capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal. A capability may also relate to how a terminal captures or renders signal.

3.2.2	chroma: An adjective specifying that a sample in a digital picture is representing one of the two colour difference signals related to the primary colours.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear

light transfer characteristics that is often associated with the term chrominance.

3.2.3	consumer: A device that receives media streams.

3.2.4	cycles per degree: The number of alternating white and black lines that can be captured or rendered per degree of visual field.

3.2.5	luma: An adjective specifying that a sample in a digital picture is representing the monochrome signal related to the primary colours.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear

light transfer characteristics that is often associated with the term luminance.

3.2.6	provider: A device that sends media streams.

{Editor's note: In defining parameters related to capabilities it is important to understand in the general sense what a capability may be. It is expected that this definition will evolve. Further definitions may be required.}

#### 7.1.2.1  Video Colour Gamut

| Identity: | ColourGamut |
| --- | --- |
| Description: | This parameter indicates the Colour Gamut used in a Telepresence Video Stream. |
| Format: | String |
| Possible Values: | BT.709, BT.601, BT.1361, BT.2020 |
| Default: | BT.709. |
| Reference: | - |
| Interface: | - |
| Supplementary Information: | In the absence of visual usability information (VUI) signalling (or if the VUI signalling indicates "unspecified"), telepresence systems shall process all received video signals per [ITU-R BT.709]. For example: Annex E.2 / [ITU-T H.264] regarding VUI has the "colour primaries" element that allows the signalling of the chromaticity and white point. If the transmitted video is not conformant with [ITU-R BT.709], senders should always signal the black level and range of the luma and chroma signals, transfer characteristics, the colour primaries, and the matrix coefficients in their video bit streams. Receivers may use this information to optimally render the received pictures. See clause 7.3.1.4 for more information on the use of colour and lighting parameters. |

#### 7.1.2.2  Video LumaBitDepth

| Identity: | LumaBitDepth |
| --- | --- |
| Description: | This parameter indicates the bit depth of the luma samples in a digital picture |
| Format: | Integer |
| Possible Values: | 8 through 14 |
| Default: | 8 |
| Reference: | - |
| Interface: | - |

#### 7.1.2.3  Video ChromaBitDepth

| Identity: | ChromaBitDepth |
| --- | --- |
| Description: | This parameter indicates the bit depth of the chroma samples in a digital picture |
| Format: | Integer |
| Possible Values: | 8 through 14 |
| Default: | 8 |
| Reference: | - |
| Interface: | - |

__________________