| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4454 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | H.225.0 Amd. Proposed changes for language cap exchange |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This proposal is to augment the definition of the language field in the RRQ, ACF, LRQ, LCF, SETUP, CONNECT messages to support the ability to compare language support of users to enable language based call routing.

# Introduction

There is a requirement for interpretation services to be able to detect and compare language support of the users in a call. For instance, when placing a call from a deaf person to a hearing person a sign interpreter would be required, however if the hearing person can also do sign language then the call should be allowed to progress point to point. Another example is where the user only speaks Spanish and requires a language interpreter when calling an English service. Another is where the call is received into a call centre and needs to be intelligently routed to a Spanish speaking person.

# Discussion

The current wording of the language field only defines the language to receive prompts and announcements and nothing directly about the language the caller supports. Although it can be referred that the user speaks or understands that language, it does not specify that in the definition.

The LCF message does not contain a language field so it cannot be determined what language the remote supports. This limits intelligent local routing rules based on language. Having a language field in the LCF allows the local gatekeeper to route the call based on the language subscription service the call originator may be signed up for.  For instance the caller may prefer to use company X for language interpretation services and/or company Y for deaf interpretation services, depending on the language the remote supports.

# Conclusion

It is proposed to make small changes to the description of the language field to encompass the language supported by the user and to include missing (where applicable) references to RFC 1766.

Add optional language field to LocationConfirm (LCF) message.

# Proposed Changes. H.225.0

_________________________________________

### 7.3.3	Connect

This message shall be sent by the called entity to the calling entity (gatekeeper, gateway, or calling terminal) to indicate acceptance of the call by the called entity. Follow Table 3-4/Q.931, as modified in Table 9 below.

…

language – Indicates the language(s) in which the user will use or would prefer to receive announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

### 7.3.10	Setup

This message shall be sent by a calling H.323 entity to indicate its desire to set up a connection to the called entity.

…

language – Indicates the language(s) of the call or in which the user would prefer to receive announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

### 7.9.1	RegistrationRequest (RRQ)

The RRQ message includes the following:

…

language – Indicates the language(s) in which the user supports or would prefer to receive announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

### 7.13.1	LocationRequest (LRQ)

The LRQ message includes the following:

…

language – Indicates the language(s) in which the user supports or would prefer to receive announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

### 7.11.2	AdmissionConfirm (ACF)

The ACF message includes the following:

…

language  Indicates the language(s)  of the call or to receive announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

### 7.13.2	LocationConfirm (LCF)

The LCF message includes the following:

requestSeqNum – This shall be the same value that was passed in the LRQ.

…

bandWidth – The allowed maximum bandwidth for the call; may be less than that requested.

language – Indicates the language(s) supported for calling or for announcements and prompts. The field contains one or more RFC 1766 compliant language tags.

_________________________________________

LocationConfirm ::= SEQUENCE --(LCF)

{

requestSeqNum				RequestSeqNum,

callSignalAddress			TransportAddress,

rasAddress				TransportAddress,

nonStandardData			NonStandardParameter OPTIONAL,

...,

destinationInfo			SEQUENCE OF AliasAddress OPTIONAL,

destExtraCallInfo			SEQUENCE OF AliasAddress OPTIONAL,

destinationType			EndpointType OPTIONAL,

remoteExtensionAddress		SEQUENCE OF AliasAddress OPTIONAL,

alternateEndpoints			SEQUENCE OF Endpoint OPTIONAL,

tokens					SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens				SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue		ICV OPTIONAL,

alternateTransportAddresses	AlternateTransportAddresses OPTIONAL,

supportedProtocols			SEQUENCE OF SupportedProtocols OPTIONAL,

multipleCalls				BOOLEAN OPTIONAL,

featureSet				FeatureSet OPTIONAL,

genericData				SEQUENCE OF GenericData OPTIONAL,

circuitInfo				CircuitInfo OPTIONAL,

serviceControl				SEQUENCE OF ServiceControlSession OPTIONAL,

modifiedSrcInfo			SEQUENCE OF AliasAddress OPTIONAL,

bandWidth					BandWidth OPTIONAL,

language					SEQUENCE OF IA5String(SIZE (1..32)) OPTIONAL

}

__________________