| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4454(Amd1) |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Spranto Australia Pty Ltd |  |  |  |
| Title: | H.225.0 Amd. Proposed changes for language cap exchange |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This proposal is to augment the definition of the language field in the RRQ, RCF, ACF, LRQ, LCF, SETUP, CONNECT messages to support the ability to compare language support of users to enable language based call routing.

# Introduction

There is a requirement for interpretation services to be able to detect and compare language support of the users in a call. For instance, when placing a call from a deaf person to a hearing person a sign interpreter would be required, however if the hearing person can also do sign language then the call should be allowed to progress point to point. Another example is where the user only speaks Spanish and requires a language interpreter when calling an English service. Another is where the call is received into a call centre and needs to be intelligently routed to a Spanish speaking person.

# Discussion

The current wording of the language field only defines the language to receive prompts and announcements and nothing directly about the language the caller supports. Although it can be referred that the user speaks or understands that language, it does not specify that in the definition.

The ACF message does not contain a language field so language cannot be assigned at registration from the gatekeeper. Having the ability to assign language tags to endpoints is important in environments where the user is unable to set the language tags themselves.

The LCF message does not contain a language field so it cannot be determined what language the remote supports. This limits intelligent local routing rules based on language. Having a language field in the LCF allows the local gatekeeper to route the call based on the language subscription service the call originator may be signed up for.  For instance the caller may prefer to use company X for language interpretation services and/or company Y for deaf interpretation services, depending on the language the remote supports.

# Conclusion

It is proposed to make small changes to the description of the language field to encompass the language supported by the user and to include missing (where applicable) and update references from RFC 1766 to the newer RFC5646.

Add optional language field to RegistrationConfirm (RCF) message.

Add optional language field to LocationConfirm (LCF) message.

# Proposed Changes. H.225.0

_________________________________________

### 7.3.3	Connect

This message shall be sent by the called entity to the calling entity (gatekeeper, gateway, or calling terminal) to indicate acceptance of the call by the called entity. Follow Table 3-4/Q.931, as modified in Table 9 below.

…

language – Indicates the language(s) in which the user will use or would prefer to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

### 7.3.10	Setup

This message shall be sent by a calling H.323 entity to indicate its desire to set up a connection to the called entity.

…

language – Indicates the language(s) of the call or in which the user would prefer to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

### 7.9.1	RegistrationRequest (RRQ)

The RRQ message includes the following:

…

language – Indicates the language(s) in which the user supports or would prefer to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

________________________________________

### 7.9.2	RegistrationConfirm (RCF)

The RCF message includes the following:

…

transportQOS – The Gatekeeper may use this field to indicate the resource reservation mechanism to be used by the endpoint.

language – Indicates the language(s) in which the user supports or would prefer to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

### 7.13.1	LocationRequest (LRQ)

The LRQ message includes the following:

…

language – Indicates the language(s) in which the user supports or would prefer to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

### 7.11.2	AdmissionConfirm (ACF)

The ACF message includes the following:

…

language  Indicates the language(s)  of the call or to receive announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

### 7.13.2	LocationConfirm (LCF)

The LCF message includes the following:

requestSeqNum – This shall be the same value that was passed in the LRQ.

…

bandWidth – The allowed maximum bandwidth for the call; may be less than that requested.

language – Indicates the language(s) supported for calling or for announcements and prompts. The field contains one or more RFC 5646 compliant language tags.

_________________________________________

Annex H

H.225.0 message syntax (ASN.1)

…

RegistrationConfirm ::= SEQUENCE --(RCF)

{

requestSeqNum						RequestSeqNum,

protocolIdentifier					ProtocolIdentifier,

nonStandardData					NonStandardParameter OPTIONAL,

callSignalAddress					SEQUENCE OF TransportAddress,

terminalAlias						SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier				GatekeeperIdentifier  OPTIONAL,

endpointIdentifier					EndpointIdentifier,

...,

alternateGatekeeper				SEQUENCE OF AlternateGK OPTIONAL,

timeToLive						TimeToLive OPTIONAL,

tokens							SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens						SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue				ICV OPTIONAL,

willRespondToIRR					BOOLEAN,

preGrantedARQ						SEQUENCE

{

makeCall						BOOLEAN,

useGKCallSignalAddressToMakeCall	BOOLEAN,

answerCall					BOOLEAN,

useGKCallSignalAddressToAnswer	BOOLEAN,

...,

irrFrequencyInCall		INTEGER (1..65535) OPTIONAL,	-- in seconds;

-- not present

-- if GK does

-- not want IRRs

totalBandwidthRestriction	BandWidth OPTIONAL,	-- total limit

-- for all

-- concurrent

-- calls

alternateTransportAddresses		AlternateTransportAddresses OPTIONAL,

useSpecifiedTransport			UseSpecifiedTransport OPTIONAL

} OPTIONAL,

maintainConnection				BOOLEAN,

serviceControl					SEQUENCE OF ServiceControlSession OPTIONAL,

supportsAdditiveRegistration		NULL OPTIONAL,

terminalAliasPattern			SEQUENCE OF AddressPattern OPTIONAL,

supportedPrefixes				SEQUENCE OF SupportedPrefix OPTIONAL,

usageSpec						SEQUENCE OF RasUsageSpecification OPTIONAL,

featureServerAlias				AliasAddress OPTIONAL,

capacityReportingSpec			CapacityReportingSpecification OPTIONAL,

featureSet					FeatureSet OPTIONAL,

genericData					SEQUENCE OF GenericData OPTIONAL,

assignedGatekeeper				AlternateGK OPTIONAL,

rehomingModel					RehomingModel OPTIONAL,

transportQOS					TransportQOS OPTIONAL,

language					   SEQUENCE OF IA5String(SIZE (1..32)) OPTIONAL

}

…

LocationConfirm ::= SEQUENCE --(LCF)

{

requestSeqNum				RequestSeqNum,

callSignalAddress			TransportAddress,

rasAddress				TransportAddress,

nonStandardData			NonStandardParameter OPTIONAL,

...,

destinationInfo			SEQUENCE OF AliasAddress OPTIONAL,

destExtraCallInfo			SEQUENCE OF AliasAddress OPTIONAL,

destinationType			EndpointType OPTIONAL,

remoteExtensionAddress		SEQUENCE OF AliasAddress OPTIONAL,

alternateEndpoints			SEQUENCE OF Endpoint OPTIONAL,

tokens					SEQUENCE OF ClearToken OPTIONAL,

cryptoTokens				SEQUENCE OF CryptoH323Token OPTIONAL,

integrityCheckValue		ICV OPTIONAL,

alternateTransportAddresses	AlternateTransportAddresses OPTIONAL,

supportedProtocols			SEQUENCE OF SupportedProtocols OPTIONAL,

multipleCalls				BOOLEAN OPTIONAL,

featureSet				FeatureSet OPTIONAL,

genericData				SEQUENCE OF GenericData OPTIONAL,

circuitInfo				CircuitInfo OPTIONAL,

serviceControl				SEQUENCE OF ServiceControlSession OPTIONAL,

modifiedSrcInfo			SEQUENCE OF AliasAddress OPTIONAL,

bandWidth					BandWidth OPTIONAL,

language					SEQUENCE OF IA5String(SIZE (1..32)) OPTIONAL

}

__________________