| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4456 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-AV: Additional render related parameters for general spatial information |  |  |  |
| Purpose: | Proposal |  |  |  |

- Summary

This contribution proposes to add some render related parameters to ITU-T draft H.TPS-AV.

- Introduction

Since the capture related parameters have already been cleared up and almost been done we’d better add some render related parameters which describe the parameters related to the render of audio and media into the render part section 7.2 in order to complete the AV recommendation.

- Discussion

We want to add the ‘General parameters’ into the render related parameters. We classify these general parameters into three types including scene related parameters, general spatial information and general encoding information.

- Conclusion

This contribution focuses on the general parameters. These general parameters mainly describe the general spatial information, such as scene area, area scale.

It’s proposed to add the text below into the clause 7.2 in H.TPS-AV.

[Begin Proposal]

    - 7.2.1 General Parameters

      - 7.2.1.4 General Spatial Information

The parameters in this clause relate to all types of media renders. It is assumed that all the spatial information parameters share a common origin.

        - 7.2.1.4.1 Scene Area

| Identity: | SceneArea |
| --- | --- |
| Description: | This parameter indicates the area of the overall scene related to the endpoint. |
| Format: | List of Strings |
| Possible Values: | 4 list elements each with (X, Y, Z) Where X, Y, Z are numbers. |
| Default: | Z=0 |
| Reference: | See 7.1.1.4.1 SceneArea for further information. |
| Interface: | - |
| Supplementary Information: | This parameter helps the provider to know the spatial arrangement of the render displays and the loud speakers. It also allows a provider to determine from what area in an overall scene a render will be displayed. For example, when the system wants the provider endpoint to do some image processing this parameter will help. Like cropping the original image into the suitable size or composing the several original image into one image according to this parameter and also the individual render area described in the render related visual parameters in another draft. This render related SceneArea parameter and the individual render area can also let the provider endpoint marks the capture streams’ spatial sign according to the consumer’s render request. |

        - 7.2.1.4.2 Area Scale

| Identity: | AreaScale |
| --- | --- |
| Description: | This parameter indicates the type of scale that is being applied to the spatial information parameters |
| Format: | Enumeration |
| Possible Values: | "millimeters" The metric system with the millimetre unit is being applied. "unknown" An unknown scale is being used that is the same for all renders in the render scene entry. This may be used to indicate relative distances. "No scale" This indicates that the scale may be different for each render in a render scene entry. |
| Default: | - |
| Reference: | See 7.1.1.4.2 Area Scale for further information. |
| Interface: | - |
| Supplementary Information: | The scale attribute is an attribute that specifies the scale of the coordinates provided in the render scene area and in the spatial information of the media render belonging to the considered render scene area. |

[End Proposal]

_________________