| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4457 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-AV: Additional render related parameters for general encoding information |  |  |  |
| Purpose: | Proposal |  |  |  |

- Summary

This contribution proposes to add some render related parameters to ITU-T draft H.TPS-AV.

- Introduction

Since the capture related parameters have already been cleared up and almost been done we’d better add some render related parameters which describe the parameters related to the render of audio and media into the render part section 7.2 in order to complete the AV recommendation.

- Discussion

We want to add the ‘General parameters’ into the render related parameters. We classify these general parameters into three types including scene related parameters, general spatial information and general encoding information.

- Conclusion

This contribution focuses on the general parameters. These general parameters mainly describe the general encoding information, such as the overall maximum bandwidth, overall maximum pixel per second.

It’s proposed to add the text below into the clause 7.2 in H.TPS-AV..

[Begin Proposal]

- 7.2	Render related Parameters

    - 7.2.1 General Parameters

      - 7.2.1.5 General Encoding Information

        - 7.2.1.5.1 Overall maximum bandwidth

| Identity: | MaxGroupBandwidth |
| --- | --- |
| Description: | This parameter indicates the maximum number of bits per second relating to all decoded media of a particular type (audio or video) received by the render endpoint. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See 7.1.1.5.1 Overall maximum bandwidth for further information. |
| Interface: | - |
| Supplementary Information: | This parameter indicates the max number of bits per second that the render endpoint can process. |

        - 7.2.1.5.2 Overall maximum pixel per second

| Identity: | MaxGroupPps |
| --- | --- |
| Description: | This parameter indicates the maximum number of pixels to be processed per second. |
| Format: | Numeric |
| Possible Values: | 0 upwards. |
| Default: | None. |
| Reference: | See " maxGroupPps " in [IETF CLUE FW] for further information. |
| Interface: | - |
| Supplementary Information: | This parameters indicates the max pixel per second that the render endpoint can process. |

[End Proposal]

_________________