| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-26 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor F.TPS-Reqs |  |  |  |
| Title: | F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence Systems” (New): Output Draft |  |  |  |
| Purpose: | Proposal |  |  |  |

Introduction

This document contains the Editor's new output draft (F.TPS-Reqs Ed. 1.3) for ITU-T F.TPS-Reqs “Definitions, requirements, and use cases for Telepresence systems”.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| F.TPS-Reqs Ed. 0.1 | TD-45a – Initial baseline text. Output from Andover meeting July 2011. |
| F.TPS-Reqs Ed. 0.2 | TD 649/WP2 (2011-11) – Input text to the November/December ITU-T SG16 meeting. It is a copy of the output of the Boston meeting with changes marks accepted. |
| F.TPS-Reqs Ed. 0.3 | Output text from the November/December ITU-T SG16 meeting. It incorporates changes from contributions to F.TPS-Reqs: C.654,655,656,711,744,745,750,751,756,757 |
| F.TPS-Reqs Ed. 0.4 | Input text to the November/December ITU-T SG16 meeting. It is a copy of the output of the Geneva 2012-12 meeting with changes marks accepted. |
| F.TPS-Reqs Ed. 0.5 | Output text from the ITU-T WP2/SG16 Rapporteurs meeting (Feb 2012). It incorporates changes from contributions to F.TPS-Reqs: AVD-4203,AVD-4213 |
| F.TPS-Reqs Ed. 0.6 | Input text to SG16, Geneva, 30 April – 11 May 2012 meeting: TD 767/WP2 (2012-04) |
| F.TPS-Reqs Ed. 0.7 | Output text from SG16, Geneva, 30 April – 11 May 2012 meeting: TD 767R1/WP2 (2012-04) |
| F.TPS-Reqs Ed. 0.8 | Input text to the September, Brisbane ITU-T WP2/16 Rapporteurs meeting, |
| F.TPS-Reqs Ed. 0.9 | Output text from the ITU-T WP2/SG16 Rapporteurs meeting (Sep 2012). |
| F.TPS-Reqs Ed. 1.0 | Input text to SG16, Geneva, 14 – 25 Jan 2013 meeting. |
| F.TPS-Reqs Ed. 1.1 | Output text from SG16, Geneva, 14 – 25 Jan 2013 meeting. |
| F.TPS-Reqs Ed. 1.2 | Input text to the Oslo, Norway meeting June 2013. It is a copy of TD-30R1 from the Geneva, Switzerland January 2013 meeting with the change marks accepted. |
| F.TPS-Reqs Ed. 1.3 | Output text from the ITU-T WP1/SG16 Rapporteurs meeting (June 2013). |

Living List Items:

- User-xx: The telepresence system shall provide unified (standardized) user operating procedures for end-users to operate it, as for basic features (i.e. open/join and close/leave a multipoint conference).

{Editor’s note: This is not about user interface. Solicit contributions on what functions and operating procedures are needed.}

CONTENTS

1	Scope	5

2	References	6

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this Recommendation	6

4	Abbreviations and acronyms	7

5	Conventions	7

6	Scenarios and use cases	7

6.1	Use case 1: Symmetric point-to-point meeting	8

6.2	Use case 2: Asymmetric point-to-point meeting	8

6.3	Use case 3: Multipoint meeting	8

6.4	Use case 4: Presentation	9

6.5	Use case 5: Heterogeneous systems	9

6.6	Use case 6: Multipoint education usage	10

6.7	Use case 7: Multipoint multiview (Virtual space)	11

6.8	Use case 8: Legacy Interworking	11

6.9	Use case 9: Telemedical use case	12

7	User experience requirements	13

8	Control Functional requirements	15

8.1	Call control functions	15

8.2	Media control functions	15

8.3	Conference control functions	16

8.4	Collaboration functions	17

8.5	Other functions	17

9	Interoperability requirements	17

9.1	Interoperation with traditional systems	17

9.2	Interoperation between telepresence systems	17

10	Network requirements	18

11	Security requirements	18

Appendix I  Requirements mapping	18

I.1	User experience requirements	18

I.2	Call control functions	19

I.3	Media control functions	19

I.4	Conference control functions	20

I.5	Collaboration functions	20

I.6	Other functions	20

I.7	Interoperability requirements	21

I.8	Network requirements	21

I.9	Security requirements	21

Annex A  <Annex Title - placeholder>	22

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure 1 – Use case 1: Symmetric point to point meeting 8 Figure 2 – Use case 2: Asymmetric point to point meeting 8 Figure 3 – Use case 3: Multipoint meeting 9 Figure 4 – Use case 4: Presentation 9 Figure 5 – Use case 5: Heterogeneous systems 10 Figure 6 – Use case 6: Multipoint education usage 11 Figure 7 – Use case 7: Multipoint multiview (virtual space) 11 Figure 8 – Use case 8: Legacy Interworking 12 Figure 9 – Use case 9: Telemedical 12 Figure 10 – User experience: image continuity 14 Figure 11 – User experience: gap minimisation 14

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

AAP Summary

<To be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

Scope

This Recommendation provides definitions, use cases and functional requirements for telepresence systems. A telepresence system is a set of functions, devices and network elements which are able to capture, deliver, manage and render multiple high quality interactive audio and video signals in a Telepresence conference. An appropriate number of devices (e.g. cameras, screens, loudspeakers,  microphones, codecs, MCUs,  PCs) and environmental characteristics are used to establish Telepresence. Specific requirements include those for:

- Call control functions

- Media control functions

- Conference control functions

- Collaboration

- Inter-operability

- Network

- Security

The requirements in this Recommendation relate to functions that a Telepresence system is required to support. That is, a Telepresence system should be able to provide the functions in its design or by utilising existing mechanisms. Recommended requirements do not mandate the implementation nor the deployment of a particular function. For example: Whilst a telepresence system should support mechanisms for assured QoS, it may be deployed to a network where these are not needed.

Unless the requirement mentions a specific function or device the requirements listed in this Recommendation apply to a Telepresence system as a whole. The inclusion of a requirement does not necessarily mean that something new is needed for the support of Telepresence. For example whilst there may be a requirement to support QoS for Telepresence, ultimately existing QoS mechanisms may be utilised for this.

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.239]	ITU-T Recommendation H.239 (09/05), Role management and additional media channels for H.300-series terminals.

[ITU-T H.241]	ITU-T Recommendation H.241 (05/06), Extended video procedures and control signals for H.300-series terminals.

[ITU-T H.243]	ITU-T Recommendation H.243 (10/05), Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 1920 kbit/s.

[H/F.TPS-Arch]	ITU-T Recommendation [FH].yyy (date), Telepresence System Architecture.

[H.TPS-AV]	ITU-T Recommendation H.yyy (date), Audio/Video Parameters for Telepresence systems.

[IETF CLUE REQ]	IETF draft-ietf-clue-telepresence-requirements-02, Requirements for Telepresence Multi-Streams.

[IETF CLUE UCS]	IETF draft-ietf-clue-telepresence-use-cases-04, Use Cases for Telepresence Multi-streams.

Definitions

<Check in the ITU-T Terms and definitions database on the public website whether the term is already defined in another Recommendation. It may be more consistent to refer to such a definition rather than redefine it>

Terms defined elsewhere

<Normally terms defined elsewhere will simply refer to the defining document. In certain cases, it may be desirable to quote the definition to allow for a stand-alone document>

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	actual size: A rendered figure in a display is the same size as if the person is in the room.

3.2.2	gaze awareness: Awareness of gaze direction of persons by watching their eyes, head and body position. There is awareness of both direct gaze and averted gaze. Direct gaze is the perception of persons looking at each other directly and simultaneously.  Averted gaze is the perception that other persons are looking at someone/something else in the environment.

3.2.3	gaze direction: the vector positioned along the visual axis, pointing from the fovea of the looker through the centre of the pupil to the gazed-at spot.

3.2.4	eye contact: Technically equivalent to a mutual direct-gaze between two persons communicating.

NOTE - In general, eye contact means non-verbal psychological communication with gazing eye-to-eye between two persons.

3.2.5	natural response: Interaction between local and remote participants that is not impeded by the delay introduced by the telepresence system.

3.2.6	telepresence: An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between participants by optimizing a variety of attributes such as audio and video quality, eye contact, gaze awareness, body language, spatial audio, coordinated environments and natural image size.

3.2.7	telepresence-enabled audio endpoint: An audio-only endpoint that supports telepresence endpoint functions/signalling and delivers a strong sense of presence and realism through the use of technologies such as wider bandwidth speech and spatial audio (e.g., stereo telephony).

Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| 3D | Three Dimensional |
| --- | --- |
| IPv4, IPV6 | Internet Protocol Version 4, Version 6 |
| MCU | Multipoint Control Unit |
| NAT | Network Address Traversal |
| PC | Personal Computer |
| QoS | Quality of Service |

Conventions

In this Recommendation, the following conventions apply.

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor's implementation must provide the option and the feature can be optionally enabled by the network operator/service provider. Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

Scenarios and use cases

The use cases in this clause illustrate various telepresence configurations for the purposes of identification of requirements. These configurations apply to scenarios where telepresence is in use and do not include legacy systems (i.e. non-telepresence systems) unless specifically indicated in the use case. They do not imply the need for mandatory support of any particular configurations.

For simplicity, the use cases below assume that each camera and microphone produces a single capture, and that each display renders a single capture.  In actual implementations, the relationship between captures and transducers can be more complex. For example, a pair of microphones may be mixed to monophonic sound (creating one capture from two transducers).  Alternatively, the same microphone arrangement can produce three captures (left, right, and centre), creating three captures from the same two transducers.

Use case 1: Symmetric point-to-point meeting

In this case each of the two sites has an identical number of screens, with cameras having fixed fields of view, and one camera for each screen.  The number of screens or cameras is not relevant so long as both sites have the same number. The sound type is the same at each end. Figure 1 illustrates the use case.

Two-screen Telepresence endpoint

Two–screen Telepresence endpoint

IP Network

Figure 1 – Use case 1: Symmetric point to point meeting

For further information see clause 3.1 / [IETF CLUE UCS].

Use case 2: Asymmetric point-to-point meeting

In this case, each site has a different number of screens and cameras than the other site. The number of screens or cameras is not relevant so long as both sites have a different number. Figure 2 illustrates the use case.

For further information see clause 3.2 / [IETF CLUE UCS].

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Figure 2 – Use case 2: Asymmetric point to point meeting

Use case 3: Multipoint meeting

In this use case of a multipoint telepresence conference, there are more than two sites participating. Figure 3 illustrates the use case.

For further information see clause 3.3 / [IETF CLUE UCS].

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Three-screen

Telepresence endpoint

Figure 3 – Use case 3: Multipoint meeting

Use case 4: Presentation

This use case builds on the previous use cases that in addition to the video and audio streams showing the participants, additional streams are used for presentations. For example the additional stream may be for content sources such as: document cameras, electronic whiteboards, computer (e.g. a laptop computer) or instant messaging. Figure 4 illustrates the use case.

The presentation viewgraphs can be distributed to remote endpoints by either adding new steams or embedding (or switching) viewgraphs in an existing video stream.

For further information see clause 3.4 / [IETF CLUE UCS].

Figure 4 – Use case 4: Presentation

Use case 5: Heterogeneous systems

This use case illustrates the common meeting scenario for people to join the conference from a variety of environments, using different types of endpoint devices.  A multi-screen immersive telepresence conference may include someone on a PC-based video conferencing system, a telepresence-enabled audio endpoint, and someone on a handheld device.

This use case assumes that all of the involved endpoint devices are able to handle signalling related to Telepresence systems. The underlying technology does not determine whether a device is “legacy” or not, the deciding factor is whether it supports telepresence related protocols (or protocol extensions). Legacy systems that do not understand telepresence related protocols (or protocol extensions) are not in scope of this use case.  Figure 5 illustrates the use case.

For further information see clause 3.5 / [IETF CLUE UCS].

Two-screen

Telepresence endpoint

IP Network

Three-screen

Telepresence endpoint

Presentation capture

Presentation

screen

Presentation capture

Presentation

screen

Three-screen

Telepresence endpoint

Personal Computer

Mobile Phone

Figure 5 – Use case 5: Heterogeneous systems

Use case 6: Multipoint education usage

This use case deviates from other use cases in that multiple video streams are not used to create an immersive conferencing experience with panoramic views at all the site.  Instead the multiple streams are dynamically used to enable full participation of remote students in a university class.  In some instances the same video stream is displayed on multiple screens in the room, in other instances an available stream is not displayed at all. Figure 6 illustrates the use case.

For further information see clause 3.6 / [IETF CLUE UCS].

Figure 6 – Use case 6: Multipoint education usage

Use case 7: Multipoint multiview (Virtual space)

This use case describes a virtual space multipoint meeting with good eye contact and spatial layout of participants. Virtual space multipoint as defined here assumes endpoints with multiple cameras and displays.  Usually there are the same number of cameras and displays at a given endpoint.  A camera is positioned above each display.  A key aspect of virtual space multipoint is the details of how the cameras are aimed.  The cameras are each aimed on the same area of view of the participants at the site.  Thus each camera takes a picture of the same set of people but from a different angle. Figure 7 illustrates the use case.

For further information, see clause 3.7 / [IETF CLUE UCS].

NOTE - This use case is not for 3D stereoscopic view.

Figure 7 – Use case 7: Multipoint multiview (virtual space)

Use case 8: Legacy Interworking

This use case describes a case where a legacy videoconferencing endpoint (or other heterogeneous device such as voice only endpoint) is involved in the telepresence session. The legacy endpoint does not understand additions related to telepresence functionality and interworking is required to communicate between the telepresence and legacy endpoints. Figure 8 illustrates the use case.

This use case may also utilise other legacy equipment such as a legacy H.323 MCU in order to participate in a conference.

Figure 8 – Use case 8: Legacy Interworking

Use case 9: Telemedical use case

This use case describes a typical application of Telepresence systems for medical usage. Multiple presentation streams are used dynamically to satisfy the need for presenting multiple videos and still images, etc. simultaneously. An endpoint shall be able to both receive and send multiple presentations. In some instances multiple presentation screens are used to enhance visibility.

Figure 9 illustrates the use case.

For further information see clause 3.8 / [IETF CLUE UCS].

Surgery Room

IP Network

Education Site

Three-screen Telepresence

Presentation screens

Surgery camera

Medical data collection equipments

One-screen Telepresence

Presentation screen

Pathology data

Expert Site

Endoscopic monitor

Presentation screens

Three-screen Telepresence

Medical records

Figure 9 – Use case 9: Telemedical

User experience requirements

This section addresses user experience requirements of a telepresence system on an end to end system basis, primarily in terms of functional requirements, rather than detailed performance requirements.

{Editor’s Note: Further contributions are needed to describe these requirements more precisely, and to limit them to not require detailed performance metrics which may not be available in the near-term. Treat the following as examples for now.

User01: It is recommended that a telepresence system support actual size high-definition (includes 1080p, 1080i, etc.) imaging of videoconferencing participants at the remote end. The support of actual size is considered an advanced Telepresence feature.

{Editor’s Note: The supported HD modes need further consideration.}

User02: It is recommended that a telepresence system provide an experience where there is awareness of both direct gaze and averted gaze, with the remote participants’ video images being rendered within a tolerable angle in both horizontal and vertical axes from the gaze direction between the two communicating participants.

Note - The visual clues for determining the gaze direction may include the orientation of the looker’s eyes with respect to their head and the orientation of the looker’s head with respect to the environment. Some research suggests that tolerable angles are 3-5 degrees horizontal and up to 12 degrees upward and 8 degrees downward.

User03: It is recommended that a telepresence system support a means of locating the speaker in a remote meeting room so that local participants can readily identify the position of different remote speakers.

User04: It is required that a telepresence system enable synchronization of audio and video from the remote location. Synchronization includes “lip-sync” and the sequencing of multiple media streams.

User05: It is recommended that a telepresence system provide a control function to allow a user to perform an operation with a minimum of input thus enabling convenient centralized management and one-touch smart control capabilities.

User06: It is recommended that a telepresence system provide harmonized environment for assuring immersive presences of remote users. Typically in order to provide immersive and realistic experiences to users, some environmental factors are aligned between local and remote locations. These environmental factors for example include: interior design such as: the desk shape, wall colour, illumination, lighting position, etc.

User07: It is recommended that a telepresence system support multiple video sources in each meeting room. It is recommended that these video sources provide a seamless view of the conference participants.

User08: It is recommended that a telepresence system support multiple screens in each meeting room. It is recommended that the gap width between neighbouring displays be minimized. Furthermore it is recommended that the output images on neighbouring displays be continuous (across the frame borders (physical or virtual), if they exist).

Figure 10 illustrates the continuity requirement.

Figure 10 – User experience: image continuity

Figure 11 shows the effects of gap width on the display of images.

Figure 11 – User experience: gap minimisation

NOTE - In some cases the images from the received video captures overlap and in other cases they are separated. In all of these cases, the displays may not be able to provide a continuous view of the conference participants without processing the images (e.g., introducing a virtual border, shifting, scaling and cropping, or other methods). It may also not be possible to provide image continuity when physically different (in terms of their own characteristics and/or their spatial placement) screens are used to display the image.

User-09: It is required that a telepresence system provide natural response for facial expression and conversational voice.

User10: It is recommended that a  telepresence system allow the receiver to switch captures between displays and assign desirable display positions for them (either automatically or via user input).

User11: It is recommended that a telepresence system indicate detailed information associated with a composed capture including the list of originating sources and the policy of the composition.

Control Functional requirements

This clause specifies a number of functional requirements of the telepresence system in different categories of control functions including call control functions, media control functions, conference control functions, collaboration functions, and other functions.

Call control functions are those functions used to connect to a session of a telepresence system, disconnect the session, and request information about the session or endpoints participating in the session. Media control functions are those functions used to support different media streams including the description and negotiation of distinct capabilities of media, description method for the spatial arrangement of media streams, and addition/deletion of media streams. Conference control functions are those functions used to manage the display mode of screens, control cameras used in telepresence systems, and support addition/deletion of participants in the telepresence session, section switch, meeting room switch. Finally, Collaboration functions are those functions used to provide additional applications for collaborating with other endpoints.

{Editor’s Note: This is taken as the start text and needs to be refined later.}

Call control functions

Cal01: It is required that a  telepresence system support interoperability with other devices that send or receive a different number of media streams.

Cal02: It is required that a telepresence system support the description, identification and negotiation of appropriate information to enable spatially coordinated rendering of audio and video streams.

Media control functions

Med001: It is required that a telepresence system support the use of video with different aspect ratios between endpoints.

Med002: It is required that a  telepresence system support a method for describing the spatial arrangement of audio and video streams in a manner which enables a satisfactory reproduction at the receiver in a spatially correct manner. Furthermore it is required to support a method to enable a satisfactory spatial matching between audio and video streams coming from the same endpoints.

This requirement encompasses the requirements REQMT-1, REQMT-2 and REQMT-3 as defined in [IETF CLUE REQ].

Med002a: It is recommended that a  telepresence system support a method to indicate that the spatial information regarding a capture is dynamic (i.e. may change during the session) and/or a method of updating the spatial information.

Med002b: It is recommended that a telepresence system support the ability for endpoints to send and receive multiple presentation streams.

Med002c: It is recommended that a telepresence system support multiple simultaneous presentation streams from different endpoints.

Med003: It is required that a  telepresence system provide interoperability between endpoints which send and receive different numbers of media streams. This requirement encompasses requirement REQMT-9 as defined in [IETF CLUE REQ].

Med004: A telepresence system can optionally support 3D multimedia stream.

Med005: A telepresence system can optionally support a means for describing 3-Dimensional multimedia stream.

{Editor’s Note: Inclusion of 3D requirements depends on appropriate contributions to support them.}

Med006: It is recommended that a  telepresence system  be able to add additional media streams, and remove existing media streams during the telepresence session.

Med007: It is recommended that a  telepresence system provide a suitable range of video frame rates, resolutions, and bit rates for presenting the shared data For example those provided by ITU-T H.239, ITU-T H.241.

Med008: A non-mixing mode of operation for MCUs connecting telepresence endpoints is recommended.

Med009: It is recommended that a telepresence system advertise a set of media characteristics suitable for use by various types of devices.

Med010: It is recommended that a telepresence system limit its advertised set of media characteristics to ensure a suitable user experience for the application (for instance ensuring high quality video for telemedicine)

Conference control functions

{Editor’s Note:  These are examples for now, that will need further discussion.}

Con01: It is required that a  telepresence system enable switching  video from the all of the remote meeting rooms to a subset of  meeting rooms, or from partial images to another image.

Con02: It is required that a  telepresence system enable the display of all remote participants at once, including remote participants (if any) from non-telepresence systems.

Con03: It is required that a  telepresence system enable the conference chairman to select some or all media streams from a given meeting room for distribution to the telepresence session.

Con04: It is required that a  telepresence system support the addition or removal of an endpoint to/from an ongoing telepresence session; for example, such as the procedures described by ITU-T H.243.

Con05: It is recommended that a telepresence system support far-end camera control (FECC), providing means to specify the capture point of each movable camera.

Con06: It is recommended that a telepresence system support local user selection of which media captures are advertised.

Con07: It is recommended that a telepresence system support  local user selection of which media captures are consumed from the far end.

Con08: It is recommended that a telepresence system support a method to allow a conference chairman to view sites currently involved in the meeting and to add and delete one or more site from the meeting.

Con09: It is recommended that a telepresence system provide a list of participants and the current conference chair to all endpoints in the conference.

Con10: It is recommended that a telepresence system provide the conference name to all endpoints in the conference.

Collaboration functions

Col001: A telepresence system can optionally provide one or more means of collaboration such as remote presentation, instant messaging, online documentation, whiteboard and file transferring based on existing protocols.

Col002: A telepresence system can optionally support a captioning function.

Other functions

This sub-clause contains requirements that are not related to the other sub-clauses in clause 8.

Other01: It is required that a  telepresence system be able to provide various statistical information which includes but is not limited to media type, maximum number of endpoints, duration, and data volume of each media type.

{Editor’s note: we need more contributions on the precise list for reporting.}

Other02: It is recommended that a telepresence system provide QoS/QoE information including media network utilization, network delay, loss rate and jitter.

Interoperability requirements

Interoperation with traditional systems

This clause presents requirements for interoperation with legacy devices/systems. When used in a conference with Telepresence system these legacy devices may not perceive telepresence.

Int01: It is required that a  telepresence system support both traditional videoconferencing endpoints and telepresence endpoints can participate in a same conference.

Int02: It is recommended that a  telepresence system support that a traditional videoconferencing system can connect to the telepresence system in a cascaded conference mode.

Int03: It is recommended that a  telepresence system allow audio only legacy devices to participate in a conference.

Interoperation between telepresence systems

This clause presents requirements for interoperation with devices/systems that have been enhanced for telepresence.

Int04: It is required that a telepresence system enable audio and video terminals based on different open standards, different software and/or hardware architectures to participate in the same conference.

Int05: It is required that a telepresence system provide a telepresence experience between telepresence endpoints and devices based on different open standards participating in the same conference. For example: A telepresence enhanced SIP endpoint and a telepresence enhanced H.323 endpoint via control signalling gateway.

Int06: It is required that a telepresence system allow telepresence endpoints and devices residing in different networks to participate in the same conference. This includes cases of intra and inter operator networks, and also between an operator network and an enterprise network.

Int07: It is recommended that a telepresence systems support interoperability between endpoints with presentation screens of different resolutions.

Network requirements

Net001: It is recommended that a telepresence system enable reservation of  network resources for assuring QoS for the telepresence session.

Net002: It is recommended that a  telepresence system enable adaptation of the quality of media based on the network status. For example: Under best-effort network such as Internet which cannot provide strictly guaranteed network QoS, the telepresence system may need to manipulate its video quality based on network status.

Net003: It is recommended that a  telepresence system provide a means to traverse NATs or firewalls automatically to assure connectivity of telepresence services with minimal user operation.

{Editor’s Note:  This requirement needs more discussion.}

Net004: It is required that a telepresence system support both IPv4 and IPv6.

{Editor’s Note: This requirement needs more discussion. Use cases would be helpful.}

Security requirements

Sec001: A telepresence system can optionally support of security mechanism for securing sessions.

Sec002: A telepresence system can optionally support of negotiation mechanism for making a secured session.

Sec003: A telepresence system can optionally support both signal security and media security.

Sec004: It is required that a telepresence system be able to authenticate each end point.

Appendix I

Requirements mapping

(This appendix does not form an integral part of this Recommendation)

{Editor’s Note:  This appendix needs to be updated according to the progress of F/H.TPS-Arch and H.TPS-AV.}

- I.1	User experience requirements

| Requirement | Covered | Details |
| --- | --- | --- |
| User01 | Yes | Clause 7.1.2.2 H.TPS-AV discusses effective resolution and gives an example usage of 1080p. |
| User02 | Part | H.TPS-AV allows for spatial information to be signalled allowing video to be correctly rendered thus increasing chances of gaze awareness. |
| User03 | ? | A method could be: displaying the image, providing spatial audio or a text tag. There appears to be no discussion in other work items. |
| User04 | Yes | H.TPS-AV includes a parameter for AudioVideoSynchronization |
| User05 | ? | There is no further discussion of this requirement. F/H.TPS-Arch provides a management function. |
| User06 | Yes | H.TPS-AV provides a number of parameters related to the room environment. |
| User07 | Yes | H.TPS-AV has parameters that allow multiple audio and video captures to be described along with their spatial co-ordinates. |
| User08 | Yes | H.TPS-AV describes this in relation to the capture area parameter. |
| User09 | Yes | This is closely tied to User04. |
| User10 | Part | Being able to signal multiple captures utilising the parameters of H.TPS-AV allows this. However this user interface action is not discussed further. |
| User11 | Part | H.TPS-AV includes a Scene Switch Policy and an Auto Switched parameter. |

- I.2	Call control functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Cal01 | Yes | Being able to signal multiple captures utilising the parameters of H.TPS-AV allows this. In addition the media control unit functions describe by F/H.TPS-Arch provides a means to transcode and/or compose audio and video streams. |
| Cal02 | Yes | H.TPS-AV has parameters related to the spatial characteristics of captures. |

- I.3	Media control functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Med001 | ? |  |
| Med002 | Yes | H.TPS-AV has parameters that allow multiple audio and video captures to be described along with their spatial co-ordinates. |
| Med002a | ? |  |
| Med002b | ? |  |
| Med002c | Part | H.TPS-AV includes a presentation attribute which can be set for multiple captures. That may not be sufficient to fully cover this requirement. |
| Med003 | Part | Being able to signal multiple captures utilising the parameters of H.TPS-AV allows this. In addition the media control unit functions describe by F/H.TPS-Arch provides a means to transcode and/or compose audio and video streams. |
| Med004 | ? | There is no discussion of 3D coding elsewhere in the TPS documents. |
| Med005 | ? | There is no discussion of 3D coding elsewhere in the TPS documents. |
| Med006 | ? | There is doesn’t appear to be specific text elsewhere in the TPS documents that talks about in session media stream changes. |
| Med007 | Part | F/H.TPS-Arch discusses these regarding endpoint characteristics. H.TPS-AV provides some encoding parameters. |
| Med008 | ? | There is no discussion elsewhere in TPS documents regarding the benefits of this. It is assumed that it is supported by signalling. |
| Med009 | ? | There is no discussion in the TPS documents regarding this. |
| Med010 | ? | There is no discussion in the TPS documents regarding this. |

- I.4	Conference control functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Con01 | Part | H.TPS-AV has attributes for switching and allows multiple captures to be advertised. It would appear that switching between them would be a local endpoint function. There appears to be little discussion elsewhere. |
| Con02 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con03 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con04 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con05 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con06 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con07 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con08 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Con09 | ? | There is no discussion in the TPS documents regarding this. |
| Con10 | ? | There is no discussion in the TPS documents regarding this. |

- I.5	Collaboration functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Col001 | Part | F/H.TPS-Arch discusses presentation streams but does not go into detail regarding the different collaboration means. |
| Col002 | ? | There is no discussion in the TPS documents regarding this. |

- I.6	Other functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Other01 | ? | F/H.TPS-Arch contains a description of a management function. However it does not indicate that it could be used for collecting statistics. |
| Other02 | ? | There is no discussion in the TPS documents regarding this. |

- I.7	Interoperability requirements

| Requirement | Covered | Details |
| --- | --- | --- |
| Int01 | Yes | F/H.TPS-Arch discusses that legacy endpoints may be part of a telepresence system. |
| Int02 | Part | F/H.TPS-Arch sect. 6.1.2 discussed cascading MCUs. |
| Int03 | Part | F/H.TPS-Arch discusses that legacy endpoints may be part of a telepresence system although it doesn’t explicitly mention audio only devices. Figure 2 shows a mobile phone. |
| Int04 | Part | F/H.TPS-Arch introduces general endpoint architecture. There is no specific mention of a hardware/software implementation. |
| Int05 | Yes | F/H.TPS-Arch introduces a general architecture that may be applied to either SIP or H.323 devices. Both are mentioned. |
| Int06 | ? |  |
| Int07 | Part | F/H.TPS-Arch describes an MCU function of media adaptation however there is no specific reference to presentation streams. |

- I.8	Network requirements

| Requirement | Covered | Details |
| --- | --- | --- |
| Net001 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Net002 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Net003 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Net004 | ? | There is no discussion elsewhere in TPS documents regarding this. |

- I.9	Security requirements

| Requirement | Covered | Details |
| --- | --- | --- |
| Sec001 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Sec002 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Sec003 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Sec004 | Part | F/H.TPS-Arch has a description of MCU and Telepresence Control unit that mentions they have authentication functions. |

Annex A

<Annex Title - placeholder>

(This annex forms an integral part of this Recommendation)

<Body of annex A>

__________________