| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD XX (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 21/16 | Geneva, 28 Octorber-8 November 2013 |
| TD |  |  |
| Source: | Editors H.P2PVSArch |  |
| Title: | H.P2PVSArch "Architecture for point-to-point visual surveillance" (New) : Initial draft |  |

Introduction

This document contains the editors’ initial draft for H.P2PVSArch "Architecture for Point-to-point visual surveillance" (New) from the SG16, Geneva, 28 Octorber – 8 November 2013 meeting.

CONTENTS

Page

1 Scope 3 2 References 3 3 Definitions 4 3.1 Terms defined elsewhere 4 3.2 Terms defined in this Recommendation 4 4 Abbreviations 4 5 Conventions 4 6 Functional Requirements 4 6.1 Service Requirements 4 6.2 Platform Requirements 5 6.3 CU/MCU requirements 6 7 Functional Architecture 6 7.1 CMU: centre management unit 7 7.2 SCU: service control unit 8 7.3 MDU: media distribution unit 8 7.4 PU: premises unit 8 7.5 NVR: network video recorder 9 7.6 CU: customer unit 9 7.7 M_CU: Mobile customer unit 9 7.8 CS: cloud storage platform 9 Bibliography 9

List of Tables

Page

No table of figures entries found.

List of Figures

Page

Figure7-1 System composition of P2P visual surveillance 7

Draft new Rec. ITU-T H.P2PVSArch

Architecture for point-to-point visual surveillance

AAP Summary

[To be provided before Consent]

Summary

TBD.

Keywords

Visual Surveillance, Architecture, Point-to-point

Scope

The Visual Surveillance service is a telecommunication service focusing on video (audio included also) application technology, which is used to capture remotely multimedia (such as audio, video, image and various alarm signal) and present them to the end users with friendly manner, based on managed broadband network with ensured quality, security and reliability ensured.

The P2P Visual surveillance is a new kind of video surveillance system, which provides a way for the users to access video stream directly to the P2P cameras through the internet, without transferring any video stream by the VS platform. And the P2P cameras may be IP cameras, 3G/4G Cameras, mobile phones with camera, etc.

The Recommendation H.P2PVSArch focuses on the functional requirements of a P2P visual surveillance system, including the functional requirements, model, architecture, entities, and reference points of it.

References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.743]	Recommendation ITU-T F.743 (2009), Requirements and service description for visual surveillance.

[ITU-T H.626]	Recommendation ITU-T H.626 (2011), Architectural requirements for visual surveillance.

[ITU-T H.627]	Recommendation ITU-T H.VSprot (2012), Signalling and Protocols for visual surveillance.

[ITU-T H.VSMarch]	Recommendation ITU-T H.VSMarch (2011), Architecture for mobile visual surveillance.

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP).

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol - HTTP/1.1.

[IETF RFC 3261]	IETF RFC 3261 (2002), SIP: Session Initiation Protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

Definitions

Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

Abbreviations

| TBD |  |
| --- | --- |

Conventions

Functional Requirements

Service Requirements

6.1.1 Self-service of users

The user can register the service using the mobile phone number as an account, and assign the specified cameras under the account by oneself, after the service is approved in business office of telecommunication operator.

6.1.2 Plug and Play

The user can connect to cameras through internet directly, without any extra configurations for the equipment, achieving the goal of zero configurations.

6.1.3 Diversified custom terminal devices

Varieties of terminal devices are supported, including mobile phone, pad and personal computer. The user can watch the real-time video at anytime and anywhere, and multiple choices of video formats are provided, CIF, D1 and 720P for instance. The frame rate also varies from 1 to 25 fps.

6.1.4 PTZ Operation (Optional)

The user can tilt the camera and zoom it in and out remotely over CU/M_CU.

6.1.5 Night Vision (Optional)

The built-in infrared lamp can increase the lightness and then enhance the video resolution under low illumination environments, especially for the dark night scenario.

6.1.6 Video Record and Playback on SD Card (Optional)

Supporting video storage, searching and playback on SD card. The record strategy is also configurable remotely.

6.1.7 Video Record and Playback on NVR

Supporting video record on NVR which is binding to specified IPC, supporting the record strategy setting, recorded video playback on the platform client. The video files can be exported from the NVR, can be played quickly, slowly and reversely, even at the speed of single frame per second.

6.1.8 Video intercom

Users can talk to each other, between the CU/M_CU and IPC.

6.1.9 Alarm messaging

Supporting to send alarm messages to users’ mobile phones, relating to events such as login in/out, full storage and IPC off-line, etc.  The content of messages can be set over CU.

6.1.10 Video sharing

Supporting major account to share its own surveillance sites to its sub account.

6.1.11 Unified wifi configuration

Users can set the wifi over the unified wifi configuration interface in CU/MCU conveniently, or switch to another wifi.

6.1.12 Fixed timing snapshot

IPC is able to snapshot images and send them to the specified phone according to given fixed timing that users set by themselves.

6.1.13 Cloud storage

It is provided to transfer and store the videos in the remote cloud, and supporting recorded video search, playback. Users will pay for extra cloud storage, given that some fixed storage space is free.

6.1.14 Network Management

It is provided to manage the IPCs, storage devices and network failures.

Platform Requirements

- A Portal is provided for users to access to the services such as self-service, video watch, recorded video search and playback, alarming and search, IPC configuration. The portal is also of functions of advertising and IPC booking.

- A Portal is provided for administrators from telecommunication operator, to access the functions such as equipment management, user management, vendor management，system configuration，device remote upgrade, log management, statistics and reporting.

- It is provided for the MCU to access to Platform.

- It is provided to interwork with cloud storage system.

- It is provided to interface with SMS gateway.

- It is provided to interwork with CRM for user authentication and verification.

CU/MCU requirements

6.3.1 MCU

- Supporting users to login in.

- Supporting to display camera status of online or offline.

- Supporting the real-time video watch  based on RTSP，RTP/RTC over TCP and H.264.

- Supporting PTZ operation.

- Supporting instant image snapshot over MCU.

- Supporting local record of video.

- Supporting audio and video simultaneously.

- Supporting remote setting of wifi to which IPC connect.

6.3.2 Web client

- Supporting the video based on H.264 and audio based on G.711 and G.722.1.

- Supporting users to login in.

- Network controls must be of digital signature, it can be upgraded to latest version automatically.

- PTZ operation.

- Users can alter the key for login, set record strategy, save the storage paths.

- Supporting the image snapshot over real-time video manually, the save path is configurable.

- Supporting both local and front-end record of video.

- Supporting remote configuration of IPC WIFI.

- Supporting cloud storage for video, and the recorded video can be searched and playback remotely.

- Record setting strategy:

  - Starting or stopping the local recording process manually and the video files are stored in the local PC, the save paths are modifiable;

  - Front-end and cloud storages includes periodic and cyclic record;

- Supporting a series of windows Operation System, such as Windows XP, Windows 2003, Windows Vista, Windows 7 and Windows 8.

- Supporting mainstream browsers based on IE, Firefox and Chrome.

Functional Architecture

TBD.

Figure7-1 System composition of P2P visual surveillance

The system architecture is presented in Fig. 7-1, includes: Pre-Unit (PU), Client Unit (CU), Mobile Client Unit (M_CU), Cloud Storage System、Network Video Recorder (NVR) and centre service platform；Central service platform is composed of Centre Management Unit (CMU), Service Control Unit (SCU), Media Distribution Unit (MDU). All components are connected via IP bearer network.

  - CMU: centre management unit

The centre management unit (CMU) is used for centralized system management, service operation management, etc.

Accordingly, CMU main functions include management, authorization, accounting and charging, location and presence. The detailed functional features are:

- Management: managing content, service, network, terminal devices, etc.

- Authorization: authorizing access right.

- Accounting and charging: providing CDR (calling detail records) and charging policy, etc.

- Location and presence: locating the PU、NVR, and Presenting PU and NVR on-line/offline status.

  - SCU: service control unit

The service control unit (SCU) is distributed network equipment，fulfilling the access of PU, NVR, CU, MCU.

Accordingly, SCU main functions include access registration, access authentication, identification, authorization, call control, location, presence, and target media serving function selection. The detailed functional features are:

- Access registration: receiving the access registration request from the CU and the PU, and registering them with the registration management server.

- Access authentication: receiving the access authentication request from the CU and the PU, and as an authentication server function, forward (proxy) authentication signals between AAA server.

- Identification: identifying the CU and the PU, and comparing identities with subscriber data and terminals data to determine matching results.

- Authorization: authorize CU and PU access rights.

- Call control: mediate call control session with the PU and the CU, establish and maintain media transport path information (media routing table, i.e. MRT). The related session methods could include setup, tear down, update, modify and keep-live in order to access, route and relay.

- Presence: presenting the status of the CU\M_CU and the PU\NVR, such as on-line, offline, active, inactive, etc.

- Clock synchronization: unified time criteria provision and clock synchronization control.

- Media serving function selection: selecting one media serving function unit (i.e. media distribution unit and related media storage unit) when one SCU controls multiple media serving function units.

  - MDU: media distribution unit

The media distribution unit (MDU) is used to transport media from the PU to the CU\M_C, when the P2P communication channel fails.

Accordingly, MDU main functions include media receiving, media processing, media routing, media transmission, media forwarding, media replication, etc. The detailed functional features are:

- Served as STUN, TURN server.

- Media receiving: receiving media (stream and files) from the PU.

- Media processing: muxing/demuxing, decoding/encoding and transcoding media that is received by the MDU, as required.

- Media transmission: transmitting media to a CU or multiple CUs on request according to media routing information (Note –Transmission from one PU to multiple CUs requires conversion from a unicast media channel to a multicast media channel).

- Media replication: replicating media for point-to-multipoint transmission, if required.

  - PU: premises unit

PU refers to the premise subsystem within the VS system. It implements the following functions:

- Capture multimedia information (such as audio, video, image, alarm signals, etc.) from the watch point.

- Multimedia (audio, video, image) encoding.

- Output alarm signal to external linkage device.

- Parse PTZ command codes and transmit them to the devices in order to control pan and camera.

- Network transport, transmit bidirectional media stream to other entities and alarm signals.

- The media storage function is optional for the PU.

- Audio decoding and play function is optional for the PU.

  - NVR: network video recorder

NVR is used to record the surveillance video in the front end. The video can be transferred to NVR, exported from NVR, searched and playback.

  - CU: customer unit

The CU is the customer unit within the VS system. It is used to present multimedia information (such as audio, video, image, and alarm signal) to the end user. It implements the following functions:

- Multimedia decoding function.

- Audio play and video/image display function.

- Console interface for end user to operate the VS system.

- Supporting access network configuration, including wifi hotspot selection and account setting.

  - M_CU: Mobile customer unit

The functions of M_CU are similar with CU, different from that M_CU is based on mobile terminal.

  - CS: cloud storage platform

Cloud storage platform is the external system interworking with P2P visual surveillance system, which provides the storage service specific for the service. It receives the video from IPC and responds to CU and MCU requests.

Bibliography

[b-ITU-T H.Sup.1]	ITU-T H-series Recommendations – Supplement 1 (1999), Application profile – Sign language and lip-reading real-time conversation using low bit rate video communication.

_______________