| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 134 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 28 October – 8 November 2013 |
| TD |  |  |
| Source: | Editor H.Supp.Prio |  |
| Title: | Approval: H.Sup12 (ex H.Supp.Prio) "Priority traffic treatment by H.248 gateways" (New) |  |

This TD contains the draft new H.Sup12 (ex H.Supp.Prio) "Priority traffic treatment by H.248 gateways", which is proposed for Approval at this SG16 meeting.

Summary

This Supplement provides complementary information on the handling of priority traffic (e.g.., IEPS/ETS traffic) by the H.248 entities: Media Gateway Controller (MGC) and Media Gateway (MG).

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined Elsewhere 5 3.2 Terms defined in this Supplement 5 4 Abbreviations 6 5 Conventions 7 6 Priority traffic – Traffic model and profile specifications 7 6.1 Traffic model 7 6.2 Conventions for abstracted H.248 signalling specifications 8 6.3 Specification plane: Type of H.248 Profile 8 6.4 Specification plane: Codepoint space 12 6.5 Specification plane: Further comments 13 7 Summary 13 Appendix I H.248 profiles and Context Attributes for priority traffic handling 14 I.1 Status H.248 Profiles 14 I.2 Discussion of the principal H.248 signalling options 14 I.2.1 Overview 14 I.2.2 Bearer dependent H.248signalling 15 I.2.3 Bearer independent H.248 signalling 16 I.3 IEPS/ETS traffic support: signalling (stage 3) solutions 16 Appendix II Potential use cases with IEPS call indicator support 18 II.1 Overview 18 II.2 Use case #A: support of multiple, different IP QoS architectures 18 II.3 Use case #B: IP bearer connection traversing different IP QoS architectures 20 II.4 Use case #C: Priority treatment with more than 16 priority levels 20 II.5 Use case #D: Network monitoring of signalling traffic (here: H.248) 21 II.6 Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators 22 II.7 Use case #F: heterogeneous vendor landscape 22 II.8 Summary and conclusions 22 Appendix III Mapping function – Examples 23 III.1 Examples of MGC and MG located mapping functions 23 III.2 Complete view on the two mapping functions in series 24

List of Tables

Page

Table I.1 – 3GPP Profiles and typical support of Context Attributes (status: Rel-11) 14 Table II.1 – Overview of use cases 18

List of Figures

Page

Figure 1 – Example overall traffic model for priority services with scope on the MG entity 8 Figure 2 – Generic profile view – Processing rule concept (plus the three functional areas of possible MG actions) 9 Figure 3 – Generic profile view – Some issues 10 Figure 4 – Generic profile view – Issue due to isolated consideration 11 Figure 5 – Generic profile view – Comprehensive consideration 11 Figure 6 – Hierarchical versus flat priority scheme 12 Figure I.1 – Control of a MG-level QoS marking function – Variants 1 and 2 15 Figure I.2 – Controlling of MG-level QoS marking function – Variant 3 16 Figure II.1 – Use case #A: support of multiple, different IP QoS architectures 19 Figure II.2 – Use case #B: IP bearer connection traversing different IP QoS architectures 20 Figure II.3 – Use case #C: Priority treatment with more than 16 priority levels 21 Figure II.4 – Use case #D: Network monitoring of signalling traffic (here: H.248) 22 Figure III.1 – Example of a MGC-level mapping function (MFMGC) 23 Figure III.2 – Three of the variants as discussed in Appendix I.2 25

Draft new Supplement 12 to the ITU-T H-Series (ex H.Supp.Prio)

Gateway Control Protocol:
Priority traffic treatment by H.248 gateways

Summary

This Supplement provides complementary information on the handling of priority traffic (e.g.., IEPS/ETS traffic) by the H.248 entities: Media Gateway Controller (MGC) and Media Gateway (MG).

# 1	Scope

This Supplement provides complementary information concerning the handling of priority traffic by the H.248 entities: Media Gateway Controller (MGC) and Media Gateway (MG).

The scope covers the following aspects:

- the notion of "priority traffic" from the perspective of the gateway control interface H.248 (clause 6);

- a comprehensive traffic model for priority services (as introduced by [ITU-T H.248.81]) with focus on the MG (clause 6.1);

- the specification aspects of H.248 profiles with respect to "generic" and "practical" profiles (clause 6.2);

- a status snapshot of some H.248 profiles and Context Attribute support for priority traffic handling (Appendix I);

- example use cases for different gateway types supporting priority traffic (such as with possible IEPS call indicator support) (Appendix II).

# 2	References

[ITU-T E.107]	Recommendation ITU-T E.107 (02/2007), Emergency Telecommunications Service (ETS) and interconnection framework for national implementations of ETS.

[ITU-T I.130]	Recommendation ITU-T I.130 (11/1988), Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (09/2005), Gateway Control Protocol: Version 3.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway Control Protocol: QoS support packages.

[ITU-T H.248.54]	Recommendation ITU-T H.248.54 (08/2007), Gateway Control Protocol: MPLS support package.

[ITU-T H.248.56]	Recommendation ITU-T H.248.56 (08/2007), Gateway Control Protocol: Packages for virtual private network support.

[ITU-T H.248.79]	Recommendation ITU-T H.248.79 (02/2012), Gateway Control Protocol: Guidelines for Packet-Based Streams.

[ITU-T H.248.81]	Recommendation ITU-T H.248.81 (05/2011), Gateway Control Protocol: Guidelines on the use of the International Emergency Preference Scheme (IEPS) Call Indicator and Priority Indicator in H.248 Profiles.

[ITU-T Q.1950]	Recommendation ITU-T Q.1950 (2002), Bearer independent call bearer control protocol.

[ITU-T Y.2111]	Recommendation ITU-T Y.2111 (2011), Resource and admission control functions in next generation networks.

[ITU-T H.Sup9]	Supplement 9 to the ITU-T H-series (05/2008), Gateway Control Protocol: Operation of H.248 with H.225.0, SIP, and ISUP in Support of Emergency Telecommunications Service (ETS)/International Emergency Preference Scheme (IEPS).

[ETSI TS 129 238]	ETSI TS 129 238, Universal Mobile Telecommunications System (UMTS); LTE; Interconnection Border Control Functions (IBCF) - Transition Gateway (TrGW) interface; Ix interface; Stage 3.

[ETSI TS 129 334]	ETSI TS 129 334, Digital cellular telecommunications system (Phase 2+);Universal Mobile Telecommunications System (UMTS); LTE; IMS Application Level Gateway (IMS-ALG) - IMS Access Gateway (IMS-AGW); Iq Interface; Stage 3.

[ETSI TS 183 018]	ETSI TS 183 018, Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.

[ETSI TS 122 153]	ETSI TS 122 153 (11/2012), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Multimedia priority service (3GPP TS 22.153 version 11.1.0 Release 11).

# 3	Definitions

## 3.1	Terms defined Elsewhere

This Supplement uses the following term defined elsewhere:

3.1.1	Emergency Telecommunications Service (ETS) [ITU-T E.107]: A national service providing priority telecommunications to the ETS authorized users in times of disaster and emergencies.

## 3.2	Terms defined in this Supplement

3.2.1	Bearer-dependent decision point: the MGC as the primary decision point with respect to the mapping of call control level information to gateway control signalling and the applicable H.248 context, termination and/or stream entity as bearer level elements. The MGC provides all necessary information about bearer plane protocol stack, supported QoS technologies, etc.

NOTE 1 – This is the usual model in the master-slave relationship of H.248 gateways.

NOTE 2 – The synonym of "technology-dependent …" / "technology-independent …" originates from the ITU-T Resource and Admission Control Function (RACF) architecture (see [ITU-T Y.2111]).

3.2.2	Bearer-independent decision point: this is in contrast to clause 3.2.1, the MGC is still the primary decision point with respect to the mapping of call control level information to gateway control signalling but lacks (partially or fully) bearer related information. The MGC applies generic signalling and delegates the final mapping to the MG.

NOTE 1 – The notion of "bearer-independent …" originates from the Bearer Independent Call Control (BICC) architecture (see ITU-T Q.19xx-series of Recommendation); applied e.g. in the H.248 profile "Bearer independent call bearer control protocol" according to [ITU-T Q.1950].

3.2.3	Mapping function: function, located in H.248 entities (MGC, MG), for mapping protocol objects between two interfaces (MGC: between call control and gateway control signalling; MG: between gateway control signalling and user plane protocols). The mapping function relates to the following, hierarchical information:

MF type (a) -Protocol mapping: e.g., H.248, H.323, SIP, IP; protocol type mapping: MGC level mapping: SIP to H.248, SIP(SDP) to H.248(SDP); MG level mapping: H.248 to IP);

NOTE 1 – The MGC located mapping function (MFMGC) could affect the two protocol levels (a1) application control protocol (e.g., H.323, SIP, BICC) and an optional embedded (a2) media description protocol (e.g., SDP).

MF type (b) - Type mapping: mapping of information element types (e.g., signalling element type, IP header field) and

NOTE 2 – At a syntactic level the mapping of information element type usually implies a mapping of the information element value.

MF type (c) - Value mapping: mapping of information element value.

NOTE 3 – The combination of signalling element/type is also known as codepoint.

The mapping function may affect all three levels in any combination.

NOTE 4 – Appendix III illustrates some examples of different mapping functions.

# 4	Abbreviations

| BD | Bearer Dependent |
| --- | --- |

| BDDP | Bearer-dependent Decision Point |
| --- | --- |

| BI | Bearer Independent |
| --- | --- |
| BICC | Bearer-Independent Call Control |

| BIDP | Bearer-Independent Decision Point |
| --- | --- |
| DPI | Deep Packet Inspection |
| DS | Differentiated Services |
| ETS | Emergency Telecommunications Service |

| IDS | Intrusion Detection System |
| --- | --- |

| IEPS | International Emergency Preference Scheme |
| --- | --- |
| IP | Internet Protocol |
| LxVPN | Layer X Virtual Private Network |

| MF | Mapping Function |
| --- | --- |

| MG | Media Gateway |
| --- | --- |
| MGC | Media Gateway Controller |
| MPLS | Multi-path label switching |
| MPS | Multimedia Priority Service |
| QoS | Quality of Service |
| RACF | Resource and Admission Control Function |

| RPH | Resource Priority Header |
| --- | --- |

| TC | Traffic Class |
| --- | --- |
| SDO | Standards Development Organization |

| SDP | Session Description Protocol |
| --- | --- |
| SIP | Session Initiation Protocol |

| TDF | Traffic Detection Function |
| --- | --- |
| ToS | Type of Service |

# 5	Conventions

[ITU-T I.130] defines a stage and description level based specification methodology. This document uses the conventions of "stage 2" and "stage 3" according to [ITU-T I.130], which relates to the following kind of information in case of signalling interface specifications (such as H.248 profiles):

- stage 2: requirement for signalling interface and

- stage 3: protocol solution.

It may be noted that stage 2 and stage 3 information are fully decoupled (according to [ITU-T I.130]), e.g., a specific requirement may be principally satisfied by multiple signalling solutions.

NOTE – Other SDOs outside ITU-T may use a different convention for stage-based specification processes.

# 6	Priority traffic – Traffic model and profile specifications

This is follow on information to [ITU-T H.248.81], which provides profile specification guidelines for H.248 profiles with support of priority traffic indication. The notion of priority traffic covers the three Context attributes priority indicator, emergency indicator and IEPS call indicator and shall be understood in this discussion as follows:

An H.248 Context is associated to priority traffic handling when following condition is true:

IF (PriorityIndicator > 0) OR (EmergencyIndicator = ON) OR (IEPScallIndicator = ON)

## 6.1	Traffic model

Figure 1 recalls again the overall traffic model from [ITU-T H.248.81]. Priority traffic handling as such affects multiple areas: "3", "4" and "5" with regards to the MG entity.

Figure 1 – Example overall traffic model for priority services with scope on the MG entity

The functional areas are (according to [ITU-T H.248.81]):

- Part III: MG Control Path – Priority command processing;

- Part IV: MG Data Path – Reservation, allocation and pre-emption of resources depending on national variation; and

- Part V: Bearer network – Support of dedicated "QoS and policy architectures".

Any kind of traffic priority indication sent from the MGC to the MG leads to correspondent actions in one or multiple functional areas. A generic MG processing rule concept for priority traffic handling may be introduced (see also Figure 2): the rule conditions are given by H.248 signalling elements and the rule actions are related to the above mentioned areas.

## 6.2	Conventions for abstracted H.248 signalling specifications

The following clauses elaborate on specification aspects concerning priority traffic treatment. The generic method of "rule-based description of MG operations" (as e.g. outlined by [ITU-T H.248.79] is used due to the general approach utilised by this Supplement, the advantage of independence of specific H.248 Descriptors and the compact format.

## 6.3	Specification plane: Type of H.248 Profile

This Supplement introduces two H.248 profile types:

- Generic H.248 profile: characterized by a mix of mandatory, optional and conditional protocol capabilities, typically specified by SDOs (like e.g. international profiles);

- Practical H.248 profile: an instantiation of a generic profile, given by a concrete implementation and a given network operational environment. There are no optional capabilities: either something is useable or not.

MG rule actions may be linked to the MG rule conditions in case of a practical profile (because there are no uncertainties from network engineering and MG capability support point of view, and also from the fact the expected MG behaviour must be determined).

However, it's rather difficult (often impossible) to specify the association between concrete MG actions and their underlying conditions in generic profiles (due to their generic nature, vagueness of operational environment, etc.). Such a situation shall be called "semantic issue of MG actions" in the context of priority traffic handling.

Figure 2 – Generic profile view – Processing rule concept
(plus the three functional areas of possible MG actions)

Figure 3 indicates some principal issues related to generic profiles:

#1: 	generic vs practical profile type (see above);

#2: 	binding of actions to conditions ("the exact specified binding determines the final service logic and hence MG behaviour");

#3:	dependencies on final operational environment of a deployed MG, concerning bearer protocol stacks, QoS architecture, overall traffic mix, etc.;

Furthermore, each of the three Context attributes may lead to MG actions in the three areas, which could lead to some overlap (and possible interaction issues) when multiple Context attributes are used in parallel (see Figure 4):

#4: 	isolated view (e.g. by defining individual call-dependent procedures for each ContextAttribute element) is not appropriate due to possible interaction problems of MG-level actions);

The isolated view is often given in specification roadmaps when an initial profile version supports one (or two) Context attributes for priority traffic indication, and subsequent profile versions add further support in this area. In order to avoid interaction issues, a reconsideration and comprehensive view would be then necessary (see Figure 5):

#5: 	comprehensive consideration of all supported Context attributes and their binding to MG actions (in order to avoid possible interaction of individual MG actions and to define a deterministic MG behaviour).

Figure 3 – Generic profile view – Some issues

Figure 4 – Generic profile view – Issue due to isolated consideration

Figure 5 – Generic profile view – Comprehensive consideration

## 6.4	Specification plane: Codepoint space

The comprehensive view (Figure 5) alludes to the usage of a single parameter (instead of three (Context attributes)) for the control of correspondent MG action(s). This clause discusses the thought of such an approach. It does not revise existing H.248 protocol syntax.

Figure 6 illustrates such an approach: the left side provides legacy H.248 protocol syntax, the three Context attributes define a compound condition at Context level with regards to the required traffic handling. It's a hierarchical scheme due to the three explicit signalling elements for action control.

The right hand side provides a flat scheme: the three Context attributes are mapped (and replaced) by a single variable, called TrafficPriorityParameter. Some aspects:

- It may be noted that the single parameter approach may be already applied today, implementation internally.

- There are 64 codepoints defined (required) by the three Context attributes (2  2  16).

- The three Context attributes could be thus unambiguously mapped on the TrafficPriorityParameter, defined by e.g. a data type of Integer and a value range of [0, 63] for this variable.

- The two signalling concepts ("three vs. a single parameter") are functionally equal because the three H.248 Context attributes are generic in the sense that they do not specify concrete MG actions.

A future gateway control (or other signalling or management) protocol design may consider to define just a single signalling element with a future safe codepoint space.

Figure 6 – Hierarchical versus flat priority scheme

## 6.5	Specification plane: Further comments

The requirement on MG behaviour should normally drive the requirement(s) for the H.248 interface (but not vice versa). The resultant MG behaviour would be from the definition of an overall network solution.

The network element behaviour is firstly at the top level of a composed "MGC/MG" gateway entity, according the overall traffic model of [ITU-T H.248.81]. Hence there is a mapping of "service requirements/capabilities" on the "composed MGC/MG" and then "functional distribution on decomposed gateway model".

# 7	Summary

This supplement illustrates the difficulty, even impossibility, of a generic specification  i.e. , a generic H.248 profile in justifying concrete, dedicated protocol solutions in case of available alternative for "emulation approaches".

Appendix I

H.248 profiles and Context Attributes for priority traffic handling

This Appendix contains an analysis of signalling support for priority traffic handling.

## I.1	Status H.248 Profiles

H.248 profile definitions include Context Attribute support information (see the profile definition template in [ITU-T H.248.1]). There are many H.248 profile specifications published by standard development organizations (SDOs). Of particular interest in this Supplement are profiles for H.248 IP-IP gateways; such as the 3GPP-defined profiles for their H.248-based reference points Ix and Iq ( [ETSI TS 129 238] and [ETSI TS 129 334]; which are both based on the ETSI TISPAN Ia profile [ETSI TS 183 018]).

The following table summarizes the status for Ix and Iq interfaces:

Table I.1 – 3GPP Profiles and typical support of Context Attributes (status: Rel-11)

| Context Attribute | Supported | Values Supported |
| --- | --- | --- |
| … |  |  |
| Priority Indicator | Optional (NOTE 1) | 0-15 (Note 2) |
| Emergency Indicator | Yes | YES/NO |
| IEPS Call Indicator | No | NA |
| … |  |  |
| NOTE 1 - This Context Attribute parameter is allowed in ETSI TISPAN Ia Profile version 3. It is also used for MPS as specified in [ETSI TS 122 153]. NOTE 2 - Priority values 11 – 15 of the Priority Indicator are reserved for MPS. |  |  |

It may be noted that there is support of emergency calls only in pure 3GPP environments, and additional priority level support in ETSI TISPAN and 3GPP instantiations.

## I.2	Discussion of the principal H.248 signalling options

H.248 supports the provisioning of protocol parameter values, which may sometimes be used as an alternative to the usual signalling of protocol values. There are further kinds of H.248 decomposed gateway models, such as bearer-independent or bearer-dependent style of operation (see clauses 3.2.2 and 3.2.1). The alternatives lead to various options which are outlined in this clause, using the concrete example of DiffServ-based QoS marking as the selected technology for priority traffic support. The different options are also related to a mapping function (MF; see clause 3.2.3) involved at MGC and/or MG level.

### I.2.1	Overview

The "principal H.248 signalling options" are given by the combination of the following characteristics:

- Kind of mapping function (related to type and/or value mapping, MFb,c)

- At MGC level: call control protocol related codepoints are mapped to H.248 codepoints ('bearer dependent' or 'bearer independent') format (by the MFMGC(b,c) ); or/and

- At MG level: mapping of H.248 codepoints to final IP bearer path format (by the MFMG(b,c));

- H.248 signalling format

- 'bearer dependent', i.e., QoS marking codepoints signalled already in the format as used in IP bearer packets  (hence, so-called type mapping (MFb) not necessary);

- 'bearer independent', i.e. (I.b) type mapping (MFb) would be always required in the MG.

Four meaningful combinations are subsequently described. Appendix III.2 further illustrates the example variants.

### I.2.2	Bearer dependent H.248signalling

H.248 signalling (i.e., ADD and MODIFY request commands) carries the bearer dependent DiffServ codepoint (as e.g. assigned for priority traffic). The MG directly uses that DiffServ codepoint for QoS marking in outgoing bearer packets. There are two variants from the MGC perspective, see Figure I.1.

Figure I.1 – Control of a MG-level QoS marking function – Variants 1 and 2

Details:

- Variant 1:  the call control signalling (SIP) includes the QoS marking codepoints applicable for this media in the format used in the IP packets. The same codepoints are sent in the H.248 signalling to the MG. Thus, type mapping and value mapping are avoided in this variant, both at MGC and MG level.

- Variant 2: either call control signalling does not support signalling of DiffServ values or a local DiffServ translation is applied at the MGC. In this case, the QoS marking is provisioned at the MGC, the MGC provides the DiffServ mapping and includes the DiffServ codepoints in the H.248 signalling. Thus, there is type/value mapping at MGC level, but not any such mapping additionally necessary by the MG.

### I.2.3	Bearer independent H.248 signalling

The DiffServ codepoint for a particular packet bearer traffic class might be also provisioned at MG level. This implies the necessity of a correspondent mapping function at MG level (MFMG, see Figure I.2) besides the MGC level MF.

Figure I.2 – Controlling of MG-level QoS marking function – Variant 3

Details:

- Variant 3:  there is no explicit signalling of DiffServ codepoints at the H.248 interface. Instead another H.248 protocol element could be signalled to trigger marking bearer packets with appropriate DiffServ codepoints.

- Variant 4: in addition to other H.248 protocol elements as in variant 3, H.248 signalling may contain direct DiffServ codepoints, which leads to a mix of both H.248 signalling options (i.e., 'bearer dependent' and 'bearer independent' formats used in parallel), and hence requires the careful consideration of possible interaction issues.

## I.3	IEPS/ETS traffic support: signalling (stage 3) solutions

There are different signalling (stage 3) alternatives as outlined by the previous clause, in order to satisfy requirements (stage 2) for IEPS/ETS traffic support:

- Native approach: direct signalling (stage 3) method based on the explicit H.248 signalling element for that purpose, e.g. the ContextAttribute IEPS call indicator. "Native" means that a stage 3 protocol element is used for the underlying stage 2 requirement. This approach is bearer independent.

- Emulation approach: indirect signalling variant by reuse of existing signalling (stage 3) capabilities for IEPS/ETS,like the ContextAttribute Priority indicator for IEPS/ETS (bearer independent) or diffserv codepoints (bearer dependent). "Emulation" means the usage of stage 3 for other stage 2 requirements (if stage 3 is applicable).

Thus, the "native" approve may require a mapping function (but not necessarily), whereas the "emulation" approach requires a mapping function.

Option (A) requires H.248.1 version 3.

Appendix II

Potential use cases with IEPS call indicator support

## II.1	Overview

The following use cases may be identified concerning H.248 IP-IP gateway types (not an exhaustive list):

Table II.1 – Overview of use cases

Potential use cases with IEPS call indicator support

Use case #A: Support of multiple, different IP QoS architectures

Use case #B: IP bearer connection traversing different IP QoS architectures

Use case #C: Priority treatment with more than 16 priority levels

Use case #D: Network monitoring of signalling traffic (here: H.248)

Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Use case #F: Heterogeneous vendor landscape

A crucial location of such an H.248 gateway is the edge of an administrative domain, such as the peering point between different network operators. Such an interconnection scenario needs to consider a wide spectrum of possible network architectures and technology support due to different preferences and/or network status of individual operators.

Figure 1 (in clause 6) summarizes priority traffic handling by H.248 entities. Any kind of traffic priority indication sent from the MGC to the MG leads to correspondent actions in one or multiple functional areas.

## II.2	Use case #A: support of multiple, different IP QoS architectures

This use case focuses on bearer network characteristics (so called area "5" in Figure 1) by considering support of multiple, different IP QoS architectures (see Figure II.1).

Figure II.1 – Use case #A: support of multiple, different IP QoS architectures

The assumption is that the stage 2 requirement could be satisfied by dedicated QoS support for IEPS/ETS traffic as a stage 3 solution. This implies a mapping of IEPS/ETS information at call control signalling level to appropriate "H.248 means" (see also Figure. II.1).

There are two basic solutions, according different mapping strategies, outlined in Figure. II.1:

- Solution A.1:

  - the MGC is responsible for all kinds of QoS mappings, thus a so-called bearer-dependent decision point (see clause 3.2.1);

  - the stage 2 requirement could already be supported by H.248 signalling elements for "QoS technologies";

  - the ContextAttribute IEPS call indicator may not be needed at all;

- Solution A.2:

- the BDDP responsibility is delegated to the slave MG entities;

- the MGC would be thus acting as a bearer-independent decision point (see clause 3.2.2);

- a generic mapping between call control signalling indicators and H.248 ContextAttributes would be possible.

Conclusions:

- Option A.2 looks attractive from MGC perspective, however, existing profile specifications assign the BDDP to the MGC (due to the "master" attribute of a MGC, see also clause 3.2.1);

- thus, the starting point for stage 3 would be the assumption of a "QoS technology aware" MGC (i.e. option A.1)

- any IEPS call indicator support isn't straight forward to justify.

## II.3	Use case #B: IP bearer connection traversing different IP QoS architectures

The next use case is similar as scenario #A, but focuses on the interworking aspect: the end-to-end IP bearer connection for IEPS/ETS traffic traversing different IP QoS architectures (Fig. II.2). Thus, the two H.248 IP stream endpoints would then have different protocol stacks and/or different policy enforcements for IEPS/ETS support, leading to additional mapping complexity as in comparison to #A.

Figure II.2 – Use case #B: IP bearer connection traversing different IP QoS architectures

Conclusion:

- the discussion (for the indicated solutions B.1 and B.2 in Figure II.2) is fairly similar as in use case #A.

## II.4	Use case #C: Priority treatment with more than 16 priority levels

The codepoint space would be exhausted in network environments which request more than 16 priority levels (due to existing protocol design of [ITU-T H.248.1]). The "IEPS/ETS service" could not be emulated ("the option B in clause I.3") by reserving a particular H.248 priority indicator codepoint exclusively for IEPS/ETS calls.

Figure II.3 – Use case #C: Priority treatment with more than 16 priority levels

Conclusions:

- there would be just one solution feasible (C.1) by support of the IEPS call indicator

- there is also not yet any stage 2 requirement announced requesting  more than 16 priority levels.

## II.5	Use case #D: Network monitoring of signalling traffic (here: H.248)

Figure II.4 illustrates a (theoretical) network monitoring scenario, based on policy rules, - such as used by deep packet inspection (DPI), traffic detection functions (TDF) or intrusion detection systems (IDS) -, for detection of application "IEPS/ETS service" in network control plane traffic.

Figure II.4 – Use case #D: Network monitoring of signalling traffic (here: H.248)

Conclusions:

- No further consideration is required since such a hypothetical monitoring service should not place any explicit requirements on the signalling.

## II.6	Use case #E: National regulations which mandate the use of both, IEPS call and priority indicators

Regulation authorities could explicitly mandate support of all ContextAttributes for national solutions.

Conclusion:

- No further consideration is needed here because a national (or local) H.248 profile would/could be derived from international (or global) H.248 profile specifications.

## II.7	Use case #F: heterogeneous vendor landscape

No further elaboration of this use case is needed, because this is rather a network operator topic and beyond the scope of SDOs.

## II.8	Summary and conclusions

The stage 2 requirement for IEPS/ETS support may be basically satisfied by two stage 3 solutions: with and without IEPS call indicator capability, called native (A) and emulation (B) approach in clause I.3.

A number of practical and more theoretical use cases are discussed in this Appendix. It might be concluded that the native approach could be beneficial in two scenarios:

- A.2: MGC as bearer-independent decision point (BIDP) when IEPS/ETS support implies a dedicated QoS technology; and

- C.1: Exhausted codepoint space due to unavailable priority level codepoints for additional IEPS/ETS indication.

However, the very majority of use cases could be already solved by an emulation approach.

Appendix III

Mapping function – Examples

The mapping function (MF) is defined in clause 3.2.3. This Appendix illustrates the concept through some examples.

## III.1	Examples of MGC and MG located mapping functions

Examples:

- MGC located mapping function (MFMGC):

- the SDP “b=” line is mapped from SIP/SDP to H.248/SDP with a modified “b=” line value: MF types (a1, c) (because the information element type (SDP “b=” line) is unchanged (Figure III.1).

Figure III.1 – Example of a MGC-level mapping function (MFMGC)

- the SIP Resource Priority Header (RPH) value x1 is mapped to
2.1 H.248 priority indicator with value x2 (see also Table 2/[ITU-T H.Sup9]): i.e., MF types (a, b, c);
2.2 H.248 ds/dscp property with value x3: i.e., MF types (a, b, c);
2.3 H.248 vlan/pri property with value x1: i.e., MF types (a, b) (because the Ethernet priority is here identical to the SIP RPH value);

- the SDP “a=qos” attribute is mapped from SIP/SDP to H.248/SDP

- MG located mapping function (MFMG):

  - the H.248 priority indicator with value x1 is mapped to
4.1 IPv6 traffic class with value x2: i.e., MF types (a, b, c);
4.2 IPv6 traffic class with value x1: i.e., MF types (a, b);
4.3 Ethernet priority header with value x1: i.e., MF types (a, b);
4.4 IPv4 DiffServ field with value x2: i.e., MF types (a,b,c);
4.5 IPv6 DiffServ field with value x2: i.e., MF types (a, c);

- the H.248 ds/dscp property with value x1 is mapped to
5.1 IPv6 DiffServ field with value x1: i.e., MF type (a);
5.2 IPv6 DiffServ field with value x2: i.e., MF types (a, c); 
5.3 Ethernet priority header with value x1: i.e., MF types (a, b);

## III.2	Complete view on the two mapping functions in series

The examples in this clause refer to the evaluation of signalling options in Appendix I.2, which selects three variants for their discussion. Figure III.2 illustrates the three variants:

Figure III.2 – Three of the variants as discussed in Appendix I.2

_________________

Such a protocol engineering approach would avoid the side discussion here: upgrade of H.248 protocol version from 1 or 2 to 3 due to IEPS call indicator support.