| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F.TPS-Req: Editor’s note clause 7 |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email:tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

- Introduction

This Contribution proposes updates to F.TPS-Req in order address the editor’s notes in clause 7 on User experience requirements.

- Discussion

F/TPS-Req contains a number of editor’s notes. This Contribution discusses several of these and proposes a resolution of these items.

Clause 7[F.TPS-Req] on User experience requirements contains two editor’s notes. These are discussed below:

{Editor’s Note: Further contributions are needed to describe these requirements more precisely, and to limit them to not require detailed performance metrics which may not be available in the near-term. Treat the following as examples for now.

In reading the user experience requirements it is not clear that the current requirements in any way require detailed performance metrics. The requirements are written in terms of a desired behaviour rather than being assigned a particular metric. The exception may be User01 which indicates particular HD modes. In detailing them more precisely metrics would have to be introduced which doesn’t appear to be in keeping with the latter part of the editor’s note. The Contributors propose to delete this editor’s note.

{Editor’s Note: The supported HD modes need further consideration.}

As discussed above User01 is the only user requirement that has specific metrics (e.g. 1080p, 1080i). The requirement appears to be focussed on the support of high quality actual sized rendering of participants at the remote end. The support of such a feature is not only dependent on the display resolution. As discussed in clause 7.1.2.4 [ITU-T H.TPS-AV] on effective resolution other elements come into play. It recommends a range for effective resolution in terms of cycles per degree. So rather than specify a range of resolutions in User01 the Contributors propose to remove the resolutions and editor’s note and simply rely on H.TPS-AV to provide a recommendation on effective resolution.

- Proposal

It is proposed to accept the following updates. Changes are marked up against TD-26a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

…

7	User experience requirements

This section addresses user experience requirements of a telepresence system on an end to end system basis, primarily in terms of functional requirements, rather than detailed performance requirements.

User01: It is recommended that a telepresence system support actual size high-definition imaging of videoconferencing participants at the remote end. The support of actual size is considered an advanced Telepresence feature.

User11: It is recommended that a telepresence system indicate detailed information associated with a composed capture including the list of originating sources and the policy of the composition.

…

I.3	Media control functions

| Requirement | Covered | Details |
| --- | --- | --- |
| Med001 | Yes | Clause 7.1.2.4/[ITU-T H.TPS-AV] contains a parameter to describe the effective resolution which takes into account the display of actual sized images. |
| Med002 | Yes | H.TPS-AV has parameters that allow multiple audio and video captures to be described along with their spatial co-ordinates. |
| Med002a | ? |  |
| Med002b | ? |  |
| Med002c | Part | H.TPS-AV includes a presentation attribute which can be set for multiple captures. That may not be sufficient to fully cover this requirement. |
| Med003 | Part | Being able to signal multiple captures utilising the parameters of H.TPS-AV allows this. In addition the media control unit functions describe by F/H.TPS-Arch provides a means to transcode and/or compose audio and video streams. |
| Med004 | ? | There is no discussion of 3D coding elsewhere in the TPS documents. |
| Med005 | ? | There is no discussion of 3D coding elsewhere in the TPS documents. |
| Med006 | ? | There is doesn’t appear to be specific text elsewhere in the TPS documents that talks about in session media stream changes. |
| Med007 | Part | F/H.TPS-Arch discusses these regarding endpoint characteristics. H.TPS-AV provides some encoding parameters. |
| Med008 | ? | There is no discussion elsewhere in TPS documents regarding the benefits of this. It is assumed that it is supported by signalling. |
| Med009 | ? | There is no discussion in the TPS documents regarding this. |
| Med010 | ? | There is no discussion in the TPS documents regarding this. |

__________________