| Question(s): | 5 |  | Meeting, date: |  | Geneva, 28th October – 8th November 2013 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 1 | Intended type of document (R-C-TD): |  | C |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |  |  |
| Title: | F.TPS-Req: Editor’s note clause 10 |  |  |  |  |  |
| Contact: | Weiwei Yang Huawei Technologies China |  |  |  | Tel: +86 29 87674367 Fax: +86 29 87674151 Email: tommy@huawei.com |  |
| Contact: | Jing Xiao Huawei Technologies China |  |  |  | Tel: +86 755 36830146 Fax: +86 755 36830131 Email: lennard.xiao@huawei.com |  |
| Contact: | Christian Groves NTEC Australia Australia |  |  |  | Tel: +61 3 9391 3457 Fax: +61 3 9391 3457 Email: Christian.Groves@nteczone.com |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This Contribution proposes updates to F.TPS-Req in order address the editor’s notes in clause 10 on network requirements.

# Discussion

F/TPS-Req contains a number of editor’s notes and a living list item. This Contribution discusses several of these and proposes a resolution of these items.

Clause 10/[F.TPS-Req] contains two editor’s note regarding the support of NAT and the support of IPv4/IPv6 in telepresence systems. Each of the editor’s notes indicates that there needs to be more discussion. These are discussed below:

Net003: It is recommended that a telepresence system provide a means to traverse NATs or firewalls automatically to assure connectivity of telepresence services with minimal user operation.

{Editor’s Note:  This requirement needs more discussion.}

Requirement Net003 was introduced as a result of AVD-4087 from the July 2011 Q.5 meeting. The contribution indicated that the scope was:

In case that telepresence system is attached into private network such as behind of NAT and firewall, it is hard to guarantee bi-directional multimedia session. There are several solutions, such as SBC, TURN, STUN, UPnP and ICE, for traversing NAT. In order to provide convenient operation of telepresence service, it should be easy to configure and use.

The current Net003 requirement does not provide any background or description as to when NAT/firewall traversal is required. The Contributors believe that it is beneficial to scope when the requirement is needed. For example: in telepresence systems where all the endpoints reside in the same private network this requirement would not be needed. Therefore it is proposed to update the requirement with this scope.

It is noted that clause 8.2 /[ITU-T F/H.TPS-Arch] on MCU functions indicates support for an automatic NAT traversal function.

Net004: It is required that a telepresence system support both IPv4 and IPv6.

{Editor’s Note: This requirement needs more discussion. Use cases would be helpful.}

Requirement Net004 was added as a result of C.757 (from the November 2011 SG16 meeting). There was little discussion regarding the scope of the requirement. It is not clear from the requirement what scope and functionality is implied. There are a number of questions:

- Does this relate to all functional components in a telepresence system?

- Does this mean all components must have a dual IPv4/IPv6 stack?

- Can different interfaces use different IP versions?

- Is a particular functional component responsible for IP transport version interworking?

It is noted that clause 8.2 /[ITU-T F/H.TPS-Arch] does indicate that one of the access functions of a MCU is to support both IPv4/v6 connections.

The Contributors agree with the editor’s note that use cases would be beneficial. The Contributor’s propose to add the above questions to the editor’s note to stimulate further discussion.

# Proposal

It is proposed to accept the following updates. Changes are marked up against TD-26a from the June 2013 Q.3/16 Rapporteurs’ meeting in Olso.

Draft new Recommendation F.TPS-Reqs

Definitions, requirements, and use cases for Telepresence Systems

…

10 Network requirements

Net001: It is recommended that a telepresence system enable reservation of  network resources for assuring QoS for the telepresence session.

Net002: It is recommended that a  telepresence system enable adaptation of the quality of media based on the network status. For example: Under best-effort network such as Internet which cannot provide strictly guaranteed network QoS, the telepresence system may need to manipulate its video quality based on network status.

Net003: It is recommended that a telepresence system provide a means for endpoints to participate in telepresence conferences when they or other telepresence system components are located in a private network.

Note: Signalling and media may be subject ofNATs or firewalls traversal techniques to assure connectivity of telepresence services. Such techniques should be applied automatically with minimal user operation.

Net004: It is required that a telepresence system support both IPv4 and IPv6.

{Editor’s Note: This requirement needs more discussion. Use cases would be helpful It is not clear from the requirement what scope and functionality is implied. There are a number of questions:

•	Does this relate to all functional components in a telepresence system?

•	Does this mean all components must have a dual IPv4/IPv6 stack?

•	Can different interfaces use different IP versions?

•	Is a particular functional component responsible for IP transport version interworking.}

…

## I.8	Network requirements

| Requirement | Covered | Details |
| --- | --- | --- |
| Net001 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Net002 | ? | There is no discussion elsewhere in TPS documents regarding this. |
| Net003 | Part | Clause 8.2 /[ITU-T F/H.TPS-Arch] on MCU functions indicates support for an automatic NAT traversal function. |
| Net004 | Part | Clause 8.2 /[ITU-T F/H.TPS-Arch] does indicate that one of the access functions of a MCU is to support both IPv4/v6 connections. |

__________________